| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Control Messages to support FBIS in 16.1a Network |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Seokki Kim, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16.1a Amendment Working Document < 80216n-10_0025.pdf > |  |
| Abstract | This provides AWD text proposals of MAC Control Messages for Forwarding Between Infrastructure Stations |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Control Messages to support FBIS in 16.1a Network

Anseok Lee, Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun,

Hyun Lee, Seokki Kim, Kwangjae Lim, Dongseung Kwon

ETRI

# 1. Introductions

This contribution provides MAC Control Messages to support FBIS (Forwarding Between Infrastructure Stations) in 16.1a network based on 802.16.1 baseline standard.

# 2. Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modification to Section 6.2.2.1.3]

Modify section 6.2.2.1.3 as indicated

Table 5 – Type field encodings for MAC signaling header types

| Type field (5-bits) | MAC signaling header type |
| --- | --- |
| 00000 | BR with STID |
| ... |  |
| 00111 | MIMO feedback |
| 01000-11111 | Reserved |
| 01000 | Switched Access Control |
| 01001-11111 | Reserved |

Add section 6.2.2.1.3.9 as indicated

6.2.2.1.3.9 Switched Access Control

The Switched Access Control signaling header shall be used to convey control signaling related to the switched access in FBIS by Designated FBIS HR-MS or any of two serving HR-BSs.

Table xxx – Switched Access Control header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Switched Access Control () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header. Set to 0010. |
| Type | 5 | MAC signaling header type = 01000 |
| Length | 3 | Indicates the length of the signaling header in bytes |
| SAC sub-type | 3 | 0b000 = Access Request 0b001 = Access Response 0b010 = Switched Access Termination Request 0b011 = Switched Access Termination Response 0b101~0b111 = Reserved |
| If (RAC subtype == 0b000) { |  |  |
| Requested Switched Access Window Size | 7 | Requested access time in frames |
| Bandwidth Request Indication | 1 | Indicates that Designated FBIS HR-MS has data to send |
| If (Bandwidth Request Indication == 1) { |  |  |
| BR Size | 19 | Aggregate bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested |
| STID | 12 | STID of Designated FBIS HR-MS which request UL bandwidth |
| } |  |  |
| } else if (SAC subtype == 0b001) { |  |  |
| Status | 1 | 0b0 : Switched access granted, allocates Switched Access Window 0b1 : Switched access not granted, Switched Access Window not allocated |
| If (Status == 0) { |  |  |
| Switched Access Window | 7 | LSB of frame sequence. Indicates the frame that Switched Access Window ended |
| } else if (RAC subtype == 0b011) { |  |  |
| Termination reason | 2 | 0b00 : Backbone recovery 0b01 : No connection for FBIS 0b10 : Link failures 0b11 : reserved |
| } else if(RAC subtype == 0b011) { |  |  |
| Status | 1 | 0b0 : Termination request granted 0b1 : Termination request not granted |
| } |  |  |
| Padding | variable | For byte alignment |
| } |  |  |

[Remedy 2: Accept the following modification to Section 6.2.3.1]

Modify section 6.2.3.1 as indicated

Table 27 – AAI-RNG-REQ message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 = HR multicast service flow update 0b1111 = reserved 0b1111 = Network reentry for FBIS operation |  |
| … |  |  |  |
| } else if (Ranging Purpose Indication == 0b1111) { |  | // Network reentry for FBIS operation |  |
| If (STID is not pre assigned) { |  |  |  |
| Serving BSID | 48 | The BSID of the AMS's previous S-ABS before incurring a coverage loss, or the BSID of the S-ABS to which the AMS is currently connected (has completed the registration cycle and is in Connected State). |  |
| Previous STID | 12 | The STID which the AMS uses in the previous S-ABS. |  |
| } else { |  |  |  |
| STID | 12 | The Station ID pre-assigned by the T-ABS |  |
| } |  |  |  |
| If (CMAC indicator == 0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| Primary Serving ABS flag | 1 | 0b0 : the AMS shall set its primary serving ABS as S-ABS (Degraded HR-BS) after network reentry 0b1 : the AMS shall set its primary serving ABS as T-ABS (Target HR-BS) after network reentry |  |
| Switched Access Mode | 1 | 0 : Switched Access with fixed Switched Access Windows 1 : Switched Access with variable Switched Access Windows |  |
| If(Switched Access Mode==0 ) { |  |  |  |
| Switched Access Window Size | 8 | The size of fixed Switched Access Window in unit of frame |  |
| } else if ( Switched Access Mode == 1) { |  |  |  |
| Maximum Switched Access Window Size | 8 | Maximum size of Switched Access Window in unit of frame |  |
| } |  |  |  |
| Switched Access Start Time | 8 | The 8 least significant bits of the absolute frame number at the T-ABS where the AMS starts to perform the Switched Access operation. |  |
| }//end of Ranging Purpose Indication |  |  |  |
| … |  |  |  |
| } |  |  |  |

[Remedy 3: Accept the following modification to Section 6.2.3.5]

Modify section 6.2.3.5 as indicated

Table 31 – AAI-SBC-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... |  |  |  |
| Multimode capability supported | 3 | If bit0 = 1, the capability of TTR relay mode supports If bit1 = 1, the capability of STR relay mode supports If bit2 = 1, the capability of base station function supports | Present as needed in HR-Network |
| FBIS capability supported | 1 | If Bit 0 = 1, FBIS capability supports | Present as needed in HR-Network |
| } |  |  |  |

[Remedy 4: Accept the following modification to Section 6.2.3.12]

Modify section 6.2.3.12 as indicated

Table 38 – AAI-HO-CMD message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI-HO-CMD message shall not include any T-ABS. However, if the requested ABSs in list available but MAC information is not shared, those ABSs may be included candidate T-ABS and serving ABS transfers MS information via backbone network or relay link in HR-Network 0b11: Alternative Path (only for HR-Network) 0b11: HO command for HR-Network |  |
| … |  |  |  |
| If (Mode == 0b00 or 0b11) { |  |  |  |
| …… | … | …… | … |
| If (HO Reentry Mode == 0b11) { |  |  |  |
| Role | 1 | 0b0: Stay as HR-MS; 0b1: Change to HR-RS; |  |
| } //end of If (HO Reentry Mode == 0b11) | - | - |  |
| … |  |  |  |
| Action Time | 8 | If HO Reentry Mode is 0b11, it is the wait time in units of 1 ms before the HR-MS starts to perform fast network reentry. Otherwise, it is tThe 8 least significant bits of the absolute frame number at the TABS where the AMS starts to perform network reentry. When CDMA_RNG_FLAG is set to 1, it indicates the frame whereafter the AMS starts a CDMA ranging process. The action time should be set to a frame that includes either a nondynamic ranging channel or a dynamic ranging channel. When CDMA_RNG_FLAG is set to 0, it indicates the frame where the AMS starts to expect the UL bandwidth allocation for transmission of RNG-REQ at target R1 BS or LZone (i.e., Fast ranging opportunity) or AAI-RNG-REQ at T-ABS. |  |
| …… | … | …… | … |
| } else if (Mode == 0b11) { |  |  |  |
| Extended HO Mode | 3 | 0b000 : Alternative Path 0b001 : FBIS Initiation 0b010 : FBIS Termination |  |
| If (Extended HO Mode == 0b000) { |  |  |  |
| Role | 1 | 0b0: Stay as HR-MS; 0b1: Change to HR-RS |  |
| Action Time | 1 | If HO Reentry Mode is 0b11, it is the wait time in units of 1 ms before the HR-MS starts to perform fast network reentry |  |
| [Copy fields of the AAI-HO-CMD message with Mode==0b00 except the Action Time to here] |  |  |  |
| } else if (Extended HO Mode == 0b001) { |  |  |  |
| Primary Serving ABS | 1 | 0b0 : the AMS shall set its primary serving ABS as S-ABS (Degraded HR-BS) after network reentry 0b1 : the AMS shall set its primary serving ABS as T-ABS (Target HR-BS) after network reentry |  |
| Switched Access Mode | 1 | 0 : Switched Access with fixed Switched Access Windows 1 : Switched Access with variable Switched Access Windows |  |
| If(Switched Access Mode==0 ) { |  |  |  |
| Switched Access Window Size | 8 | The size of fixed Switched Access Window in unit of frame |  |
| } else if ( Switched Access Mode == 1) { |  |  |  |
| Maximum Switched Access Window Size | 8 | Maximum size of Switched Access Window in unit of frame |  |
| } |  |  |  |
| targetBSID | 48 | BSID of the T-ABS |  |
| SA-Preamble Index | 10 | Indicate the SA-Preamble index of the carrier |  |
| Action Time | 8 | The 8 least significant bits of the absolute frame number at the T-ABS where the AMS starts to perform network reentry. When CDMA_RNG_FLAG is set to 1, it indicates the frame whereafter the AMS starts a CDMA ranging process. The action time should be set to a frame that includes either a nondynamic ranging channel or a dynamic ranging channel. When CDMA_RNG_FLAG is set to 0, it indicates the frame where the AMS starts to expect the UL bandwidth allocation for transmission of AAI-RNG-REQ at T-ABS. |  |
| Switching Access Start Time offset | 8 | Difference between Switching Access Start time and Action time in units of frames. The value of Switching Access Start time shall be calculated by adding this value with the value of Action time specified for this T-ABS. |  |
| CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. |  |
| If(CDMA_RNG_FLAG==0) { |  |  |  |
| offsetData | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | optional |
| offsetControl | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | optional |
| Pre-assigned STID | 12 | STID assigned to the AMS by the T-ABS |  |
| }else { |  |  |  |
| Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | Shall be presented if Seamless HO = 1; otherwise optionally presented |
| Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity of the dynamic ranging channel used in the RA-ID. Ranging opportunity index shall be assigned by the T-ABS. The T-ABS shall assign unique ranging opportunity index which is not overlapped with other ranging channel in the allocated frame. 0b0: 0b01 0b1: 0b10 | Optional |
| Ranging opportunity Subframe Index | 2 | Indicates the subframe index of the allocated ranging opportunity. The subband of a dynamic ranging channel is same as the ranging channel allocated by SFH. | Optional |
| Pre-assigned STID | 12 | STID assigned to the AMS by the T-ABS | Optional |
| } |  |  |  |
| Reentry process optimization | 5 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI-SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI-REG-REQ/RSP message during reentry processing. Bit #3: Omit higher layer protocol triggering for IP address refresh during reentry processing. Bit #4: If Bit #4 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines |  |
| Seamless HO | 1 | Indicates whether seamless HO is supported |  |
| Ranging initiation deadline | 8 | An AMS shall send the AAI-NG-REQ message during HO until Ranging initiation deadline |  |
| Pre-assigned MAPMask Key | 15 | The value of this parameter is the seed used at the T-ABS to initiate the PRBS generator used to scramble the 40-bit A-AMAP IE when the value of the STID included in this message is used as the CRC Mask Masking Code. See 16.3.5.3.2.4. |  |
| Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0b00: No service possible for this AMS 0b01: Some service is available for one or several service flows authorized for the AMS 0b10: For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 0b11: No service level prediction available. |  |
| T-ABS Physical_Carrier_Index | 6 | Physical carrier index of the recommended T-ABS | Shall be included when T-ABS is not included in AAI-NBR-ADV message or is multi-carrier ABS |
| Channel bandwidth | 8 | Channel bandwidth of recommended T-ABS in units of 125 KHz. | Shall be included when T-ABS is not included in AAI-NBR-ADV message |
| CP length | 2 | CP length of the recommended T-ABS 0b00: 1/8 0b01: 1/16 0b10: 1/4 0b11: reserved | Shall be included when T-ABS is not included in AAI-NBR-ADV message |
| For(j=0;j<N_Preassigned_Carriers; j++) { |  | N_Preassigned_Carrier is the number of preassigned carriers by the T-ABS | Shall be included when one or more carriers of the AMS is pre-assigned by the T-ABS. |
| Carrier Status Indication | 1 | Indicating whether this preassigned carrier will be activated immediately after HO procedure is done |  |
| Physical carrier index of the secondary carrier index | 6 | Physical carrier index of the preassigned secondary carrier |  |
| } |  |  |  |
| SFH delta information | Variable | Delta encoding with reference to the S-ABS | May be included when SFH delta information is not included in the AAI-NBR-ADV message for the T-ABS |
| S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information | Shall be included when SFH delta information is included |
| S-SFH application indicator | 1 | Indicate if the SFH delta information at the T-ABS is applied at the Action Time 0b0: not applied 0b1: applied | Shall be included when SFH delta information is included |
| } else if (Extended HO Mode == 0b010) { |  |  |  |
| Termination Reason | 2 | 0b00 : Backbone recovery 0b01 : No connection for FBIS 0b10 : Link failures 0b11 : reserved |  |
| Action Time | 8 | The 8 least significant bits of the absolute frame number at the serving ABS where the AMS starts to terminate FBIS. |  |
| Resource_Retain_Time | 8 | The time when the serving BS discards MS's context. | Optional. If not included, the value included in the AAI-RNG-RSP or AAI-REG-RSP previously received at the S-ABS shall be used. |
| } |  |  |  |
| } // end of if(Mode==0b11) |  |  |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE 802.16n-11/0048r2, “802.16n System Requirements Document including SARM annex”

[2] IEEE 802.16n-11/0025, “ P802.16.1a Draft AWD”