| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Key Request Procedure for IEEE 802.16.1a |  |
| Date Submitted | 2011-10-3111-07 |  |
| Source(s) | Wai-Leong Yeow, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute For Infocomm Research | E-mail: wlyeow@i2r.a-star.edu.sg cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | Detail description of Multicast Security to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Key Request Procedure for IEEE 802.16.1a

Wai-Leong Yeow, Joseph Chee Ming Teo,

Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# The IEEE 802.16n System Requirements Document (SRD) specifies shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection. The HR-Network shall provide multicast key management for the group of HR-MSs and the master key shared within the group should be distributed securely.

# To ensure that an attacker is not able to masquerade as a multicast member or eavesdrop in the multicast communications, multicast key management (MKM) protocols have to be designed for the 802.16n networks. Although a Multicast and Broadcast Rekeying Algorithm (MBRA) is already supported in IEEE 802.16-2009, however, it is based on PKMv2 and the rekeying is performed through multicast without any ARQ support. The current IEEE 802.16n AWD supports a key hierarchy for security in multicast but does not describe how the root key is shared between stations of the multicast group.

# In this contribution, we propose a procedure for an HR-MS to initiate a request to the HR-BS and obtain the Multicast Authentication key securely.

# 2. Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

[Adopt the following text in the 802.16.1a AWD Document (C802.16x-xx/xxxx)]

[Change Table 71 as indicated:]

Table 71 – PKMv3 message types

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| 1 | PKMv3 Reauth-Request | AAI-PKM-REQ |
| 2 | PKMv3 EAP-Transfer | AAI-PKM-REQ/AAI-PKM-RSP |
| 3 | PKMv3 Key_Agreement-MSG#1 | AAI-PKM-RSP |
| 4 | PKMv3 Key_Agreement-MSG#2 | AAI-PKM-REQ |
| 5 | PKMv3 Key_Agreement-MSG#3 | AAI-PKM-RSP |
| 6 | PKMv3 TEK-Request | AAI-PKM-REQ |
| 7 | PKMv3 TEK-Reply | AAI-PKM-RSP |
| 8 | PKMv3 TEK-Invalid | AAI-PKM-REQ/AAI-PKM-RSP |
| x | PKMv3 MulticastKey-Request | AAI-PKM-REQ |
| x+1 | PKMv3 MulticastKey-Response | AAI-PKM-RSP |
| 9y—16 | Reserved | — |

[Add the following text into Section 6.2.3.43 as indicated:]

### 6.2.3.43.x PKMv3 MulticastKey-Request message

The HR-MS transmits the PKMv3 MulticastKey-Request message as a first step for an HR-MS initiated multicast key request. The MulticastKey-Request message identifies the multicast group, the serving HR-BSID and MAC address of the requesting HR-MS.

The message shall include a Nonce_HR-MS generated by the requesting HR-MS and a Timestamp_HR-MS for freshness. The CMAC Digest attribute (i.e., CMAC_PN_U and CMAC value) shall be transmitted for CMAC verification, which is computed from CMAC_KEY _U.

Code: x

The message attributes are shown in Table aaa.

Table aaa – PKMv3 MulticastKey-Request message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | Multicast Group ID |
| Timestamp_HR-MS | Timestamp generated by HR-MS |
| Nonce_HR-MS | Nonce generated by HR-MS |
| HR-BSID | HR-BSID |
| HR-MS MAC address | 48-bit MAC address |
| CMAC_HR-MS | Message digest calculated by HR-MS |

### 6.2.3.43.x+1 PKMv3 MulticastKey-Response message

The HR-BS transmits the PKMv3 MulticastKey-Response message to an HR-MS for disseminating information necessary for derivation of the multicast keys of a multicast group. The message is transmitted in response as a second step to the HR-MS initiated multicast key request event.

The message shall include a Nonce_HR-MS sent by the requesting HR-MS in the MulticastKey-Request message, a Nounce_HR-BS generated by the HR-BS and a Timestamp_HR-BS for freshness. The multicast group ID, MAC address of the requesting HR-MS, HR-BSID shall be included in the message for verification. The MAK lifetime, MC_Nonce and COUNTER_MTEK shall be included in the message for the requesting HR-MS to derive the multicast keys.

The messaged shall be encrypted with the current TEK for MAK confidentiality, and contain the CMAC Digest attribute (i.e., CMAC_PN_D and CMAC value) for CMAC verification, which is computed from CMAC_KEY _D.

Code: x+1

The message attributes are shown in Table bbb.

Table bbb – PKMv3 MulticastKey-Response message attribute

| Attribute | Contents |
| --- | --- |
| MulticastGrpID | Multicast Group ID |
| Timestamp_HR-BS | Timestamp generated by HR-BS |
| Nonce_HR-BS | Nonce generated by HR-BS |
| HR-MS MAC address | 48-bit MAC address |
| HR-BSID | HR-BSID |
| Nounce_HR-MS | Nonce sent by HR-MS |
| MC_Nonce | The number used to derive the MCMAC-MTEK Preky |
| MAK Lifetime | Remaining Multicast Authentication Key Lifetime |
| COUNTER_MTEK | The current COUNTER_MTEK in use |
| CMAC_HR-BS | Message digest calculated using CMAC key by HR-BS |

[Add the following text after Section 6.12.10.2 as indicated:]

### 6.12.10.z Multicast Key Request

The Multicast Key Request Procedure takes place whenever an HR-MS wishes to derive up-to-date Multicast Keys of a secured multicast group from the serving HR-BS. Figure X1 shows the flow diagram.

The Multicast Key Request Procedure is described by the following steps:

Step 1: Mobile station HR-MS first generates Timestamp_HR-MS and Nonce_HR-MS. Next, HR-MS transmits a MulticastKey-Request message using with the nonce, timestamp, multicast group ID, the serving HR-BSID and its MAC address to the HR-BS. The message is protected by the CMCAC_HR-MS digest tuple using the current CMAC_KEY_U.

Step 2: HR-BS first verifies the received timestamps and nonce for freshness. If the verifications are incorrect, the request is silently dropped. Otherwise, HR-BS generates Nonce_HR-BS and Timestamp_HR-BS. If needed, a new MC_Nonce is generated or obtained and the MCMAC-MTEK Prekey and all sub keys are computed. HR-BS

then transmits an encrypted MulticastKey-Response message to the HR-MS containing the MC_Nonce, remaining lifetime of the MAK, the current COUNTER_MTEK, Nonce_HR-BS, Timestamp_HR-BS and the received Nonce_HR-MS, its own BSID and MAC address of the requesting HR-MS. This message shall be encrypted using the current TEK for confidentiality and protected by a CMAC_HR-MS digest tuple using the current CMAC_KEY_D.

Step 3: HR-MS first verifies the received timestamp and nonces for freshness. The MulticastKey-Response message shall be verified using the CMAC_HR-MS digest tuple. If the verifications fail, the MulticastKey-Response message is silently dropped. Otherwise, the message is decrypted with the current TEK, and the up-to-date MCMAC and MTEK keys can be derived for the multicast operation.

Figure X1 - Flow Diagram for MulticastKey-Request

# 6.12.10.z.a Message Type

Table aaa – Message Type

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| x | MulticastKey-Request | AAI-PKM-REQ |
| x+1 | MulticastKey-Response | AAI-PKM-RSP |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]