| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for addressing of STID Addressing Scheme in IEEE 802.16p system |  |
| Date Submitted | 2011-03-03 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty, Hujun Yin, Belal Hamzeh | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p Systems Requirements Document |  |
| Abstract | This contribution proposes performance requirements relevant to network entry by large number of devices. |  |
| Purpose | For review and adoption into the performance requirements section of the 802.16p SRD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for STID Addressing Scheme in IEEE 802.16p system

Rui Huang, Honggang Li, Shantidev Mohanty, Hujun Yin, Belal Hamzeh

Intel

# Introduction

According to SRD [1], one of requirements for IEEE 802.16p system is supporting a large number of devices in comparison with the human interface systems. Therefore, some discussion about how to handle the large number of M2M devices are raised, e.g. the need of extended STID address or not [2].

In this contribution, we analysis the current STID addressing scheme in 16m and evaluate the required number of STID needed in several typical M2M usage scenarios. In the conclusion we propose the legacy STID address scheme could be reused for 16p also.

# STID Addressing Scheme in 16m

In [3], a 12-bit STID value is defined to uniquely identify an AMS in the domain of the ABS.  For 16p, the number of M2M devices in a cell seems exceeding 2^12 generally and 12-bit STID addressing is not enough. An STID is valid for an MS when the MS is in the connected state. considering a large portion of M2M devices are in idle state more than in connected state, the required number of STID identifiers may far fewer then the number of M2M devices.  Figure1 shows that the usage of STID for multiple M2M devices.

Fig1. STID valid cycle

# Required STID Addressing Number Evaluation

In this section we will evaluate the total number of valid STIDs in a simulation duration, in which STID identifiers are only assigned to the M2M devices which are in connected state. One STID valid cycle is the sum of all required durations, including bandwidth request, data packet transmission, HARQ process, sleep windows etc.

The table in the appendix gives M2M devices deployment parameters in a cell. But in order to simplify this evaluation, we consider the two key M2M devices in the typical scenario: Home-security system and elderly sensor devices, which will impact occupied most of addressing resource in a cell since their transmission rate and packet size is much higher than others.

| Parameters | Value |
| --- | --- |
| Cell radius | 1km |
| Number of homes in a cell | 12077 |
| Number of M2M devices in a cell | Home-Security: 1 * 1, Elderly-Sensor: 0.1 + 1*11+ 2 + 3<17 |
| Simulation duration | Home-Security: 60s, Elderly-Sensor: 600s |
| Max HARQ retransmission times | 4 |
| Max HARQ retransmission interval(Frames) | 2 |
| Sleep window size(Frames) | 4 |
| Deregistration Duration(Frames) | 4 |
| Start assignment index | Randomly distributed within the simulation duration with the specific pdf which is up to the type of M2M service |
| HARQ retransmission time | Normally distributed within [1, Max HARQ retransmission times] |

The maximum required STID number needed for the connected M2M devices is given in the below table.

| Simulation Cases | Maximum STID number needed |
| --- | --- |
| Home-Security | 759 |
| Elderly sensor | 83*17 |
| Total | 2170 |

In conclusion, the maximum required number of STID identifiers for all M2M devices in typical deployment scenarios is much less than the 16m STID addressing scope(2^12) .So, the 16m STID addressing scheme can be reused for 16p.

# Text Proposal

Add the following to section 16 of the 802.16p AWD.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

### 16.2.1 Addressing

### 16.2.1.2 Logical identifiers

### 16.2.1.2.1 Station Identifier

A 12-bit STID is used to identify a M2M device in domain of the M2M BS also. M2M BS shall assign a STID to each M2M device or M2M device group during Network entry.

---------------------------End Text Proposal----------------------------------

# References

- IEEE 802.16p-10/0004r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” Jan. 2011

- C80216p-rg-11_0031,  IEEE 802.16p Machine to Machine (M2M) Discussion Points for Large Number of Devices (DEV) RG.

- 16m D11

- [71#52] UMTS/LTE: MTC - Simulations assumptions and output

- IEEE 80216p-10_0005, “IEEE 802.16p Machine to Machine (M2M) Technical Report,” November, 2010.

- http://www.zpryme.com/SmartGridInsights/2010_Smart_Appliance_Report_Zpryme_Smart_Grid_Insights.pdf

# Appendix:  Typical M2M Usage Models

Table below shows smart home appliances and possible sensor device examples:

| Appliances/ Devices | Average Message Transaction Rate/s | Average Message Size (Bytes) | Date Rate (b/s) | Reliability | Security | Number Per Home | Distribution of arrival |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Home Security System | 1/600 | 201 | 0.2667 | High | High | 1 | Poisson, λ=1/600 |
| Elderly Sensor Devices | 1/60 | 1282 | 17.0667 | High | High | 0.1 | Uniform |
| Refrigerator | 1/3600 | 303 | 0.0667 | Low | Low | 1 | Uniform |
| Clothes Washer | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| Clothes Dryer | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| Dishwasher | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| Freezer | 1/24*60*60 | 303 | 0.0028 | Low | Low | 1 | Uniform |
| Stoves/Ovens | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| Microwaves | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| Coffee Makers | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| Toaster Ovens | 1/24*60*60 | 8 | 7.407e-04 | Low | Low | 1 | Uniform |
| PHEV to SM | 2.4e-4 | 97.6 | 0.0234 | Med | Low | 2 | Uniform |
| Smart Meter | 1.1e-4 | 2017 | 1.775 | High | High | 3 | Uniform |

Table below shows the average number of homes within a cell in different scenarios

| Scenario | Max Cell Radius(m) | Min Cell Radius(m) | *Average number of homes within cell (Max) | *Average number of home within cell (Min) | Aggregated Data Rate in Cell(b/s) (Max) | Aggregated Data Rate in Cell(b/s) (Min) |
| --- | --- | --- | --- | --- | --- | --- |
| Urban (New York City) | 1000 | 500 | 12077 | 3021 | 89610 | 22416 |
| Sub-urban (Washington D.C.) | 1500 | 1000 | 10456 | 4647 | 745370 | 34480 |