| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of M2M multicast service in IEEE P802.16.1b |  |
| Date Submitted | 2011-11-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | IEEE 802.16-11/0028, IEEE 802.16 Working Group Letter Ballot #34 |  |
| Abstract | This contribution proposes the text to clarify the relationship between MGID and FID in M2M multicast service |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of M2M multicast service in IEEE P802.16.1b

Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

According to the definition of MGID in IEEE 802.16.1b, an MGID is assigned to a service flow of an M2M device through AAI-DSA procedure. In this case, an FID included in DSA message with the MGID will not be used to identify the multicast service flow and will be set to default value (e.g., 0b0000). Therefore, some texts related to FID of multicast service flow should be cleaned up.

- Modifications

  - When MGID is updated though AAI-RNG-RSP message, AAI-PAG-ADV message or MGMC message, the related FID is not changed. Delete the FIDs in the related MAC messages

  - Some texts should be modified in subclause 6.2.28.4.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 685 as follows:]

Table 685—AAI-RNG-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Unsolicited bandwidth grant indicator | 1 |  |  |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if the mapping between MGID and FID needs to be updated |
| Current MGID | 15 |  |  |
| Current FID | 4 |  |  |
| New MGID | 15 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| … | … | … | … |

[Remedy #2: Modify the Table 763b as follows:]

Table 763b—AAI-MGMC message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Code | 2 | Use to indentify the purpose if this message 0b00: re-assignment of MGID value 0b01-0b11: Reserved |  |
| if (Action code = 0b00) { |  |  |  |
| Current MGID | 15 | Current MGID value to be assigned |  |
| New MGID | 15 | New MGID value to be assigned |  |
| Current FID | 4 | Current FID value to be assigned |  |
| New FID | 4 | New FID value to be assigned |  |
| } |  |  |  |

[Remedy #3: Modify the Table 706 as follows, on page20, line 60:]

Table 706—AAI-PAG-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| if (Action code = 0b11) { |  |  |  |
| New MGID | 15 | New MGID value to be assigned |  |
| Current FID | 4 | Current FID value to be assigned |  |
| New FID | 4 | New FID value to be assigned |  |
| } |  |  |  |
| … | … | … | … |

6.2.28.4 Multicast operation for machine to machine (M2M) applications

Multicast Service for M2M applications provides concurrent transport of DL data common to M2M devices belonging to an M2M device group using an MGID in an ABS. Multicast service is associated with an ABS and is offered in the downlink only. Each multicast connection is associated with a service flow provisioned with the QoS and traffic parameters for that service flow. Service flows to carry multicast data are instantiated on individual M2M devices participating in the service while in Connected State. During such instantiation, the M2M device learns the parameters that identify the service and associated service flows.

The ABS shall use a combination of MGID and FID to provide the multicast service. The same MGID and FID is assigned to a group of M2M devices that participate in the same multicast service and is assigned by a network during DSA procedure. The FID is same for all M2M multicast service flows.

To access the multicast service, the M2M device that is assigned an MGID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b010, and decimal value = 4094 to decode the assignment A-MAP IE. If the MGID is included in the Broadcast Assignment A-MAP IE, the M2M device shall obtain the multicast burst according to the instruction in the Broadcast Assignment A-MAP IE.

6.2.28.4.1 Multicast operation

An ABS may establish a DL multicast service by creating a multicast connection with each M2M device to be associated with the service. Any available default FID (i.e., 0b0000) may shall be used for the multicast service and the FID is included in AGMH of the multicast MPDU (i.e., there are no dedicated FIDs for multicast transport connections). The multicast connection shall be established using a combination of MGID and FID assigned through AAI-DSA MAC control. Since a multicast connection is associated with a service flow, it is associated with the QoS and traffic parameters of that service flow. For multicast connections, ARQ is not applicable, but a common security key is used to provide encryption and integrity protection for multicast traffic as described in 6.2.5.5.

---- End of proposed text ----