| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Mutual Identity Verification During HO |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Stavros Tzavidas Motorola Inc. 1501 W. Shure Dr. Arlington Heights IL 60004 | Voice: E-mail: stavros.tzavidas@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to Letter Ballot recirc #26a |  |
| Abstract | [Description of document contents.] |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Mutual Identity Verification During HO

Motorola

# Introduction – Problem Description

In the current hard handover (HHO) algorithm, when a mobile station (MS) moves from a Serving Base Station (S-BS) to a Target Base Station (T-BS), the MS and T-BS prove their identity to each other by including a valid CMAC tupple in RNG-REQ and RNG-RSP MAC management messages respectively.

This procedure for the case of fully optimized HHO is depicted in Fig. 1. below.

MOB_HO_IND

MOB_BSHO_REQ / RSP

RNG_REQ

CMAC tupple

UL_MAP (Fast Ranging IE)

RNG_RSP

UL_MAP (data allocation)

DL data

UL data

CMAC tupple

HO-REQ

HO-RSP

MS

S- BS

T-BS

MS switch from S-BS to T-BS

Figure 1: Current method for identity verification during HO

As shown in Fig. 1, the MS sends RNG-REQ MAC management message to BS signed with a shared secret (CMAC key in this case). The BS sends RNG-RSP MAC management message, which is also signed using a valid shared secret (CMAC key). Valid CMAC tupples can be generated only by parties that possess the shared secret and can be verified only by a party that also possesses the shared secret. Therefore correctly signing RNG-REQ/RSP proves possession of the correct CMAC key to the other side.

In the current HO algorithm, data communication resumes only after control messages (RNG-REQ/RSP) have been exchanged and the MS and BS have mutually verified each other’s identity. This procedure increases HO delay, as it delays data communication until at least one pair of control messages have been exchanged and processed. In this contribution we describe a method which enables mutual identity verification to be performed while exchanging data packets, thus allowing data communication to resume immediately after the MS switches to the T-BS during a HO. Using the proposed method, HO delay can be significantly reduced.

# Proposed Solution

We first recognize the fact that one side can prove its identity to the other side by demonstrating that it is capable of creating valid signatures on some specific piece of information. This is true in the case that valid signatures can only be produced by a party that possesses a shared secret (e.g. when the signature is a keyed hash).  In the current HO algorithm, the signed information is a control message (RNG-REQ for the MS and RNG-RSP for the BS) and the shared secret is the CMAC keys. In order to reduce HO delays, we propose to use data packets instead of control packets, as the signed information, and to include a signature which is calculated using the already available shared secrets (CMAC keys) but data instead of control packets as input.

More specifically, we propose to enable both sides (MS and T-BS) to append authentication signatures on data packets. The proposed authentication signature to be appended to data packets is termed “data CMAC” and its construction follows similar rules to those used for generating CMAC tupples for control messages in the current standard.

Under the proposed algorithm, one side can prove its identity to the other side by sending a data PDU, and appending a CMAC tupple TLV on the PDU. The CMAC tupple is calculated using the rules for generating CMAC tupples for unicast control messages, as described in section 7.5.4.4.1 “Calculation of CMAC value” of the current version of the standard, with the only difference that instead of using a MAC management message as input into the calculation of the CMAC tupple, the entire data PDU is used. If the data packet that is being sent belongs to a connection that requires encryption (the most likely case) then the encrypted PDU shall be used as input in the calculation of the data CMAC tupple. The Packet Number (PN) field and all other related fields shall be incremented in the same manner as when sending MAC management messages.

Both sides, when receiving a data packet with an appended CMAC tupple, can verify the identity of the other side by checking the validity of the data CMAC tupple.

It should be noted that even though some of the encryption schemes allowed in the current version of the standard already provide integrity protection (IP) of the data PDUs, it is not desirable to rely on the integrity of the IP fields for verifying the identity of the other side, since the IP fields are generated using a set of keys different than the CMAC keys (namely the TEKs associated with the SA that applies to each particular connection). We wish to maintain the state of affairs in the current standard where identity verification relies on keys derived by the Authentication Key (namely the CMAC keys), and where the two sets of keys (used for authentication and encryption) are decoupled.

The proposed method of mutual identity verification during HO is depicted in Fig. 2. below.

MOB_HO_IND

MOB_BSHO_REQ / RSP

HO-REQ

MS

S- BS

T-BS

data (encrypted)

Data CMAC

data (encrypted)

Data CMAC

HO-RSP

Figure 2: Proposed scheme for identity verification during HO

When one side (MS or BS) receives a data packet with an appended data CMAC tupple, and when the identity of the sender is correctly verified, it sends an acknowledge message to the other side. This acknowledgement, termed “Auth-ACK” is itself signed using the shared secret, and it can also itself be appended to a data packet, if available at the respective direction.

When data is exchanged after a HO, and while mutual identity verification is pending, sent and received data packets need to be handled in a special manner. This is because all data communication that has taken place (packets sent or received) is with a party that may eventually prove to be illegitimate (if identity verification fails).

For example, suppose the the MS starts sending data packets to a T-BS, while the identity verification process is still in progress. In this case, the MS is not yet sure of identity of the BS and therefore it must store sent data packets and it must further encrypt them (if the connection requires encryption) so that if authentication of the other side fails, the contents of the data packet are not revealed. The packets must be stored because if the BS fails to prove its identity then the MS will have to attempt to connect to a different BS and re-send the packets there. Similarly, the BS when sending data to an MS, and while the identity verification process is pending, must store and (if applicable) encrypt the data packets.

Both sides, when receiving the first packet, first verify the authentication field to verify that it was produced from a sender possessing the right shared secret. If the authentication field is verified, the other party is marked as authenticated and an ACK message is sent.

Furthermore, all packets (including the first one) received while identity verification is pending are stored and not released to the upper layers (also possibly not decrypted either since, if identity verification fails they will be discarded). If identity verification of the other side succeeds, received packets are decrypted and delivered to the higher layers. If identity verification fails, received packets are not decrypted and discarded.

The process described above is summarized in the state diagram in Fig. 3. Note that, under the proposed scheme, both sides (MS and BS) implement this state machine independently during a HO.

Tauth expired

/ start Tauth

/ (re)create authentication field, append it to data packet, if available

Packet received which successfully authenticates other side

/ flush packets stored pending authentication (unless required otherwise by QoS and/or retransmission protocols)

/ Create AUTH-ACK, append it to data packet if available and send it

Received ACK for sent authentication credentials (implicitly also authenticates other side)

/ flush packets stored pending authentication (unless required otherwise by QoS and/or retransmission protocols)

/ stop Tauth

Received ACK for sent authentication credentials

/ stop Tauth

“Other side AUTH” = FALSE

“AUTH by other side” = FALSE

While sending data packets, encrypt them and store them pending authentication of other side. Remove sent packets from data queues only if expired under QoS rules

“Other side AUTH” = TRUE

“AUTH by other side” = FALSE

“Other side AUTH” = TRUE

“AUTH by other side” = TRUE

START

/ start Tauth

/ create authentication field, append it to data packet, if available

Tauth expired

/ start Tauth

/ (re)create authentication field, append it to data packet, if available

Figure 3: State machine for proposed HO identity verification scheme

The terms used in Fig. 3 are defined as follows:

- “Other side AUTH”: set to TRUE when the identity of the other side is verified

- “AUTH by other side”: set to TRUE when the sending side can be sure that the other side has verified its identity. For example MS sets this variable to TRUE when it is sure that the BS has verified the MS’ identity (i.e. when it receives an ACK from BS)

- Authentication key: the shared secret (e.g. AK, CMAC key etc)

- Authentication field: The “data CMAC tupple”. A field containing MS or BS ID, HOID, signed with authentication key. Can be appended to data packets or other control messages, or can be sent in independent control message when no data or other control packet is scheduled in that direction. When appended to other message, the contents of that message are also input to the calculation of the authentication field.

- Tauth: timer controlling retransmissions of authentication field

- AUTH-ACK: acknowledgement for receiving authentication field from other side Contains a copy (or portion) of authentication field sent by other side and is also signed using authentication key

By implementing the state machine in Fig. 3, mutual identity verification during a HO can be achieved through the process depicted in Fig. 2, thus allowing data communication to resume immediately after an MS switches to a T-BS during HO. This procedure clearly reduces HO delay, as desired.

In closing, we describe how some special cases are to be handled under the proposed algorithm:

- When no data flow exists in a particular direction: simply send control message with authentication field in that direction. If no data flows exist in either direction then use control messages for both. If no data flow exists, there is no data to be delayed and sending control messages is not a concern.

- How long are data packets stored for, while authentication of the other side is pending:

  - Data packets that have been sent to the other side, while authentication of the other side is pending are stored until either:

    - The identity of the other side is verified and the packets are ACKed (if HARQ or ARQ is used)

    - Packets expire, per the QoS requirements of the flow to which they belong (if the packets expire, then there is no point in retransmitting them anyway, if the authentication fields need to be retransmitted they will be appended to a different packet)

- When authentication field is retransmitted, does it have to be with the same data packet as the original transmission?: No, when the authentication field is retransmitted, it can be appended to a different data packet, or it can be put in a control packet if no data packet is available in the respective direction at that time. The authentication field will be re-calculated based on the contents of the new data packet.

- How does a side know that it has been authenticated by the other side? The authentication ACK notifies one side that it has been successfully authenticated by the other side.

# Proposed Text Changes

[TBD]

# References

[IEEE802.16-Rev2/D2]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D2 (December 2007) Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007 .