| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A more efficient DL-MAP IE for MAPs |  |
| Date Submitted | 2008-05-08 |  |
| Source(s) | Kanchei (Ken) Loa, Tsung-Yu Tsai, Yi-Hsueh Tsai, Yung-Ting Lee Institute for Information Industry 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan [add other co-authors here] | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16j-08/020: “IEEE 802.16 Working Group Letter Ballot Recirc #28c: Announcement” |  |
| Abstract | This contribution proposes the compressed DL-MAP IE |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A more efficient DL-MAP IE for MAPs

Kanchei (Ken) Loa, Tsung-Yu Tsai, Yi-Hsueh Tsai, Yung-Ting Lee

Institute for Information Industry (III)

Introduction

In the current definition of the allocation of downlink data in WirelessMAN-OFDMA, each allocation (i.e. a burst) is indicated as a two-dimensional rectangle located in a table with time axis (OFDMA symbol) and frequency axis (sub-channel). For description of such a 2-D allocation, the current DL-MAP IE adopts totally four parameters, that is, a “OFDMA symbol offset” and a “OFDMA sub-channel offset” to locate the coordinate of the left-top point of the allocated rectangular, then, “number of OFDMA symbols” and “number of OFDMA sub-channels” to indicate its length and width respectively. This approach is quite intuitive, but seems not be efficient as well, since some parameters may not be necessary and bring additional MAP overhead. In this contribution, we prove that even without the knowledge of symbol offset and sub-channel offset, if we know the transmission sequence of each IE is conformed to some rules which are relative to the location of the bursts (e.g. appear along the time axis in a frequency first manner), it can also provide sufficient information to uniquely determine the location of all the bursts with only the knowledge of their length and width. Furthermore, based on the result of our proving, we propose a more efficient DL-MAP IE with only two parameters: “number of OFDMA symbols” and “number of sub-channels”. Theorically speaking, the new DL-MAP IE can save up to 30% overhead per IE. This improvement will become significant in several prevalent scenarios such as environment dominant by services with low data rate and small packet like VoIP where large amount of bursts may be aggregated in the same frame, or environment supporting multi-hop relay with non-transparent relay stations where not only DL-MAP but also R-MAP transmitted in a frame, where the impact of MAP overhead becomes more crucial.

Proposed MAP IE

Table 1 is our proposed new DL-MAP IE. Briefly speaking, our proposed new IE has two main differences with that of the conventional DL-MAP IE in current draft document. First, instead of using four parameters to describe a burst, our new IE removes two parameters and only uses “number of OFDMA symbols” and “number of sub-channels” to represent a burst, thus our new IE has much less overhead. Second, the sequence of IEs is relative to the location (the left-top coordinate) of each bursts in which appears along the symbol axis in a frequency-first manner. That is, if the symbol offset of a burst is smaller than that of another burst, the IE of this burst must be prior to that of another burst. For two bursts with the same symbol offset, the IE of the one with smaller sub-channel offset is the prior. With the information in the IEs, one can uniquely identify the location of each burst. The prove of the uniqueness and a demonstrative solution to decode the IEs are shown in Annex.

Table 1 DL-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| DL-MAP_IE() { |  |  |
| DIUC | 4 bits |  |
| if (DIUC == 14 { |  |  |
| Extended-2 DIUC dependent IE | variable |  |
| } else if (DIUC == 15) { |  |  |
| Extended DIUC dependent IE | variable | See subclauses following 8.4.5.3.1 |
| } else { |  |  |
| if (INC_CID == 1) { |  | The DL-MAP starts with INC_CID =0. INC_CID is toggled between 0 and 1 by the CID-SWITCH_IE() (8.4.5.3.7) |
| N_CID | 8 bits | Number of CIDs assigned for this IE |
| for (n=0; n< N_CID; n++) { |  |  |
| If ( included in SUB-DL-UL-MAP) { |  |  |
| RCID_IE() | variable | For SUB-DL-UL-MAP, reduced CID format is used |
| } else { |  |  |
| CID | 16 bits | Represents the assignment of the IE to a broadcast, multicast, or unicast address. |
| } |  |  |
| } |  |  |
| } |  |  |
| OFDMA Symbol offset | 8 bits |  |
| if (Permutation = 0b11 and (AMC type is 2x3 or 1x6)) { |  |  |
| Subchannel offset | 8 bits |  |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| No. OFDMA triple symbol | 5 bits | Number of OFDMA symbols is given in multiples of 3 symbols |
| No. Subchannels | 6 bits |  |
| } else { |  |  |
| Subchannel offset | 6 bits |  |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| No. OFDMA Symbols | 7 bits |  |
| No. Subchannels | 6 bits |  |
| } |  |  |
| Repetition Coding Indication | 2 bits | 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| } |  |  |
| } |  |  |

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D4 are listed below.

Spec changes

8.4.5.3 DL-MAP IE format

[Insert the following paragraphs at end of 8.4.5.3 DL-MAP:]

Table xxx Compressed DL-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Compressed_DL-MAP_IE() { | - | - |
| DIUC | 4 bits |  |
| if (DIUC == 14 { | - | - |
| Extended-2 DIUC dependent IE | variable |  |
| } else if (DIUC == 15) { | - | - |
| Extended DIUC dependent IE | variable | See subclauses following 8.4.5.3.1 |
| } else { | - |  |
| if (INC_CID == 1) { | - | The DL-MAP starts with INC_CID =0. INC_CID is toggled between 0 and 1 by the CID-SWITCH_IE() (8.4.5.3.7) |
| N_CID | 4 bits | Number of CIDs assigned for this IE |
| for (n=0; n< N_CID; n++) { | - | - |
| RCID_IE() | variable | For R-MAP, RS_Access-MAP and RS_Relay-MAP, reduced CID format is used |
| } | - | - |
| } | - | - |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| if (Permutation = 0b11 and (AMC type is 2x3 or 1x6)) { | - | - |
| No. OFDMA triple symbol | 5 bits | Number of OFDMA symbols is given in multiples of 3 symbols |
| } else { | - | - |
| No. OFDMA Symbols | 7 bits |  |
| } | - | - |
| No. Subchannels | 6 bits |  |
| Repetition Coding Indication | 2 bits | 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| } | - | - |
| } | - | - |

[Modify the following subclause 6.3.2.3.90 as following indicated:]

6.3.2.3.90 RS access MAP (RS_Access-MAP) message

Table 183ac—RS Access MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Indicator | 8 bits | Bit 0: 0: Parameters of DL_Frame_Prefix remain same with the latest Configuration. 1: The parameters of DL_Frame_Prefix are updated. Bit 1: 0: RS shall use Normal map format, 1: RS shall use Compressed map format Bit 2: 0: DL-MAP not included 1: DL-MAP included Bit 3: 0: UL-MAP not included 1: UL-MAP included Bit 4: 0: SUB-DL-UL-MAP not included 1: SUB-DL-UL-MAP included Bit 5: 0: HARQ-MAP not included 1: HARQ-MAP included Bit 6: 0: Normal DL-MAP IE 1: Compressed DL-MAP IE Bit 6-7: reserved |

| DL_IE count | 8 bits | Number of DL_IE in the burst |
| --- | --- | --- |
| If(DL-MAP IE format == 0b0) { |  |  |
| for(i=0;i<DL_IE count; i++){ |  |  |
| DL-MAP_IE() | variable |  |
| } |  |  |
| } else { |  |  |
| RCID_Type | 2 bits | 0b00 = Normal CID 0b01 = RCID 11 0b10 = RCID 7 0b11 = RCID 3 |
| for(i=0;i<DL_IE count; i++){ |  |  |
| Compressed_DL-MAP_IE() | variable |  |
| } |  |  |
| } |  |  |

[Modify the following subclause 8.4.5.10 as following indicated:]

8.4.5.10 R-MAP message

Table 496a—R-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RCID_Type | 2 bits | 0b00 = Normal CID 0b01 = RCID 11 0b10 = RCID 7 0b11 = RCID 3 |
| DL-MAP IE format | 1 bit | 0b0 = Normal DL-MAP IE 0b1 = Compressed DL-MAP IE |
| Length | 11 bits | Length of R-MAP in bytes |
| DL_IE count | 6 bits | Number of DL_IE in the burst |
| UL_IE count | 6 bits | Number of UL_IE in the burst |
| for(i=0;i<DL_IE count; i++){ |  |  |
| If(DL-MAP IE format == 0b0) { |  |  |
| DL_-MAP_IE() | variable |  |
| } else { |  |  |
| Compressed_DL-MAP_IE() | variable |  |
| } |  |  |
| } |  |  |

Annex

Figure 1 Rectangular zone

In Figure 1, let the rectangular zone and the k-th burst denote as R and Bk, and the left-top corner of the R and Bk are pR  <cR, sR> and pk  <ck, sk>, where CR and SR are number of the sub-channel and the OFDMA symbol of the zone, respectively; Ck and Sk are numbers of the sub-channel and the OFDMA symbol of the k-th burst, respectively; CR, SR, Ck, Sk  Positive Integer; cR, sR, ck, sk  Non-negative Integer.

- Define f(p) = a(p  pR) = (c  cR) + CR(s  sR), where a  (1, CR) and p  (c, s) represent the location of the point at the c-th sub-channel and the s-th OFDMA symbol. The following axioms shall be satisfied

- f(pR) = 0
proof:	f(pR) = a(pR  pR) = 0

- CRSR  1  f(p)  0 for every p  R
proof:	CR  1  c  cR  0 and SR  1  s  sR  0 for every p  R
	 f(p) = (c  cR) + CR(s  sR)  0 for every p  R
	 f(p) = (c  cR) + CR(s  sR)  (CR  1) + CR(SR  1) = CRSR  1 for every p  R
	proofed

- f(pi) = f(pj) if and only if pi = pj, for every pi, pj  R

- f(pi)  f(pj) if and only if pi  pj, for every pi, pj  R
proof: () if ci = cj and si = sj
	 f(pi)  f(pi) = (ci  cj) + CR(si  sj) = 0
	 f(pi) = f(pj)
	() if ci  cj and si = sj
	 f(pi)  f(pi)| = (ci  cj) + CR(si  sj) = (ci  cj)  0
	 f(pi)  f(pj)
	() if si  sj
	 CR  1  |ci  cj|  0 and |si  sj|  1
	 |f(pi)  f(pj)| = |(ci  cj) + CR(si  sj)|  CR|(si  sj)|  |(ci  cj)|  CR  (CR  1) = 1
	 f(pi)  f(pj)
	()proofed

- pk  Bk and f(pk)  f(p) for every p  Bk if and only if pk is the left-top corner of Bk
proof: () if pk  Bk and pk is the left-top corner of Bk
	 c  ck and s  sk for every p  Bk
	 f(p) = (c  cR) + CR(s  sR)  (ck  cR) + CR(sk  sR) = f(pk) for every p  Bk
	() let q  Bk and f(q)  f(p) for every p  Bk and pk is the left-top corner of Bk
	 f(pk)  f(q)  f(pk)
	 f(q) = f(pk)
	 from axiom (3), q = pk
	() proofed

- Assume the whole zone is filled up non-overlapped bursts, i.e., R=i=1NBi and Bi  Bj =  for i  j. The following axioms shall be satisfied

- Let p  R, then we can find only one Bk  R such that p  Bk
proof:	assume p  Bk for every Bk  R
	 p∉∪i=1NBi=R
	 conflict, we can find at least Bl  R such that p  Bl
	assume we can find Bi, Bj  R such that p  Bi and p  Bj
	 p  Bi  Bj = 
	 conflict, we can find only one Bl  R such that (c, s)  Bl

- Let Qk=i=kNBi and q  Qk such that f(q)  f(p) for every p  Qk, then we can find only one Bl  Qk such that q is the left-top corner of Bl
proof:	assume q  Bl for every Bl  Qk
	 q∉∪i=kNBi=Qk
	 conflict, we can find at least Bl  Qk such that q  Bl
	f(q)  f(p) for every p  Qk
	 f(q)  f(p) for every p  Bl  Qk
	 from axiom (5), q is the left-top corner of Bl

- Assume the k-th burst IE, denotes as IEk, contains non-zero parameter (Ck, Sk) and appears in the MAP with following sequence IE1, …, IEk , …, IEN, where i < j if f(pi) < f(pj) and pk denotes the left-top corner of the k-th burst (Bk); Ck and Sk are numbers of the sub-channel and the OFDMA symbol of the k-th burst, respectively; Ck, Sk  Positive Integer; ck, sk  Non-negative Integer. The following axioms shall be satisfied

- The sequence of IE: IE1, …, IEk , …, IEN is unique
proof:	Bi  Bj =  for i  j
	 pi  pj, where pk denotes the left-top corner of the k-th burst
	 from axiom (4), f(pi)  f(pj)
	 The sequence of IE: IE1, …, IEk , …, IEN is unique

- Let pR  <cR, sR> denotes the left-top corner of the zone (R), pk  <ck, sk> denotes the left-top corner of the k-th burst (Bk), Dk = {p| p  <c, s>; Ck > c  0 and Sk > s  0 } and Bk = Dk + pk = {p + pk| p  Dk}, then the estimated left-top corner of the k-th burst is qk can be determined as follows:
(a) The left-top corner of the 1st IE is q1 = pR.
(b) Let Q2 = R  C1 = {p| p  R and p  C1}, where C1 = D1 + q1 = {p + q1| p  D1}
(c) The left-top corner of the kth IE is qk such that f(qk)  f(p) for every p  Qk
(d) Let Qk+1≡Qk−Ck=p|p∈R and p∉∪i=1kCi, where Ck = Dk + qk = {p + qk| p  Dk}
The following axioms shall be satisfied

- Based on (Ck, Sk) and sequence IE1, …, IEk , …, IEN, qk = pk for i = 1 to N
proof: from axiom (5) and (6)
	 we can find only one Bk  R such that pR  Bk and pR is the left-top corner of Bk
	 Bk = B1 since f(pR) < f(pk) for every pk  R and pk  pR
	assume qk = pk for k = 1 …, n
	 Cn = Dn + qn = Dn + pn = Bn
		and Qn+1≡Qn−Bn=p|p∈R and p∉∪i=1nBi=p|p∈∪i=n+1NBi
	 let qn+1 is the point such that f(qn+1)  f(p) for every p  Qn+1
	 from (7), we can find only one Bl  Qn+1 such that qn+1 is the left-top corner of Bl
	 qn+1=pl∈pk|k=n+1,...,N
	 qn+1 = pl = pn+1 since f(qn+1)  f(pn+1)  f(pk) for every pk  Qn+1

- Let Bm is a known rectangular zone with (Cm, Sk) and the left-top corner pm and let R'=R−Bm=i=1NBi. Assume the k-th burst IE, denotes as IEk, contains non-zero parameter (Ck, Sk) and appears in the MAP with following sequence IE1, …, IEm-1, IEm+1, …, IEN, where i < j if f(pi) < f(pj) and pk denotes the left-top corner of the k-th burst (Bk); Ck and Sk are numbers of the sub-channel and the OFDMA symbol of the k-th burst, respectively; Ck, Sk  Positive Integer; ck, sk  Non-negative Integer.
Let pR  <cR, sR> denotes the left-top corner of the zone (R), pk  <ck, sk> denotes the left-top corner of the k-th burst (Bk), Dk = {p| p  <c, s>; Ck > c  0 and Sk > s  0 } and Bk = Dk + pk = {p + pk| p  Dk}, then the estimated left-top corner of the k-th burst is qk can be determined as follows:
(a) The left-top corner of the 1st IE is q1 = pR.
(b) Let Q2 = R  C1 = {p| p  R and p  C1}, where C1 = D1 + q1 = {p + q1| p  D1}
(c) The left-top corner of the kth IE is qk such that f(qk)  f(p) for every p  Qk
(d) Let Qk+1≡Qk−Ck=p|p∈R and p∉∪i=1kCi, where Ck = Dk + qk = {p + qk| p  Dk 
The following axioms shall be satisfied

- The sequence of IE: IE1, …, IEm-1, IEm+1, …, IEN is unique
proof:	Bi  Bj =  for i  j
	 pi  pj, where pk denotes the left-top corner of the k-th burst
	 from axiom (4), f(pi)  f(pj)
	 The sequence of IE: IE1, …, IEm-1, IEm+1, …, IEN is unique

- Based on (Ck, Sk) and sequence IE1, …, IEm-1, IEm+1, …, IEN, qk = pk for i = 1 to N
proof: from axiom (9), we can find qk = pk for i = 1 to m  1
	since Bm is a known rectangular zone thus qm = pm
	then from axiom (9), we can find qk = pk for i = m + 1 to N