| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Remedy on RS_BW-ALLOC_IE |  |
| Date Submitted | 2008-07-15 |  |
| Source(s) | Kanchei(Ken) Loa, Chih-Peng Hsu, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin Institute for Information Industry Shiann-Tsong Sheu National Central University (NCU) Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) | E-mail: loa@iii.org.tw |
| Re: | IEEE 802.16-08/028: “IEEE 802.16 Working Group Letter Ballot Recirc #28d: Announcement” |  |
| Abstract | This contribution proposes a method to provide sufficient RS broadcast message relaying scheme. |  |
| Purpose | Discuss and adopt proposed text in TG16j |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on RS_BW-ALLOC_IE

# Introduction

As defined in current draft 16j/D5, under centralized scheduling, the MR-BS should schedule the downlink access bandwidth in RS_Access-MAP message for its subordinate RS. It should notify the subordinate RS the allocated burst by RS_BW-ALLOC_IE in R-MAP message for sending RNG-RSP message or broadcasting UCD/DCD message. To handle this task, the RS has to find the RS_Access-MAP message associated with the RS_BW-ALLOC_IE in R-MAP message, which could be received in the different frame. That is, the RS has to be capable of handling the inconsistency between the received RS_Access-MAP and R-MAP messages, if any.

In order to simplify the procedure, we suggest moving the RS_BW-ALLOC_IE from the R-MAP message to the RS_Access-MAP message. As a result, the subordinate RS could compose the DL-MAP message and find the designated burst to send the broadcast message (such as RNG-RSP, UCD, DCD) by processing the RS_Access-MAP message.

For more than two hop MR system, the MR-BS should also schedule the downlink relay bandwidth in the RS_Relay-MAP message for its subordinate RS to broadcast RCD. For the same reason, we suggest moving the RS_BW-ALLOC_IE from R-MAP message to the RS_Relay-MAP message.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D5 are listed below.

# Proposed Text

6.3.2.3.87 RS access MAP (RS_Access-MAP) message

Table 183z—RS Access MAP message format

| Syntax | Size | Note |
| --- | --- | --- |
| RS _Access-MAP_Message_Format{ | - | - |
| Indicator | 8bits | Bit 0: 0: Parameters of DL_Frame_Prefix remain same with the latest Configuration. 1: The parameters of DL_Frame_Prefix are updated. Bit 1: 0: RS shall use Normal map format, 1: RS shall use Compressed map format Bit 2: 0: DL-MAP not included 1: DL-MAP included Bit 3: 0: UL-MAP not included 1: UL-MAP included Bit 4: 0: SUB-DL-UL-MAP not included 1: SUB-DL-UL-MAP included Bit 5: 0: HARQ-MAP not included 1: HARQ-MAP included Bit 6: 0: RS_BW-ALLOC_IE not include 1: RS_BW-ALLOC_IE include Bit 6-7: reserved |
| If(bit #6 of Indicator == 1) { |  |  |
| Nr of IE | 4 bits | Number of IE |
| For (i = 0; i < Nr. of IE; i++) { |  |  |
| RS_BW-ALLOC_IE () | variable |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

[insert the following Table on line 29 of page 79 as follows ]

RS_BW-ALLOC IE is transmitted to an RS from MR-BS in RS_Access-MAP message. This IE provides the allocation to RS for transmission of messages composed by the RS over the access link to MSs. An RS may modify the CID in the DL-MAP IE pointed by RS_BW-ALLOC_IE.

| RS_BW-ALLOC_IE () { |  |  |
| --- | --- | --- |
| Type | 2bits | 0b00:Response for RS BR header 0b01:RS broadcasting RNG-RSP 0b10: unsolicited DL bandwidth allocation to RS by MR-BS 0b11:reserved |
| If(Type==0x00) { |  |  |
| TID | 4bits | Transaction ID |
| DL-MAP IE index | 8bits | RS shall transmit message on the burst described by the DL MAP IE within the DL-MAP message described in the RS_Access-MAP. |
| }else if(Type==0x01) | - | - |
| Frame Number Index | 4bits | LSBs of relevant frame number |
| Number of rejected SS | 4bits | Number of rejected SS (i.e. RNG-RSP message with status “Abort”) |
| INC_RNG_SUC | 1bit | Include bandwidth for RNG-RSP message with status “success” (0b1:no, 0b1:yes) |
| INC_DFO | 1bit | Include bandwidth for RNG-RSP message containing downlink frequency override (0b1:no, 0b1:yes) |
| DL-MAP IE index | 8bits | RS shall transmit message on the burst described by the DL MAP IE within the DL-MAP message described in the RS_Access-MAP. |
| }else if(Type==0x10) | - | - |
| Message Type | 2 bits | 0b00: DCD 0b01: UCD 0b10-0b11:reserved |
| DL-MAP IE index | 8bits | RS shall transmit message on the burst described by the DL MAP IE within the DL-MAP message described in the RS_Access-MAP. |
| } |  |  |
| } |  |  |

6.3.2.3.88 RS relay MAP (RS_Relay-MAP) message

Table 183ad—RS relay MAP message format

| Syntax | Size | Note |
| --- | --- | --- |
| RS _Relay-MAP_Message_Format{ | - | - |
| Indicator | 1 bits | 0: R-DL-MAP IE index for the RCD message not include 1: R-DL-MAP IE index for the RCD message include |
| If(Indicator == 0b1) { |  |  |
| R-DL-MAP IE index | 8bits | RS shall transmit the RCD message on the burst described by the R-DL MAP IE within the R-DL-MAP message broadcasted by the RS at the next available frame after receiving this IE. |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

[Delete the following subclause as indicated:]

8.4.5.10.1.2 RS bandwidth allocation IE (RS_BW-ALLOC_IE)