| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to primary carrier change for the IEEE 802.16m/D2 (15.2.8.2.11) |  |
| Date Submitted | 2009-11-6 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim LG Electronics | Voice : +82-31-450-1876 E-mail: mehappy3@lge.com, ksryu@lge.com, sixs@lge.com, ronnykim@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to primary carrier change for the IEEE 802.16m/D2 (15.2.8.2.11)

Eunjong Lee, Kiseon Ryu, Youngsoo Yuk and Ronny Yongho Kim

LG Electronics

- Introduction

This contribution clarifies the procedure for primary carrier change.

- Discussions

Figure 1 illustrates the procedure for primary carrier change based on the description in 15.2.8.2.11.2 Primary Carrier Change of AWD [6]. When an AMS receives the AAI_CM-CMD MAC control message, the AMS transmits the AAI_CM-IND MAC control message and the AMS changes its primary carrier to the target fully configured carrier at action time specified by the ABS. However, if the action time is set to an insufficient value for ensuring the reception of AAI_CM-IND, the ABS can misunderstand that the AMS fails the reception of AAI_CM-CMD or rejects action directed by AAI_CM-CMD although the AMS is ready to transmit the AAI_CM-IND message. As a result, this leads to the failure of primary carrier change. Figure 2 shows an example for the error case of primary carrier change procedure.

Figure 1. Primary carrier change procedure based on current AWD

Figure 2. An example for the error case of current primary carrier change procedure

- Proposals

In order to provide clearer procedure of primary carrier change, we propose to clarify the action time defined in AAI_CM-CMD. In our clarified procedure, the ABS shall start the retransmission timer for AAI_CM-CMD when transmitting the AAI_CM-CMD message, as described in 15.2.21 MAC Management Reliability of current AWD [6] and the action time included in AAI_CM-CMD shall be set to the value more than the retransmission timer. If the ABS receives the AAI_CM-IND message before the expiration of the retransmission timer, ABS stops the timer and performs the corresponding procedure. Else, the ABS may retransmit the AAI_CM-CMD message. If the AAI_CM-CMD is retransmitted, the retransmission timer shall restart and the action time included in AAI_CM-CMD shall be also modified based on the restarted retransmission timer.

If the local NACK is enabled for AAI_CM-IND, the AMS can detect the failure of the corresponding procedure via the local NACK when the HARQ process fails. In addition, if the AMS requests ABS to send a MAC layer acknowledgement for the control message by setting the polling bit in FEH, the AMS can also detect the success or failure of the corresponding procedure through AAI_MSG_ACK message or Message ACK EH. For the reliable transmission of the AAI_CM-CMD and AAI_CM-IND, it shall follow the procedure described in section 15.2.21 MAC Management Reliability of AWD [6].

Figure 3. Proposed Primary carrier change procedure

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-09/0034, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the text at line 8 on page 160 as follows:]

15.2.8.2.11.2 Primary Carrier Change

The ABS may instruct the AMS, through the AAI_CM-CMD MAC control message on the current primary carrier, to change its primary carrier to one of the assigned fully configured carriers within the same ABS for load balancing purpose, carriers' varying channel quality or other reasons. When an AMS receives the AAI_CM-CMD MAC control message, the AMS transmits the AAI_CM-IND MAC control message and the AMS switches to the target fully configured carrier at action time specified by the ABS. The action time in AAI_CM-CMD shall be set to the value more than the retransmission timer for AAI_CM-CMD message. If the target carrier is one of the active secondary carriers of the AMS, the AMS may receive data and control signal on the target carrier immediately after switching. Otherwise, the AMS may transmit a bandwidth request to notify its readiness of the target carrier to the ABS. If the AMS has UL data to be transmitted, it follows the normal bandwidth request procedure. If the AMS does not have any UL data, it transmits a bandwidth request with zero bandwidth request size. The ABS may transmit data and control signal after a bandwidth request is received from the AMS through the target primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. The ABS may provide the system information of the target primary carrier that is different from the serving primary carrier via the serving primary carrier. The logical carrier indices of the serving and target primary carrier are swapped after the primary carrier change. The ABS may direct an AMS to change the primary carrier without scanning.

[Insert the text at line 7 on page 161 as follows:]

15.2.8.2.11.4 MAC Control Messages for Carrier Management

15.2.8.2.11.4.1 Carrier Management Command (AAI_CM-CMD) MAC control message

An AAI_CM-CMD message shall be transmitted by an ABS to request a procedure related to the carrier management, such as carrier activation/deactivation, primary carrier change or carrier switching. When an AMS receives the AAI_CM-CMD MAC management message, the AMS shall transmit the AAI_CM-IND MAC management message.

The ABS shall start retransmission timer for the AAI_CM-CMD when transmitting the AAI_CM-CMD message as described in <<15.2.21>>. If the ABS successfully receives the AAI_CM-IND message before expiration of the retransmission timer, ABS stops the timer and performs the corresponding procedure. If the retransmission timer is expired, the ABS may retransmit AAI_CM-CMD message.

Table 726—AAI_CM-CMD MAC Management Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management Command message format(){ |  |  |
| Control Message Type | 8 | AAI_CM-CMD |
| Action code | 2 | 00: Secondary carrier management 01: Primary carrier change 10: Carrier switching 11: reserved |
| If (Action code == 00){ |  | This message is for secondary carrier activation and/or deactivation |
| Indication Type | 2 | For Activation and/or Deactivation Bit #0: ‘0’: No action, ‘1’: Activation Bit #1: ‘0’: No action, ‘1’: Deactivation |
| If (Indication Type #0 == 1) |  |  |
| Num of target carrier | [TBD] | The number of newly activated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for activation |
| Activation of DL/UL | 1 | ‘0’: Both DL/UL are activated ‘1’: DL is activated but UL is not activated |
| Ranging indicator | 1 | Ranging indicator for target carrier ‘0’: No ranging is required for the target carrier ‘1’: Periodic ranging is required for the target carrier |
| } |  |  |
| } |  |  |
| If (Indication Type #1 == 1){ |  |  |
| Num of target carrier | [TBD] | The number of deactivated carrier(s) |
| For(i=0; i< Num of target carrier; i++){ |  |  |
| Target carrier index | [TBD] | Target carrier index for deactivation |
| Deactivation of DL/UL | 1 | ‘0’: Both DL/UL are deactivated ‘1’: UL is deactivated but DL is kept active |
| } |  |  |
| } |  |  |
| } |  |  |
| If (Action code == 01){ |  | This message is for primary carrier change. |
| Target carrier index | [TBD] | Target carrier index for primary carrier change |
| Action Time | [TBD] | LSB bits of Superframe number at the time to switch to the target carrier. This value shall be set to the value more than the retransmission timer for AAI_CM-CMD message. |
| Next state of serving primary carrier | 1 | ‘0’: Serving carrier will be deactivated after primary carrier change ‘1’: Serving carrier is kept active after primary carrier change |
| } |  |  |
| If (Action code == 10){ |  | This message is for secondary carrier switching. |
| Target carrier index | [TBD] | Target carrier index for carrier switching |
| Switching mode | 1 | Periodic or event-triggered switching 0: Periodic switching 1: Event-triggered switching |
| Start time | [TBD] | TBD LSB bits of superframe number at the start time of carrier switching |
| Switching interval | [TBD] | Number of superframes during which an AMS stays at the target secondary carrier before returning to the primary carrier |
| If (Switching mode == 0){ |  |  |
| Switching period | [TBD] | Number of superframes between consecutive switching interval |
| } |  |  |
| } |  |  |
| } |  |  |

15.2.8.2.11.4.2 Carrier Management Indication (AAI_CM-IND) MAC control message

An AAI_CM-IND message shall be transmitted by an AMS as a response for AAI_CM-CMD message. In order to ensure the reliability of this message, the AMS shall follow the procedure described in <<15.2.21>> when transmitting this message.

Table 727—AAI_CM-IND MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_Carrier Management-Indication message format(){ |  |  |
| Control message type | 8 | AAI_CM-IND |
| Action code | 2 | 00 : Secondary carrier management 01: Primary carrier change 10: Carrier switching 11: reserved |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------