| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to Tables 850 and 851 (Quantization Parameters of Matrix R) Section 15.3.7.2.5.6.1 of the IEEE 802.16m/D2 |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Alexei Davydov | alexei.davydov@intel.com |
| Re: | Category: LB30a / Area: Chapter 15.3.7 (DL MIMO) |  |
| Abstract | The contribution proposes binary encoding for the quantized elements of matrix R |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Modifications to Tables 850 and 851 (Quantization Parameters of Matrix R) Section 15.3.7.2.5.6.1 of the IEEE 802.16m/D2

Alexei Davydov

Intel Corporation

# Motivation

# Binary encoding of quantized elements of matrix R is not defined in 802.16m/D2.

# Text proposal for inclusion in the 802.16m D2

<Modify contents of Table 850 Quantization parameters for diagonal entries of R and Table 851 Quantization parameters for non-diagonal entries of R on page 435 line 60>

--------------------------------------------------  Text Start  ------------------------------------------------

Table 850 Quantization parameters for diagonal entries of R

| Diagonal Entries | binary encoding | a | b | q |
| --- | --- | --- | --- | --- |
| q1 | 0 | 0.6 | 0 | 0.6000 |
| q2 | 1 | 0.9 | 0 | 0.9000 |

Table 851 Quantization parameters for non-diagonal entries of R

| Non-Diagonal Entries | binary encoding | a | b | q |
| --- | --- | --- | --- | --- |
| q1 | 0000 | 0.1 | 0 | 0.1000 |
| q2 | 0001 | 0.1 | 1/8 | 0.0707 + 0.0707i |
| q3 | 0010 | 0.1 | 1/4 | 0.0000 + 0.1000i |
| q4 | 0011 | 0.1 | 3/8 | -0.0707 + 0.0707i |
| q5 | 0100 | 0.1 | 1/2 | -0.1000 + 0.0000i |
| q6 | 0101 | 0.1 | 5/8 | -0.0707 - 0.0707i |
| q7 | 0110 | 0.1 | 3/4 | -0.0000 - 0.1000i |
| q8 | 0111 | 0.1 | 7/8 | 0.0707 - 0.0707i |
| q9 | 1000 | 0.5 | 0 | 0.5000 |
| q10 | 1001 | 0.5 | 1/8 | 0.3536 + 0.3536i |
| q11 | 1010 | 0.5 | 1/4 | 0.0000 + 0.5000i |
| q12 | 1011 | 0.5 | 3/8 | -0.3536 + 0.3536i |
| q13 | 1100 | 0.5 | 1/2 | -0.5000 + 0.0000i |
| q14 | 1101 | 0.5 | 5/8 | -0.3536 - 0.3536i |
| q15 | 1110 | 0.5 | 3/4 | -0.0000 - 0.5000i |
| q16 | 1111 | 0.5 | 7/8 | 0.3536 - 0.3536i |

---------------------------------------------------  Text End  ---------------------------------------------------