| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional Frame Configuration for Legacy Support Mode (15.3.3.5.1) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Jaeweon Cho, Hokyu Choi Samsung Electronics Hujun Yin Intel Masoud Olfat Clearwire Jin Sam Kwak LGE | jaeweon.cho@samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot #30a” |  |
| Abstract | This contribution proposes additional frame configurations for legacy support mode. The proposed frame configuration allows a different mix ratio of 16e and 16m between adjacent ABSs while keeping the same frame offset. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional Frame Configuration for Legacy Support Mode (15.3.3.5.1)

Jaeweon Cho, Hokyu Choi

Samsung Electronics

Hujun Yin

Intel

Masoud Olfat

Clearwire

Jin Sam Kwak

LGE

- Introduction

In this contribution, we propose additional frame configurations for legacy support mode. The proposed frame configuration allows a different mix ratio of 16e and 16m between adjacent ABSs while keeping the same frame offset, so that loading of 16e and 16m terminals can be balanced in each cell and the preamble and SFH timing is kept aligned among ABSs as well.

- Additional frame configurations for legacy support

The current frame configurations defined in P802.16m/D2 comprise one legacy zone and one 16m zone in DL, and the same frame offset shall be applied to all BSs in the same deployment region, i.e. the same division between legacy zone and 16m zone shall be applied to the all ABSs.

Figure 1. Frame configurations for 16e and 16m mix mode

In order to allow different divisions of 16e and 16m among adjacent ABSs while keeping the same timing of preamble and SFH, we may introduce another legacy zone following the 16m zone. Figure 1 depicts a frame configuration having such two legacy zones and one 16m zone. We can see in this figure that by operating two separate legacy zones, a different mix ratio of 16e and 16m between adjacent ABSs is allowed while fixing the frame offset.

- Proposed Text Changes in P802.16m/D2

[Remedy 1: Insert the following text, into line 60 on the page 277, in 15.3.3.5.1]

The subset of DL AAI subframes dedicated to the WirelessMAN-OFDMA operation should either one group of contiguous DL AAI subframes or two separate groups of contiguous DL AAI subframes. When comprising the two separate groups, the second group shall include the last DL AAI subframe.

[Remedy 2: Change the contents of Table 765, 766, 767, on the page 285 ~ 287, in 15.3.3.8, as follows:]

Table 765 – Frame Configuration and Indexing (5/10/20MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA Support - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix - offset | - UL MUX - DL length | - UL Mix - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 11 | - 5/10/20 | - 1/8 | - 5 | - TDD | - 5:3 | - 3:2 (3) - 3 | - FDM - 2 | - 21:14 - 3 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 12 | - 5/10/20 | - 1/8 | - 6 | - TDD | - 5:3 | - 3:2 (3) - 3 | - TDM - 2 | - 2:1 - 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 13 | - 5/10/20 | - 1/8 | - 7 | - TDD | - 5:3 | - 2:3 (2) - 2 | - FDM - 3 | - 14:21 - 3 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 14 | - 5/10/20 | - 1/8 | - 8 | - TDD | - 5:3 | - 2:3 (2) - 2 | - TDM - 3 | - 1:2 - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 15 | - 5/10/20 | - 1/8 | - 9 | - TDD | - 5:3 | - 2 | - 2 | - 3 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 / 60 |
| - 16 | - 5/10/20 | - 1/8 | - 10 | - TDD | - 5:3 | - 2 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 105.714 / 60 |
| - 1517 | - 5/10/20 | - 1/8 | - 911 | - TDD | - 5:3 | - 1:4 (1) - 1 | - FDM - 4 | - 8:27 - 3 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 1618 | - 5/10/20 | - 1/8 | - 1012 | - TDD | - 5:3 | - 1:4 (1) - 1 | - TDM - 4 | - 1:2 - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 19 | - 5/10/20 | - 1/8 | - 13 | - TDD | - 5:3 | - 1 | - 3 | - 3 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 / 60 |
| - 20 | - 5/10/20 | - 1/8 | - 14 | - TDD | - 5:3 | - 1 | - 3 | - 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 / 60 |
| - 21 | - 5/10/20 | - 1/8 | - 15 | - TDD | - 5:3 | - 1 | - 2 | - 3 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 / 60 |
| - 22 | - 5/10/20 | - 1/8 | - 16 | - TDD | - 5:3 | - 1 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 105.714 / 60 |
| - 1723 | - 5/10/20 | - 1/8 | - 1117 | - TDD | - 6:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 1824 | - 5/10/20 | - 1/8 | - 1218 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 1925 | - 5/10/20 | - 1/8 | - 1319 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 311.428 / 60 |
| - 2026 | - 5/10/20 | - 1/4 | - 0 | - TDD | - 5:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 2127 | - 5/10/20 | - 1/4 | - 1 | - TDD | - 4:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 2228 | - 5/10/20 | - 1/4 | - 2 | - TDD | - 3:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 2329 | - 5/10/20 | - 1/4 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  | - N/A |

Table 766 – Frame Configuration and Indexing (8.75MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA Support - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix - offset | - UL MUX - DL length | - UL Mix - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 9 | - 8.75 | - 1/8 | - 4 | - TDD | - 5:2 | - 3:2 (3) - 3 | - FDM - 2 | - 21:14 - 2 | - DL - Type1 | - DL - Type1 | - UL - Type1 | - UL - Type4 | - Not Used | - Not Used | - Not Used |  | - 82.2/ - 74.4 |
| - 10 | - 8.75 | - 1/8 | - 5 | - TDD | - 5:2 | - 3:2 (3) - 3 | - TDM - 2 | - 1:1 - 1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - Not Used | - Not Used | - Not Used |  | - 82.2/ - 74.4 |
| - 11 | - 8.75 | - 1/8 | - 6 | - TDD | - 5:2 | - 2:3 (2) - 2 | - FDM - 3 | - 14:21 - 2 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - UL - Type1 | - UL - Type4 | - Not Used | - Not Used |  | - 82.2/ - 74.4 |
| - 12 | - 8.75 | - 1/8 | - 7 | - TDD | - 5:2 | - 2:3 (2) - 2 | - TDM - 3 | - 1:1 - 1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - Not Used | - Not Used |  | - 82.2/ - 74.4 |
| - 13 | - 8.75 | - 1/8 | - 8 | - TDD | - 5:3 | - 2 | - 2 | - 2 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type4 | - Not used | - Not used |  | - 82.2/ - 74.4 |
| - 14 | - 8.75 | - 1/8 | - 9 | - TDD | - 5:3 | - 2 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 82.2/ - 74.4 |
| - 1315 | - 8.75 | - 1/8 | - 810 | - TDD | - 5:2 | - 1:4 (1) - 1 | - FDM - 4 | - 8:27 - 2 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - UL - Type1 | - UL - Type4 | - Not Used |  | - 82.2/ - 74.4 |
| - 1416 | - 8.75 | - 1/8 | - 911 | - TDD | - 5:2 | - 1:4 (1) - 1 | - TDM - 4 | - 1:1 - 1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - Not Used |  | - 82.2/ - 74.4 |
| - 17 | - 8.75 | - 1/8 | - 12 | - TDD | - 5:2 | - 1 | - 3 | - 2 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - UL - Type4 | - Not Used |  | - 82.2/ - 74.4 |
| - 18 | - 8.75 | - 1/8 | - 13 | - TDD | - 5:2 | - 1 | - 3 | - 1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - Not Used | - UL - Type1 | - Not Used |  | - 82.2/ - 74.4 |
| - 19 | - 8.75 | - 1/8 | - 14 | - TDD | - 5:2 | - 1 | - 2 | - 2 | - DL - Type1 | - DL - Type1 | - Not Used | - Not Used | - UL - Type1 | - UL - Type4 | - Not Used |  | - 82.2/ - 74.4 |
| - 20 | - 8.75 | - 1/8 | - 15 | - TDD | - 5:2 | - 1 | - 2 | - 1 | - DL - Type1 | - DL - Type1 | - Not Used | - Not Used | - Not Used | - UL - Type1 | - Not Used |  | - 82.2/ - 74.4 |
| - 1521 | - 8.75 | - 1/8 | - 0 | - TDD | - 4:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 1622 | - 8.75 | - 1/8 | - 0 | - TDD | - 3:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 1723 | - 8.75 | - 1/8 | - 0 | - TDD | - 2:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 1824 | - 8.75 | - 1/8 | - 0 | - FDD | - F/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 |  |  | - N/A |

Table 767 – Frame Configuration and Indexing (7MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA Support - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix - offset | - UL MUX - DL length | - UL Mix - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 8 | - 7 | - 1/8 | - 4 | - TDD | - 3:2 | - 1:2 (2) - 2 | - FDM - 2 | - 12:23 - 2 | - DL - Type1 | - DL - Type1 | - UL - Type1 | - UL - Type1 | - Not Used |  |  |  | - 188/ - 60 |
| - 9 | - 7 | - 1/8 | - 5 | - TDD | - 3:2 | - 1:2 (2) - 2 | - TDM - 2 | - 1:1 - 1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - Not Used |  |  |  | - 188/ - 60 |

[Remedy 3: Change the text, line 30~41, on the page 285, in 15.3.3.8, as follows:]

For frame configurations to support the WirelessMAN-OFDMA frame in the three tables, DL Mix of X:Y (Z) denotes a ratio of DL AAI subframes dedicated to WirelessMAN-OFDMA and Advanced Air Interface operations, i.e. X : Y = WirelessMAN-OFDMA : Advanced Air Interface. The value Z in parentheses indicates FRAME_OFFSET. Similarly, UL Mix of X:Y denotes a ratio of UL AAI subframes dedicated to WirelessMAN-OFDMA and Advanced Air Interface operations for the UL TDM mode defined in 15.3.3.6.1, and a ratio of UL subchannels dedicated to WirelessMAN-OFDMA and Advanced Air Interface operations for the UL FDM mode defined in 15.3.3.6.1.

In Table 765, for 5 and 20 MHz bandwidths, WirelessMAN-OFDMA support is not required..

The frame structure supporting the WirelessMAN-OFDMA frame defined in 15.3.3.5, shall be configured with one of  configuration number (i.e. ‘No.’ in the table) from 11 to 22 in the table 765 for 5/10MHz channel bandwidth, 9 to 20 in the table 766 for 8.75MHz channel bandwidth, 8 to 9 in the table 767 for 7MHz channel bandwidth. In this case, DL offset and DL length denote Frame_Offset and the number of DL AAI subframes dedicated to Advanced Air Interface operations, respectively. UL length denotes the number of UL AAI subframes dedicated to Advanced Air Interface operations.

When supporting WirelessMAN-OFDMA, for the case when UL length is less than total number of UL AAI subframes in a frame, the UL TDM mode defined in 15.3.3.5.1 is applied. For the case when UL length is the same as the total number of UL AAI subframes in a frame, the UL FDM mode defined in 15.3.3.6.1 is applied. For the UL FDM mode, the default number of UL subchannels dedicated to WirelessMAN-OFDMA is X = max(8, ceil(35 x the number of DL AAI subframes dedicated to WirelessMAN-OFDMA / (total number of DL AAI subframes in a frame))), and the number of UL subchannels dedicated to Advanced Air Interface operations is Y = 35 – X.