| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Cleanup Text for AAI_FFR-CMD (16.2.3.19) for the IEEE 802.16m/D3 |  |
| Date Submitted | 2010-01-05 |  |
| Source(s) | Yuan Zhu, Qiuping Huang, Xiaogang Chen, Yi Hsuan, Hujun Yin Intel Corporation Jeongho Park Samsung Electronics | yuan.y.zhu@intel.com jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D3 16.2.3.19 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Cleanup Text for AAI_FFR-CMD (16.2.3.19) for the IEEE 802.16m/D3

Yuan Zhu, Qiuping Huang, Xiaogang Chen, Yi Hsuan, Hujun Yin

Intel Corporation

Jeongho Park

Samsung Electronics

- Introduction

This contribution proposes some cleanup text for AAI_FFR-CMD Message section 16.2.3.19 of P802.16m/D3 [1].

In [1], two FFR measurement related MAC management messages are defined and they are AAI_FFR-CMD and AAI_FFR-REP. ABS can use AAI_FFR-CMD message to ask AMS to feedback SINR and/or interference measurement for one or more frequency partitions. AMS will use AAI_FFR-REP message to feedback SINR and/or interference measurement for one or more frequency partitions.

Currently there is one bit field “Full Report Flag” defined for both AAI_FFR-CMD and AAI_FFR-REP. The existence and contents of the other fields will depend on the value of this field. When “Full Report Flag”=1, AMS needs to feedback measurements for all frequency partitions. When “Full Report Flag”=0, AMS needs to feedback measurements according to “Frequency Partition Bitmap”. Thus the case “Full Report Flag”=1 is the same as “Full Report Flag”=0 and all bits in “Frequency Partition Bitmap” equals to 1. Thus the first change is to remove the “Full Report Flag” field.

The second change is to add “Reporting Type” filed in AAI_FFR-CMD message to make it compatible with AAI_FFR-REP message.

- References

[1] Draft Amendment, P802.16m/D3, December 2009.

-------------------------------------------------- Text Change #1 starts------------------------------------------------

Add one row in section 16.2.3, Table 673, page 38

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| AAI_FFR-CMD | FFR measurement report command | Encrypted/ICV | Unicast |

-------------------------------------------------- Text Change #1 ends-------------------------------------------------

-------------------------------------------------- Text Change #2 starts------------------------------------------------

Make the following changes in section 16.2.3.19, page 79, line 1

16.2.3.19 AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message shall be transmitted by the ABS to instruct the AMS to perform measurement over specific frequency partition(s). The following are Table 696 lists the parameters shall to be included into the AAI_FFR-CMD message:

Table 696—AAI_FFR-CMD message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_FFR-CMD_Message_format(){ |  |  |
| Control Message Type = XX | 8 |  |
| Full Report Flag | 1 | 0: Report the measurement results for specified frequency partition 1: Report the measurement result for full frequency partition |
| If(Full Report Flag==0){ |  |  |
| Frequency Partition Bitmap | N_FPFPCT | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics N_FPFPCT: the number of Frequency Partitions defined in Table 777 to Table 781 Frequency Partition Bitmap shall contain at least one bit with value “1”. The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. |
| Reporting Type | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value ‘1’, it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value ‘1’. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |
| } |  |  |
| Frame Offset | 8 | The offset (in units of frames) from the current frame in which AAI_FFR-REP message shall be transmitted on the unsolicited UL resource. |
| Padding | Variable | if needed for alignment to byte boundary |
| } |  |  |

-------------------------------------------------- Text Change #2 ends-------------------------------------------------