| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the CDMA-based Ranging Procedure (D3 - 16.2.3.11) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim LG Electronics Kelvin Chou MediaTek Inc. | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes CDMA-based ranging procedure. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the CDMA-based Ranging Procedure (D3 - 16.2.3.11)

Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim

LG Electronics

Kelvin Chou

MediaTek Inc.

- Introduction

This contribution includes the proposed text for the ranging procedure in the IEEE 802.16m DRAFT Amendment [1]. Ranging is a collection of processes by which the AMS and ABS maintain the quality of the RF communication link between them. It consists of three procedures — initial ranging, periodic ranging and handover ranging. Initial ranging allows an AMS joining the network to acquire correct transmission parameters, such as time offset and power level, so that the AMS can communicate with the ABS. Periodic ranging allows the AMS to adjust transmission parameters so that the AMS can maintain UL communications with the ABS. Handover ranging is allows an AMS re-entering the network to acquire correct transmission parameters so that the AMS can communicate with the target ABS. If the CDMA_RNG_FLAG in AAI_HO-CMD is set to 1, the AMS shall perform the CDMA-based handover ranging during network re-entry. In order to meet handover interruption time requirement [2], the ABS needs to transmit AAI_RNG-ACK for the handover ranging as soon as possible. Therefore, we propose that ABS should send an AAI_RNG-ACK within HO_RNG_ACK_TX_Time if some handover ranging preamble codes were detected. However, it is not efficient to transmit an AAI_RNG-ACK for the initial / periodic ranging in the same way.

- Modifications and key-descriptions

- HO_RNG_ACK_TX_Time vs. MAX_RNG_ACK_TX_Time

  - These times are defined as global values.

  - Target ABS should send an AAI_RNG-ACK in response to the detected handover ranging preamble code(s) within HO_RNG_ACK_TX_Time.

  - Target ABS should send an AAI_RNG-ACK in response to the detected initial / periodic ranging preamble code(s) within MAX_RNG_ACK_TX_Time.

Figure 2 - Proposed ranging procedure

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] IEEE 802.16m-07/002r9, “IEEE 802.16m System Requirements”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D3, page 165, line 59, make the following changes to subclause 16.2.6.3.5.1]

16.2.6.3.5.1 CDMA-based HO Ranging Procedure

If CDMA-based HO ranging is not omitted and if an AMS does not have a dedicated ranging preamble code or a dedicated ranging opportunity at the target BS, the AMS shall transmit a random handover ranging preamble code at the earliest available ranging opportunity.

An AAI_RNG-ACK message shall be sent within HO_RNG_ACK_TX_Time from the frame where some HO ranging preamble code(s) is detected. If the ABS provides UL BW allocation before sending the response to the successfully received ranging preamble code, the ABS may omit the response in the AAI_RNG-ACK.

[On the IEEE P802.16m/D3, page 243, line 8, make the following changes to subclause 16.2.15.3.1]

16.2.15.3.1 Contention-based initial ranging and automatic adjustments

- The ABS should respond with an AAI_RNG-ACK message in a predefined, subsequent DL AAI subframe within MAX_RNG_ACK_TX_Time from the frame where some initial ranging preamble code(s) is detected. The AAI_RNG-ACK format is defined in 16.2.3.3. as define in 16.2.3.19 The AAI_RNG-ACK message provides responses to all the successfully received and decoded IR requests initial ranging preamble codes in all the initial ranging slots opportunities in a previous UL AAI subframe . If the ABS provides UL BW allocation before sending the response to the successfully received ranging preamble code, the ABS may omit the response in the AAI_RNG-ACK.

[On the IEEE P802.16m/D3, page 17, line 1, make the following changes to subclause 10.1]

10. Parameters and constants

10.1 Global values

[Add the global values as indicated:]

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| AMS | MAX_RNG_ACK_TX_Time | AAI_RNG-ACK reception timeout following the transmission of an initial/periodic ranging preamble code. | - | - | 20ms |
| AMS | HO_RNG_ACK_TX_Time | AAI_RNG-ACK reception timeout following the transmission of a handover ranging preamble code. | - | - | 10ms |

-----------------------------------------------------------End of the Text---------------------------------------------------------