| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI_MC-ADV and AAI_NBR-ADV Message Formats (16.2.8) |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng MediaTek Inc. Hyunjeong Kang Samsung Electronics | IK.Fu@mediatek.com hyunjeong.kang@samsung.com |
| Re: | P802.16m/D3 |  |
| Abstract | The AAI_MC-ADV and AAI_NBR-ADV message contents are still incomplete. This contribution proposes some modification for clarification. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI_MC-ADV and AAI_NBR-ADV Message Formats

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following text Page #117, Line #46 in P802.16m/D3]

16.2.3.49 AAI_Global-Config (global carrier configuration) Message

The AAI_Global-Config message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global-Config message to an AMS right after network entry completes.

Table xxx - AAI_Global-Config MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |
| Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 770 |
| Start Frequency Assignment Index | 6 | Frequency Assignment Index of the first carrier in carrier group #i |
| For(j=0;j<Number of Carriers; j++) { |  | “Number of Carriers” is obtained from Table 770 with corresponding carrier configuration across network |
| Physical Carrier Index | 6 | Index of the physical carrier |
| Carrier Type | 1 | “0” for fully configured “1” for partially configured |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| } |  |  |
| } |  |  |
| Physical Carrier Index of current carrier | 6 | The carrier that broadcasts this message; the physical carrier index refers to AAI_Global-Config message |

[Adopt the following text modification started from line#56, page#185 in P802.16m/D3]

16.2.8.2.3.1.1 AAI_MC-ADV (multicarrier Advertisement) Message

The MC ABS shall periodically broadcast AAI_MC-ADV message for the reception by all AMSs.

Table 735 AAI_MC-ADV MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Multi-carrier configuration change count | 4 | A change count associated with a particular configuration for all possible carriers in the network. Incremented by 1 upon each update. |
| MC configuration broadcast flag | 1 | 0: only broadcast serving BS specific info 1: broadcast multi-carrier definition fro all available carriers in the network. |
| If (MC configuration broadcast flag == 1) { |  |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| Uniformity Flag | 1 | 0: All Carriers in one group have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power 1: otherwise |
| For (i=1; i<= Number of Carrier Groups; i++){ |  |  |
| Carrier Configuration Index | 6 | Index associated to Table 648 |
| Start Frequency Assignment Index | 6 | Frequency Assignment Index of the first carrier in carrier group #i |
| For (j= i; j <= Number of Carriers ’ j++) { |  |  |
| Physical Carrier Index | 3 |  |
| If (Uniformity Flag == 1) { |  |  |
| Carrier Type | 1 | “0” for fully configured “1” for partially configured |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Transmit Power | 6 | A-Preamble transmit power level |
| } |  |  |
| Physical Carrier Index of Current Carrier | 6 | The carrier that ABS broadcast this message |
| } |  |  |
| } |  |  |
| } |  |  |
| Serving BS Carrier Number | 36 | //start broadcasting serving BS specific info |
| Serving BS Uniformity Flag | 1 | 0: All Carriers supported by serving ABS have the same Protocol Version, SFH_Info 1: otherwise |
| Physical Carrier Index of current carrier | 6 | The carrier that ABS broadcast this message; the physical carrier index refers to AAI_Global-Config message |
| MAC Protocol version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| If (WirelessMAN-OFDMA support ==1) { |  | “WirelessMAN-OFDMA support” is obtained from SFH |
| BSID | 48 |  |
| FA Index | 8 |  |
| Preamble Index/Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY specific preamble. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index of the neighbor BS. |
| } |  |  |
| For (i=0 1; i<=Serving BS Carrier Number-1; i++){ |  |  |
| Physical Carrier Index | 6 | //physical carrier index in AAI_Global-Config message |
| SA-Preamble Index | 10 |  |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| If (WirelessMAN-OFDMA support==1) For (i=0; i<P; i++) |  | “WirelessMAN-OFDMA support” is obtained from SFH |
| { |  |  |
| BSID | 48 |  |
| FA Index | 8 |  |
| Preamble Index/Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY specific preamble. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index of the neighbor BS. |
| } |  |  |
| If (Serving BS Uniformity Flag==1){ |  |  |
| MAC Protocol version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| SFH_encoding_format | 1 | 0b0: full Subpkt information 0b1: delta encoding, w.r.t. the current carrier |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. |
| If( SFH_encoding_format =00 |  |  |
| { |  |  |
| If( Control_bitmap[0] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt 1 | 88 |  |
| } |  |  |
| If( Control_bitmap[1] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt2 | 88 |  |
| } |  |  |
| If( Control_bitmap[2] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt3 | 88 |  |
| } |  |  |
| } |  |  |
| If( SFH_encoding_format =01) |  |  |
| { |  |  |
| Delta information | variables | Delta encoding, w.r.t. the current carrier |
| } |  |  |
| } | — | — |
| } |  |  |

[Adopt the following text modification started from line#24, page#63 in P802.16m/D3]

16.2.3.12 AAI_NBR-ADV

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

1)ABS type (macro, micro, macro hotzone, Femto, relay, legacy TBD)

a)physical carrier index referring AAI_Global-Config message which provides carrier frequency, BW, CP info, TDD/FDD and related definitions (expected to be the same given carrier frequency)

a)MAC version

a)TDD/FDD and related definitions (expected to be the same given carrier frequency)

a)BW, CP info: may not be required if carried in P/S-SCH channel

a)Multicarrier capability and configuration

ABS determines and indicates the system configuration information included for each deployment type and their corresponding broadcast frequency.

To allow AAI_NBR-ADV fragmentation while providing flexibility for AMS HO operation without requiring acquisition of the whole AAI_NBR-ADV message, ABS always provides the total number of deployment types and total number of recommended T-ABS for each type. Each AAI_NBR-ADV fragment has corresponding indexes for each deployment type and each neighbor ABS. ABSs with identical type are listed in the AAI_NBR-ADV message in descending order of their cell coverage.

Each AAI_NBR-ADV message carries

- AAI_NBR-ADV change count

- number of total cell types

- segment information for this AAI_NBR-ADV message

- system information of ABSs from one or more cell types, which is of variable size.

For each cell type, shared configuration parameters are carried as follows

- IDcellA-Preamble Index range

- FA range

The IDcell A-Preamble Index range and FA ranges are optional fields, which can be used when the serving ABS chooses not to broadcast configuration information for each individual ABS within the cell type.

For each fragment of the AAI_NBR-ADV, the fragment shall include the following parameters - Starting BS Index (i.e., this is the index offset from the last BS of the previous fragment. If this is first fragment, the Starting Index will be 0. Hence, each fragment has one Index which corresponds to the first BS in that fragment.)

Within each cell type, if serving ABS chooses to broadcast configuration information for each individual ABS instead of specifying IDcell A-Preamble Index range and FA range, a list of ABSs are provided and the following parameters are carried for each ABS

•48bit BS-ID

•ABS IDcell A-Preamble index

•indication whether full system information or partial information is carried for this ABS, which includes

—SFH information

—ESI information

—Index to MC configuration,

—Physical carrier index (6 bits, refer to the "physical carrier indexMulti-carrier configuration index" defined in AAI_Global-Config AAI_MC-ADV)

—Number of MAC protocol versions supported (4 bits)

—list of supported MAC protocol versions (4 bits per entry)

where for ABS of macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type); and for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in AAI_NBR-ADV.

•SFH_encoding_format: Based on the present system information, the list of ABSs shall be categorized into three groups. The categorization is indicated by SFH_encoding_format parameter

  - 0b00: full Subpacket information Reserved

  - 0b01: delta encoding for which the first BS in this loop is used as reference BS full Subpacket information

  - 0b10: no SFH included delta encoding with TBD Format, for which the first BS in this loop is used as reference BS

  - 0b11: Reserved no SFH included

Table 684 - AAI_NBR-ADV message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell types | 3 |  |
| Cell type in this message | 3 | Cell type in this message 0b000 macro 0b001 micro 0b010 macro hotzone 0b011 femto 0b100 relay 0b101 legacy 0b110-0b111 reserved |
| Total Number of AAI_NBR-ADV Segments | TBD 4 | Total nNumber of segments of AAI_NBR-ADV for this cell type |
| AAI_NBR-ADV Segment Index | TBD 4 | Indicates Current sSegment index of this message in the specific cell type |
| BS number M | TBD 8 | Total number of BSs to be included in this AAI_NBR-ADV segment |
| Multi-carrier configuration change count | 4 | Refer to the change count in AAI_MC-ADV |
| If (Cell type == 0b101) { |  |  |
| For (i=0; i<M; i++) { | — | — |
| BSID | 48 |  |
| FA Index | 8 |  |
| Preamble Index/Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY specific preamble. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index of the neighbor BS. |
| DCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current DCD configuration change count. |
| UCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current UCD configuration change count. |
| } |  |  |
| } else { | — | — |
| for (i=0; i<M; i++) |  |  |
| { |  |  |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 |  |
| Number of carriers (NC) | 3 | Number of carriers of the BS |
| for( j=0; j<NC; j++) { |  |  |
| SA-PREAMBLE index | TBD10 |  |
| A-PREAMBLE transmit power | 8 |  |
| Physical carrier index | 6 | Refer to the physical carrier index change count in AAI_Global-Config message AAI_MC-ADV |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| Number of MAC protocol versions supported: P Mixed Deployment | 41 | Greenfield: 10 Mixed-mode: >1 |
| If (Mixed Deployment==1) For (i=0; i<P; i++) { |  | //LZone information |
| BSID | 48 |  |
| FA Index | 8 |  |
| Preamble Index/Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY specific preamble. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index of the neighbor BS. |
| DCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current DCD configuration change count. |
| UCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current UCD configuration change count. |
| } |  |  |
| SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) 0b10: no SFH included 0b11: reserved For macrocell ABS, the bitmap shall be either 0b00 or 0b01 |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111. |
| If( SFH_encoding_format =00 |  | //encoding format type-1 |
| { |  |  |
| If( Control_bitmap[0] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt 1 | 88TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[1] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt2 | 88 TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[2] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt3 | 88 TBD | //exclude those fields already in cell type info |
| } |  |  |
| } |  |  |
| If( SFH_encoding_format =01) |  |  |
| { |  |  |
| Delta information | variables | Delta encoding, w.r.t. the reference BS |
| } |  |  |
| } | — | — |
| } | — | — |

--------------------------------------------------------End of the Text-----------------------------------------------------------