| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Multi-Carrier Handover (16.2.8.2.9) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | Clarification on multi-carrier handover |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Multi-Carrier Handover (16.2.8.2.9)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

Introduction

Proposed Text

In the P802.16m/D3, page 191, line 49, modify the following text:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.8.2.9.2 Multicarrier handover (MCHO) procedure

The multicarrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers, which includes multi-carrier EBB HO <<15.2.9.2.9.2.2.1>> and HO with secondary carrier pre-assignment <<15.2.9.2.9.2.2.2>> as described in this section. For a single-carrier AMS handover from one carrier to another carrier of the same ABS, the handover procedure follows the primary carrier change procedure per 16.2.8.2.11.

16.2.8.2.9.2.1 MCHO preparation

An AMS in multicarrier operation follows the handover operations defined in 15.2.6.216.2.6.3. MAC control messages in relation with handover preparation and initiation between the AMS and the serving ABS are transmitted over the primary carrier of the AMS.

During HO preparation, the AMS may request or be requested by the serving ABS to perform MCHO procedure through AAI_HO-REQ/AAI_HO-CMD messages. The serving ABS informs the AMS the carrier information (e.g. target primary carrier index) of the target ABSs through AAI_HO-CMD message. The serving ABS may communicate with the target ABS(s) to help the AMS obtain the assigned secondary carriers before handover execution. The serving ABS will forward the information received from AAI_MC-REQ message to the target ABS(s) for secondary carrier pre-assignment. The serving ABS will reply the secondary carrier pre-assignment results to the AMS if Carrier_Preassignment_Indication is set to 1 in the AAI_HO_CMD message.

16.2.8.2.9.2.2 MCHO Multi-carrier EBB HO execution and network reentry

Multicarrier EBB HO reentry procedure

The AMS with multicarrier capability follows the network reentry procedure per section <<15.2.6.2.5>>16.2.6.3.5. When HO_Reentry_Mode is set to 1 and HO_Reentry_Interleaving_Interval is set to 0, the AMS performs network reentry to the target ABS on one carrier and maintains normal communication with the serving ABS on another carrier not performing network reentry procedure. The AMS may use the original primary carrier for network reentry to the target ABS, as illustrated in Figure 419. It may also use another carrier different from its original primary carrier for network reentry procedures, as illustrated in Figure 420. In this case, Disconnect_time should be long enough that network reentry procedure to target ABS can be completed prior to the expiration of Disconnect_time. In case of AAI_HO-CMD message with multiple target ABS and carriers, the physical index of each candidate carrier provided by each target ABS should also be indicated in the AAI_HO-CMD message The AMS may inform the serving ABS through AAI_HO-IND the carrier to be used for network reentry operations to avoid resource allocation by the serving ABS on that carrier.

{No change to Figure 419 and 429}

From AMS point of view, if network entry is completed (see 15.2.6), the AMS shall stop communicating with the serving ABS. Then, the AMS may send UL data or BW-REQ message to the target ABS.

16.2.8.2.9.2.2  HO with secondary carrier pre-assignment

The serving ABS may negotiate with the target ABS for secondary carrier pre-assignment as illustrated in Figure 421. When Carrier_Preassignment_Index is set to 1 in the AAI_HO-CMD message, the pre-assignment information is forwarded from the Target ABS(s) via backbone to the Serving ABS, and then sent to the AMS by the serving ABS through AAI_HO-CMD message, and part of the pre-assigned secondary carriers (indicated by the Carrier Status Bitmap) may be activated right after network reentry. The target ABS may start transmitting data on such activated secondary carrier(s) if the AMS sends AAI_CM-IND message to the target ABS after network reentry. Before secondary carrier pre-assignment, the serving ABS shall forward the multicarrier capability of the AMS to the target ABS(s) via backbone. If the AMS is not in multi-carrier aggregation mode at the serving ABS, the AMS enters multi-carrier aggregation mode after handover network reentry to the target ABS is done.

{No change to Figure 421}

When Carrier_Preassignment_Index is set to 0 in the AAI_HO-CMD message, the AMS follows the operation of secondary carrier assignment (see 15.2.9.2.3.216.2.8.2.3.2) and carrier activation (see 15.2.9.2.11.1 16.2.8.2.11.1) after the AMS completes its network reentry with the target ABS.

------------------------------------------------------  Text End  ---------------------------------------------------------------