| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed structure of AAI_MC-REQ/RSP MAC Management Messages (16.2.3) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Youngsoo Yuk, Eunjong Lee, Inuk Jung, Kiseon Ryu and Yongho Ronny Kim LG Electronics | youngsoo.yuk@lge.com, mehappy3@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b |  |
| Abstract | The contribution proposes the text to clean-up MAC management messages of secondary carrier assignment operation in Multi-carrier. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed structure of AAI_MC-REQ/RSP MAC Management Messages (16.2.3)

Youngsoo Yuk, Eunjong Lee, Inuk Jung, Kiseon Ryu and Yongho Ronny Kim

LG Electronics

- Introduction

In P802.16m/D3, after finishing initial entry, an AMS and an ABS may perform multicarrier capability negotiation by exchanging AAI_MC-REQ/RSP MAC management messages.

We proposed some more parameters to be included in those messages.

Especially, we proposed a parameter of “TX spectral Mask shape” in DG discussion during session #62.

Figure 1 shows the spectral mask defined in IEEE 802.16-2009 system. The value for various BW is shown in Table 1.

The spectral mask of the aggregated bandwidth should be different from that of the non-aggregated bandwidth. The Figure 2 shows the difference between two cases. We can define a parameter, “TX spectral Mask shape” as the ration between the effective BW and overall BW. In the legacy system, it was 0.95 for all bandwidths. However, as you can see in figure 2, to receive all PRUs in 20MHz which is composed of two 10MHz carriers, it should be 0.975. With the transmitter achieving legacy spectral requirement, 4 PRUs (2 PRUs in each edge) cannot be used. It is related to the number of taps of the transmitter filter in an AMS. Thus, the ABS should consider this capability of an AMS when it decides to grant any PRU in the edge area to the AMS.

Thus the value should be transmitted by AMS’s as a multicarrier capability during multicarrier capability negotiation. Table 2 shows the parameter we propose.

Figure 1 Transmit Spectral Mask

Figure 2 Spectral Mask for Aggregated multicarrier

Table 1. Spectral Mask parameters

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| 10MHz | 4.75 | 5.45 | 9.75 | 14.75 |
| 20MHz | 9.5 | 10.9 | 19.5 | 29.5 |
| Aggregated 20MHz | 9.75 | 10.45 | 14.75 | 19.75 |

Table 2. TX Spectral Mask Shape

| TX spectral Mask shape | 1: 0.95, 2: 0.975, 3: 0.9875, 4: 0.99375 | Ratio between effective BW and overall BW |
| --- | --- | --- |

------------------------------------------------------Proposed Text --------------------------------------------------------

[On the IEEE P802.16m/D2, line 39, page 150, modify the text as follows]

16.2.3.47 AAI_MC-REQ (multicarrier Request) Message

The multicarrier Request Message (AAI_MC-REQ) is sent by an AMS to an ABS to request the list of Assigned Carriers. This message shall include all information about the AMS's supported multicarrier configurations that is needed by the ABS to assign carriers to the AMS and activate the carriers when needed. The AMS cannot send the AAI_MC-REQ message until it receives the AAI_MC-ADV message from its serving ABS. The following are the parameters shall be included in the AAI_MC-REQ message:

- AMS multi-carrier capability parameters for carrier assignment

- List of candidate of assigned carriers

- Support of data transmission over guard sub-carrier

- Table zzz - AAI_MC-RSP MAC control message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| AAI_MC-REQ Message Format (){ |  |  |
| Management Message Type = xx | 8 | AAI_MC-REQ Message |
| Number of RF Transceiver (NRF) | [2] | Number of RF Transceiver |
| Maximum number of Simultaneous RF carriers | [TBD] | Maximum number of RF carriers can be received/transmitted by the AMS 0b000: 1, 0b001: 2, 0b010: 3, 0b011: 4, … |
| For (n=0 ; n< NRF ; n++){ |  |  |
| Supportable RF band | [TBD] | Bitmap for each Bandclass (for each RF transceiver) |
| Maximum bandwidth | 4 | Maximum BW for each RF transceiver (MHz) Unit of 5 MHz |
| TX spectral Mask shape | 2 | Ratio between effective BW and overall BW 0b00: 0.95, 0b01: 0.975, 0b10: 0.9875, 0b11: 0.99375 |
| Support of data transmission over guard sub-carrier | 1 | Whether this AMS has a capability to utilize data burst in guard subcarrier region 0b0 : AMS has no capability on guard subcarrier transmission 0b1 : AMS has a capability on guard subcarrier transmission |
| } |  |  |
| } |  |  |

16.2.3.48 AAI_MC-RSP (multicarrier Response) Message

Based on information provided by the AMS in the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message:

- List of assigned carrier

- Support of data transmission over guard sub-carrier

The AAI_MC-RSP (multicarrier Response) Message is typically sent to the AMS in response to the AAI_MC-REQ message, but it may also be sent by the ABS to an AMS to update the list of assigned carriers in unsolicited manner.

- Table zzz - AAI_MC-RSP MAC control message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| AAI_MC-RSP Message Format (){ |  |  |
| Management Message Type = xx | 8 | AAI_MC-RSP Message |
| Number of Assigned Carriers (N) | 4 | Number of Assigned Secondary Carriers. |
| For (i=0; i<N;i++) { |  |  |
| Physical carrier index | 6 | The index refers to a Physical carrier index in AAI_MC-ADV message The order of appearance is logical carrier index of the assigned carrier |
| Support of data transmission over guard sub-carrier | 2 | This flag indicates whether this ABS utilize data burst in guard subcarrier region. 0b00: not support in this carrier 0b01: support only in the lower edge 0b10: support only in the upper edge 0b11: support in both edges |
| } |  |  |
| } |  |  |

------------------------------------------------------ Proposed Text--------------------------------------------------------