| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS identification during coverage loss recovery procedure (16.2.26) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Youngkyo Baek Hyunjeong Kang Yeongmoon Son Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.26” |  |
| Abstract | This contribution proposes MS identification scheme during coverage loss recovery procedure to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS identification during coverage loss recovery procedure (16.2.26)

Youngkyo Baek, Hyunjeong Kang,Yeongmoon Son

Samsung Electronics

# 1. Introduction

According to 16m/D3[1], if an AMS shares valid security context when the AMS performs Network reentry from the coverage loss, then the AMS shall include its previous MSID* in AAI_RNG-REQ. However, in case that the ABS discards the AMS’s context, the ABS cannot retrieve the AMS’s security context (e.g. AK context) because MSID*’s uniqueness cannot be guaranteed (i.e. MSID* can not be an identifier) and the ABS does not know the information of the authenticator which retains the AMS’s context. Even though the authenticator retains MS MAC address as an MS identifier, and the AMS transmits the MS MAC address and the authenticator ID to retrieve AMS’s context, the MS MAC address cannot be transmitted in the AAI_RNG-REQ message due to MS privacy

Hence we suggest a new MS identifier (says Loss-tolerant ID) which the authenticator manages. The suggested identifier can be used as an MS identifier in DCR mode.

# 2. Solution

When the AMS perform NW entry, through AAI_REG-RSP message an anchor authenticator ID and a loss-tolerant ID which the anchor authenticator assigns to AMS, are transmitted.

Once coverage loss occurs and the AMS performs the NW reentry, AAI_RNG-REQ message contains the authenticator ID and the loss-tolerant ID for MS identification. Here, the loss-tolerant ID is exposed and the anchor authenticator shall update the loss-tolerant ID for MS privacy. A new loss-tolerant ID is transferred using a disguise ID which is defined by current loss-tolerant ID XOR new loss-tolerant ID. The reason why we use a disguise ID is described in figure 1.

In case that anchor authenticator is changed during RNG message transactions, AAI_RNG-RSP message shall contain a new anchor authenticator ID and a disguise ID which is defined by current loss-tolerant ID XOR new loss-tolerant ID.

In case that anchor authenticator is changed and a key agreement procedure happens, anchor authenticator ID and loss-tolerant ID shall be changed. And so the key agreement MSG#3 message shall contain a new anchor authenticator ID and a disguise ID which is defined by current loss-tolerant ID XOR new loss-tolerant ID (see figure 2).

The reason we use a disguise ID is because the key agreement messages are not encrypted and so we need some mechanism for transferring a loss-tolerant ID securely. Since the current loss-tolerant ID is not exposed during key agreement procedure, even if the disguise ID is disclosed, the new loss-tolerant ID can be transferred securely. To keep consistency, we use disguise ID for the case of update through AAI_RNG-RSP message.

Figure1. MS identification during NW entry and NW reentry from coverage loss.

Figure2. MS identification during key agreement procedure after authenticator relocation.

In addition, the assigned Loss-tolerant ID and the Authenticator ID are used to identify the AMS in DCR mode. When the AMS enters DCR mode, the authenticator retains the AMS’ context including the Loss-tolerant ID. The AMS includes the Loss-tolerant ID and Authenticator ID in the AAI_RNG-REQ message when the AMS exits from DCR mode and initiates network re-entry.

# 3. Text Proposal

# 3.1 Text Proposal #1

Inset the proposed subclause at page 20, line 17 as follows

======================== Start of Proposed Text =====================

16.2.1.2.4 Loss-tolerant Identifier (LTID)

The Authenticator shall assign a 48 bit LTID to each AMS during network entry. The LTID shall uniquely identify the AMS in the domain of the Authenticator. An AMS may be assigned a new LTID during network reentry or key agreement procedure.

============================== End of Proposed Text ===============

# 3.2 Text Proposal #2

Add the following parameters into the table 674(AAI_RNG-REQ) at page 46, line 32 as follows

======================== Start of Proposed Text =====================

| Name | Value | Usage |
| --- | --- | --- |
| Serving BSID | The BSID of the AMS’s previous serving ABS before incurring a coverage loss, or the BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry or a network reentry after a coverage loss |
| Authenticator ID | Anchor authenticator ID which maintains the AMS’s context. | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode |
| Loss-tolerant ID | The MS identifier which the AMS has been assigned for coverage loss or DCR mode and currently maintains. |  |

============================== End of Proposed Text ===============

# 3.3Text Proposal #3

Add the following parameters into the table 675(AAI_RNG-RSP) at page 48, line 32 as follows

======================== Start of Proposed Text =====================

| Name | Value | Usage |
| --- | --- | --- |
| Authenticator ID | Anchor authenticator ID which maintains the AMS’s context. | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform network reentry/location update/zone-switch with authenticator relocation |
| Disguise ID | ID used for transferring a new loss-tolerant ID securely, where disguise ID = current loss-tolerant ID  new loss-tolerant ID. AMS retrieves the new loss-tolerant ID by disguise ID current loss-tolerant ID. |  |

============================== End of Proposed Text ===============

# 3.4 Text Proposal #4

Modify the text at page 56, line 41 as follows

======================== Start of Proposed Text =====================

The following parameter is included in the AAI_REG-RSP message:

—STID: MS identifier which the ABS assigns to the AMS.It is used for AMS identification in place of the temporary STID which has been transferred by AAI-RNG-RSP message.

—Authenticator ID: ID of anchor authenticator which maintains the AMS’s context.

—LTID(loss-tolerant ID): MS identifier which the anchor authenticator assigns to the AMS.

============================== End of Proposed Text ===============

# 3.5 Text Proposal #5

Add the following parameters into the table 714 (key agreement MSG#3) at page 107, line 32 as follows

======================== Start of Proposed Text =====================

| attribute | contents |
| --- | --- |
| Authenticator ID | Anchor authenticator ID which maintains the AMS’s context. It shall be included when anchor authenticator is changed. |
| Disguise ID | ID used for transferring a new loss-tolerant ID securely, where disguise ID = current loss-tolerant ID  new loss-tolerant ID. AMS retrieves the new loss-tolerant ID by disguise ID  current loss-tolerant ID. It shall be included when anchor authenticator is changed. |

============================== End of Proposed Text ===============

# 3.6 Text Proposal #6

Modify sentences at page 245, line 25 as follows

======================== Start of Proposed Text =====================

The ABS shall allocate and transfer a STID to the AMS through encrypted AAI_REG-RSP message if control message encryption is supported. The temporary STID, which was allocated during initial ranging procedure, is discarded when the ABS recognizes that the AMS received the AAI_REG-RSP messages successfully. The AAI_REG-RSP message contains the Loss-tolerant ID and Authenticator ID, where the authenticator retains the AMS contexts including the Loss-tolerant ID and AMS MAC address.

============================== End of Proposed Text ===============

# 3.7 Text Proposal #7

Modify sentences at page 288, line 65 as follows

======================== Start of Proposed Text =====================

When receiving the UL BW allocation, the AMS shall send the AAI_RNG-REQ message, with the bit#7 = 1 in the Ranging Purpose Indication parameter (i.e., indicating a network reentry after a coverage loss). If the AMS shares valid security context, then the AMS shall include its Loss-tolerant ID and Authenticator ID previous MSID* in AAI-RNG-REQ protected with a CMAC derived from new AK.

============================== End of Proposed Text ===============

# 3.8 Text Proposal #8

Modify sentences at page 268, line 29 as follows

======================== Start of Proposed Text =====================

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network reentry. Deregistration identifier (DID) is assigned to AMS with paging cycle and paging offset value of 0.

Loss-tolerant ID DID together with Authenticator ID of the anchor authenticator ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

============================== End of Proposed Text ===============

# 3.9 Text Proposal #9

Modify sentences at page 268, line 49 as follows

======================== Start of Proposed Text =====================

In the AAI-DREG-CMD, DID shall be assigned which is used for network reentry from DCR mode to identify the AMS.

============================== End of Proposed Text ===============

# 3.10 Text Proposal #10

Modify sentences at page 268, line 65 as follows

======================== Start of Proposed Text =====================

At the time of DCR mode change, paging cycle, paging offset, paging controller values shall be updated to be 0 for the AMS and the Loss-tolerant ID and the Authenticator ID of the anchor authenticator ID of the network entity that assigns/retains the context shall be used to uniquely identify DCR mode AMS.

============================== End of Proposed Text ===============

# 3.11 Text Proposal #11

Modify sentences at page 269, line 13 as follows

======================== Start of Proposed Text =====================

For the network reentry from DCR mode, the AMS shall initiate network reentry with the ABS by sending an AAI_RNG-REQ message with ranging purpose code of 0x06 and the Loss-tolerant ID and the Authenticator ID of the anchor authenticator ID of the network entity that assigns/retains the context. Rest of reentry procedure shall be performed same as network reentry from idle mode described in section 16.2.17.5.

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.