| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Remedy for Wideband CQI Report with Long Period (16.3.9) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Kwanhee Roh Sangheon Kim David Mazzarese Heewon Kang Samsung Electronics Co. Ltd. | kwanhee.roh@samsung.com sangheon.kim@samsung.com d.mazzarese@samsung.com hkang@samsung.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | The contribution proposes the text for supporting wideband SFBC CQI with long period. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Suggested Remedy for Wideband CQI Report with Long Period (16.3.9)

Kwanhee Roh, Sangheon Kim, David Mazzarese, Heewon Kang

Samsung Electronics Co., Ltd.

# Introduction

In current IEEE 802.16m/D3, transmission power adaptation is supported for non-user specific AMAP IEs. To better utilize this functionality, wideband CQI for MIMO mode 0 (OL-SU SFBC) is necessary. In current specification, ABS needs to de-allocate previous FBCH and re-allocate MIMO feedback mode 0 for an AMS to get the required wideband CQI. Furthermore, even though AMS is allocated with MIMO feedback mode 0, AMS may report CQI for higher rank rather than SFBC CQI.

Thus, it’s necessary to mandate active AMSs to report wideband CQI with SFBC encoding periodically.

# Proposed text

[modify 16.2.3.29 as follows:]

-------------------------------------------------------Start of the Text #1-------------------------------------------------------

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change		INTEGER (0 ~ 15)

BS_Restart_Count				INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Wideband_SFBC_CQI_Report_Interval	INTEGER (0 ~ 15)

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

…

}

-------------------------------------------------------End of the Text #1-------------------------------------------------------

[add the following text at the end of 16.3.9.3.1.5, line 55 in page 559, as follows:]

-------------------------------------------------------Start of the Text #2-------------------------------------------------------

Rules for reporting wideband CQI

If the AMS successfully receives AAI-SCD and if the Wideband_SFBC_CQI_Report_Interval (= r) parameter in AAI-SCD is greater than 0, the AMS shall report the wideband CQI for SFBC encoding for frequency partition 0 whenever it is allocated a fast feedback channel (PFBCH or SFBCH) with any MIMO Feedback Mode. The AMS shall report the wideband CQI for SFBC encoding for frequency partition 0 using a PFBCH with encoding type 0 by puncturing the first or second short-term feedback opportunity right after every r-th long-term feedback opportunity. Whenever more than one short-term feedback opportunities are available between two adjacent long-term feedback opportunities, AMS shall puncture the second short-term feedback opportunity. If only one reporting period is allocated to the AMS, then the AMS shall puncture every r-th feedback opportunity, starting at the first r-th feedback opportunity after the MIMO Feedback Mode allocation.

-------------------------------------------------------End of the Text #2-------------------------------------------------------