| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Status Reporting Section for the IEEE 802.16m/D3 Amendment |  |
| Date Submitted | 2010-01-10 |  |
| Source(s) | Rongzhen Yang, Shi Liu, Hujun Yin, Yang-seok Choi Intel Corp. Tim Wilson Motorola | Email: Rongzhen.yang@intel.com Email : Tim.Wilson@motorola.com |
| Re: | Comments on P802.16m/D3 |  |
| Abstract | This contribution provides the proposed text for 16m uplink power status reporting. |  |
| Purpose | To be discussed and adopted into P802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Power Status Reporting Section for the IEEE 802.16m/D3 Amendment

Rongzhen Yang, Shi Liu, Hujun Yin, Yang-seok Choi
Intel Corp.

Tim Wilson

Motorola

- Introduction

Power status reporting form AMS to ABS provides very important reference information for ABS uplink resource scheduling, which includes:

- Uplink power status definition;

- Uplink power status transmission conditions;

- Uplink power status reporting message design;

All those parts are missed in current 802.16m D3 amendment, in this contribution, the major parts of uplink power status reporting text are proposed.

- References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Oct. 2009.

- Text proposal

Instruction to editor;

Black text: existing D3 text

Blue and strike out text: for deletion

Red and underline text: for addition

[Remedy 1:]

[Insert one new subsection 16.3.9.4.6 “Uplink Power Status Reporting” after line 57 on the page 564, as follows:]

============================== Start of Proposed Text =================================

16.3.9.4.6 Uplink Power Status Reporting

16.3.9.4.6.1 Power Status Reporting Information

There are two key parameters for power status reporting: the base uplink transmission PSD and the SIRDL defined in section 16.3.9.4.1.

The base uplink transmission PSD is derived from equation (269) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI				(273)

The parameter is reported in dBm and is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF).

The reported SIRDLvalue is coded using 10 bits in 0.05 dB steps ranging from -10dB (coded 0x000) to 41.15 dB (coded 0x3ff).

After the ABS has received PSD(base) and SIRDL from the AMS, the PSD for all uplink channels can be estimated in ABS.

16.3.9.4.6.2 Status Transmission Condition

The AMS shall send the status reporting message AAI_UL_PSR_Message (TBD) to the ABS when the conditions listed below are met:

M(nlast)−M(n)≥Tx_Power_Report_Threshold(dB) andn−nlast≥Tx_Power_Report_Minimum_Interval	(274)

or

n−nlast≥Tx_Power_Report_Periodical_Interval		(275)

Where,

M is defined in equation 276:

M=L+SINRTarget(Reported)		(276)

Here, L is the pathloss defined in equation (269)

nlast is the frame index when the last AMS status reporting message was sent,.

n is the current frame index.

Tx_Power_Report_Threshold(dB) is a 4 bit unsigned integer value in 0.5 dB steps, the specific value “0b1111” shall be interpreted as “infinite”;

Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Intervalare coded by 4 bit unsigned integer values d representing 2dframes, the specific value d=0b1111 shall be interpreted as “infinite”

The status reporting configuration parameters Tx_Power_Report_Threshold(dB), Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Interval are sent from ABS to AMS in the message AAI_UL_PSR_Config (TBD).

============================== End of Proposed Text =================================

[Remedy 2:]

[Insert the new text from line 44 on the page 117, under 16.2.3, as follows:]

============================== Start of Proposed Text =================================

16.2.3.xx Uplink Power Status Reporting Config (AAI_UL PSR_Config) message

An AAI_UL_PSR_Config message is used to config the AMS uplink power status reporting by unicasting. It is defined as:

-- ASN1START

AAI_UL_PSR_Config:: = CHOICE {

Disable_Report			NULL,

Enable_Report			SEQUENCE {

Tx_Power_Report_Threshold_dB		INTEGER (0..15)

Tx_Power_Report_Minimum_Interval	INTEGER (0..15)

Tx_Power_Report_Periodical_Interval 	INTEGER (0..15)

}

}

-- ASN1STOP

AAI_UL_PSR_Config Field Descriptions

Tx_Power_Report_Threshold_dB: Tx_Power_Report_Threshold_dB is a 4 bit unsigned integer value in 0.5 dB steps, the specific value “0b1111” shall be interpreted as “infinite”

Tx_Power_Report_Minimum_Interval: Tx_Power_Report_Minimum_Interval is coded by 4 bit unsigned integer values d representing 2dframes, the specific value d=0b1111 shall be interpreted as “infinite”

Tx_Power_Report_Periodical_Interval: Tx_Power_Report_Periodical_Interval is coded by 4 bit unsigned integer values d representing 2dframes, the specific value d=0b1111 shall be interpreted as “infinite”

16.2.3.xx Uplink Power Status Report (AAI_UL_PSR) message

An AAI_UL_PSR message is used by the AMS to report uplink power status to ABS. It is defined as:

-- ASN1START

AAI_UL_PSR:: = SEQUENCE {

Tx_Power_PSD_Base		INTEGER (0..255)

Tx_SIR_Download		INTEGER (0..1024)

}

-- ASN1STOP

AAI_UL_PSR Field Descriptions

Tx_Power_PSD_Base: Tx_Power_PSD_Base (PSD(base)) is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF).

Tx_SIR_Download: Tx_SIR_Download (SIRDL) is coded using 10 bits in 0.05 dB steps ranging from -10dB (coded 0x000) to 41.15 dB (coded 0x3ff)..

============================== End of Proposed Text =================================

[Remedy 3:]

[Insert the new text from line 53 on the page 40, in 16.2.3, as follows:]

============================== Start of Proposed Text ==============================

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| AAI_UL_PSR_Config | Uplink Power Status Reporting Configuration | Encrypted/ICV | Unicast |
| AAI_UL_PSR | Uplink Power Status Report | Encrypted/ICV | Unicast |

============================== End of Proposed Text ================================