| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial Changes to DL Basic Assignment A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Hyunkyu Yu, David Mazzarese Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot#30b” Target topic: “IEEE P802.16m/D3 section 16.3.6.5”. |  |
| Abstract | This contribution provides editorial changes to DL basic assignment A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial Changes to DL Basic Assignment A-MAP IE (Section 16.3.6.5)

Hyunkyu Yu, David Mazzarese

Samsung Electronics

- Introduction

This contribution provides editorial changes to DL basic assignment A-MAP IE in D3 [1].

- Proposed text with markups

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

- References

[1] IEEE P802.16m/D3, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2009-12-08

- Proposed Text Changes

[Remedy#1: Add the following text, at line 37 in page 395, section 16.3.6.5.2.4.1]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.2.4.1 DL basic assignment A-MAP IE

Table 812 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 812.

Table 812 - DL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL_Basic_Assignment_A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: CDR |
| if (MEF == 0b01) { |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission (Mt <= Nt ), Nt: Number of transmit antennas at the ABS. 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if (MEF == 0b10) { |  | Parameters for horizontal encoding |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream10 and other modulation =QPSK 0b0001: 2 streams with PSI=stream10 and other modulation =16QAM 0b0010: 2 streams with PSI=stream10 and other modulation =64QAM 0b0011: 2 streams with PSI=stream10 and other modulation information not available 0b0100: 2 streams with PSI=stream21 and other modulation =QPSK 0b0101: 2 streams with PSI=stream21 and other modulation =16QAM 0b0110: 2 streams with PSI=stream21 and other modulation =64QAM 0b0111: 2 streams with PSI=stream21 and other modulation information not available 0b1000: 3 streams with PSI=stream10 0b1001: 3 streams with PSI=stream21 0b1010: 3 streams with PSI=stream32 0b1011: 4 streams with PSI=stream10 0b1100: 4 stream with PSI=stream21 0b1101: 4 streams with PSI=stream32 0b1110: 4 streams with PSI=stream43 0b1111: n/a |
| } else { |  |  |
| Reserved | 4 | Reserved bits |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 2 | Reserved bits |
| } |  |  |

-------------------------------------------------End proposed text----------------------------------------------------------------