| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed format for the Extended Assignment A-MAP IE (16.3.6.5.2.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 on P802.16m/D4 |  |
| Abstract | The contribution proposes the format for the Extended Assignment A-MAP IE (16.3.6.5.2.4). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed format for the Extended Assignment A-MAP IE (16.3.6.5.2.4)

Lei Wang

InterDigital Communications, LLC

- Introduction

In the Table 818 on page 460 of the 802.16m/D4, the A-MAP IE type=0b1111 is reserved as the extended Assignment A-MAP IE. However, its format has not been defined. For a forward compatibility, it is important to define its format.

This contribution proposes a format for the extended Assignment A-MAP IE.

- Suggested changes in the 802.16m/D4

Based on the above discussion, we propose the following changes in the 802.16m/D4. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: on page 460, line 55

Insert the following text and Table:

The Extended Assignment A-MAP IE is defined in Table 818a.

Table 818a—Extended Assignment A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Extended_Assignment_A-MAP IE() { |  |  |
| A-MAP IE Type | 4 | 0b1111: Extended Assignment A-MAP IE |
| Extended Assignment A-MAP IE Type | 4 | Extended Assignment A-MAP IE Type |
| Length of Extended Assignment A-MAP IE Body | 4 | Length in bytes of the Extended A-MAP IE Body |
| Extended A-MAP IE Body | variable | Contents of the extended assignment A-MAP IE with the size in bytes indicated by the length field. |
| } |  |  |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”