| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9) |  |
| Date Submitted | 2010-03-09 |  |
| Source(s) | Jeongki Kim, Wookbong Lee, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, wookbong.lee@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot Recirc #31 |  |
| Abstract | This contribution proposes the texts related to GRA MIMO Mode. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9)

Jeongki Kim, Wookbong Lee, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

According to 16m D4 [1], if UL MIMO Mode set ID in group resource allocation is 0b10, Mode 2 (CL SU-MIMO with Mt=1, TNS=2) is used as the MIMO mode of the group. However, TNS=2 cannot be used for UL CL SU-MIMO and UL CL SU-MIMO is unsuitable for UL group resource allocation because the downlink  control overhead will be increased in indication of PMI or because the pre-coding by mid-amble can be used only for TDD (not FDD). So, we propose the texts to clarify the UL MIMO mode set ID = 0b10.

- Use OL MU-MIMO with Mt=1, TNS=2 instead of CL SU-MIMO

  - An AMS can know the resource allocation size of each AMS belonging to the AMS’s group by Resource allocation bitmap (RAB) in Group resource allocation A-MAP IE.

  - Allocation of group resource and stream index in Mode 3

    - An ABS allocates the first N (Indicated by RAB) LRUs from resource offset to the first scheduled AMS and the second scheduled AMS in a group and the stream index of each AMS is 1 and 2.

    - The stream index 1 and the stream index 2 are allocated to the scheduled AMSs in order of scheduling.

    - If the stream index of N-th AMS is 1, the stream index of (N+1)-th AMS is 2 and vice versa.

    - The resource of N-th AMS is allocated right after the resource of the (N-2) th AMS.

The Table 1 shows an example of group resource allocation when the UL MIMO Mode is Mode 3 and Resource offset is 2(LRU index).

Table 1

| Order of scheduled AMSs | Size of resource allocation | Allocated LRU indices | Stream Index |
| --- | --- | --- | --- |
| AMS1 | 4 | 2 ~5 | 1 |
| AMS2 | 3 | 2~4 | 2 |
| AMS3 | 3 | 6~8 | 1 |
| AMS4 | 4 | 5~8 | 2 |
| AMS5 | 1 | 9 | 1 |
| … | … | … | … |

Figure 1

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: Modify table 755 as follows:]

Table 755—MIMO Bitmap Information for UL

| MIMO Mode Set | Existence of Second Bitmap | Length of Bit per Scheduled AMS | MIMO Mode Indication |
| --- | --- | --- | --- |
| 0b00 | No | - | OL SU-MIMO (SFBC with non-adaptive precoder) |
| 0b01 | Yes | 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) with Mt=2 0b1: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 |
| 0b10 | No | - | CL SU-MIMO with Mt=1, TNS=12 |

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------