| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Interaction between HARQ and ARQ (16.2.13.2.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Chun-Ting Chou National Taiwan University Yih-Shen Chen, I-Kang Fu, Paul Cheng MediaTek Inc. | chuntingchou@cc.ee.ntu.edu.tw Yihshen.Chen@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | This contribution proposes method to improve the efficiency of ARQ protocol |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Interaction between HARQ and ARQ

Chun-Ting Chou
National Taiwan University

Yih-Shen Chen, I-Kang Fu, Paul Cheng

MediaTek Inc.

- Introduction

Both HARQ and ARQ are used to provide transmission reliability. The transmission results from HARQ module can provide some very useful information to make ARQ operation more efficiently. Actually, the concept of “local NACK” has been adopted in current specification [1]. In this contribution, we investigate some stalling problems and suggest to have a more close interaction between HARQ and ARQ modules.

- Problem Description

The existing ARQ mechanism could get stalled abnormally due to a single PHY (HARQ) transmission error. As a result, the throughput could be severely affected. The scenarios are given as follow.

Scenario 1: Assume that the ARQ window size is 5 and there are 5 HARQ channels. ARQ blocks #14, #15, #16, #17 and #18 are transmitted by the TX to the RX via 5 HARQ channels. Since the ARQ buffer size in the TX is full, an ARQ feedback poll will be included in block #18 (i.e., the AFP bit=1). Now assume that all but block #14 are received correctly by the RX. The following occurs sequentially based on the latest draft (D4).

- Since the ARQ feedback poll is received, the RX ARQ will send back an ARQ feedback IE with SN=14, Flag=1, NSI=1 and ACK MAP=01111. In addition, the purge timers for blocks #15 to block #18 are started and the error detection timer for block #14 is also started.

- In the meantime, the TX HARQ keeps retransmitting block #14

- TX ARQ receives the ARQ feedback and does nothing (It knows block #14 is pending given that NSI=1 and it also knows the PHY is retransmitting block #14 as the local NACK has not been received yet)

Now assume that the TX HARQ successfully delivers block #14 in the next retransmission. The following will occur

- RX now receives everything and thus delivers 5 blocks to the upper layer. In addition, all running purge timers and error detection timers are cancelled/invalid

- TX HARQ does nothing

- TX ARQ does nothing until all block lifetime timers expire!

Scenario II: Assume that the ARQ window size is 5 and there are 5 HARQ channels. ARQ blocks #14, #15, #16, #17 and #18 are transmitted by the TX to the RX via 5 HARQ channels. Since the ARQ buffer size in the TX is full, an ARQ feedback poll will be included in block #18 (i.e., the AFP bit=1). Now assume that at the very beginning, only block #18 is received correctly by the TX ARQ. The following occurs sequentially based on the latest draft (D4).

- Since the ARQ feedback poll is received, the RX ARQ will send back an ARQ feedback IE with SN=14, Flag=1, NSI=1 and ACK MAP=00001. In addition, the purge timer for block #18 is started and the error detection timers for block #14 to block 17 are also started.

- In the meantime, the TX HARQ keeps retransmitting block #14 to block #17

- TX ARQ receives the ARQ feedback and does nothing (It knows blocks #14 to #17 are pending given that NSI=1 and it also knows the PHY is retransmitting them as the local NACKs have not been received yet)

Now assume that the TX HARQ believes that it successfully delivers blocks #14 and #17 in the next retransmission. However, in fact, block #14 was not delivered correctly (This may occur if the HARQ feedback bit is flipped). Therefore, block #14 will be declared as an error eventually by the RX ARQ and an ARQ feedback will be sent.

If now this ARQ feedback is lost, the ARQ process gets stalled until block lifetime timer of block #14 expires!

The aforementioned problems result from many of the existing designs that we believe ``improper’’. These include

- Insufficient interaction between ARQ and local HARQ --- In the current draft, the ARQ interacts with its local HARQ when the local PHY gives up transmitting an ARQ block. In this case, a local NACK is generated so that the ARQ can retransmit that block. When the local HARQ believes that a block is successfully transmitted, no interaction is activated (which is a waste from the viewpoint of information, as we will point out later)

- Unsolicited ARQ feedback sent by the RX ARQ is unreliable--- In the current draft, an ARQ feedback IE is sent when (1) a block is declared as an error and (2) a purge timer corresponding to a block expires (and the other two situations below). The ARQ feedback sent in these two cases is categorized as an ‘’unsolicited” ARQ feedback since the transmission of the ARQ feedback is not solicited by the TX ARQ.

The problem of unsolicited ARQ feedbacks is that such feedbacks could get lost and TX ARQ has no idea if such feedbacks were ever transmitted. As a result, the information included in the unsolicited ARQ feedback is lost (and cannot be recovered). In contrast, solicited ARQ feedbacks are sent when (3) an ARQ feedback poll is requested by the TX ARQ or (4) when a discard message is sent by TX ARQ. If such solicited ARQ feedbacks are corrupted, TX ARQ will ask for feedback information again when the poll timer or discard message timer expires. In other words, using solicited ARQ feedback is more reliable.

Another problem of unsolicited ARQ feedbacks is that the amount of ARQ feedback exchanged between TX and RX ARQ cannot be easily controlled, given that the RX ARQ may send undue ARQ feedback without any control by TX ARQ.

Proposed text:

Reference

[1] IEEE 802.16m/D4, Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface