| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Channel quality measurements (16.3.14) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Huaning Niu, Yi Hsuan, Hujun Yin, Yang-seok Choi Intel Corp | Huaning.niu@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D4 LB31 |  |
| Abstract | Modifications are proposed for channel quality measurements (16.3.14) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to Channel quality measurements (16.3.14)

Huaning Niu, Yi Hsuan, Hujun Yin, Yang-seok Choi

Intel Corp

# Introduction

RSSI and CINR measurement in 802.16m is defined to be the same as 802.16-2009. This definition is not appropriate due to

- Link adaptation in 16m is based on CQI, which is the effective CINR derived based on different MIMO scheme. The physical CINR and RSSI is mainly used for FFR response, HO and SCAN

- The physical CINR and RSSI are measured using PA-preamble, SA-preamble and midamble, not on the pilots since pilots are precoded and power boosted.

- The 16m CINR and RSSI equation should take into account multiple antennas at transmitter and receiver  side

In this contribution, we propose the 16m physical CINR and RSSI definition. How AMS reports these measurements is controlled by various MAC messages and is not discussed in this contribution ([2][3]).

- References

[1] Draft Amendment, P802.16m/D4, Feb. 2010.

[2] C80216m-10_0081.

[3] C80216m-10_0082.

# Text proposal for inclusion in the 802.16m amendment

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-------------------------------------------------- Text Change ------------------------------------------------

16.3.14 Channel quality measurements

16.3.14.1 Introduction

RSSI and CINR measurements and associated statistics can help characterizing signal quality as well as assisting BS in selection/assignment of radio resources to the mobile stations according to their channel conditions. As channel behavior is time-variant, both mean and standard deviation are required to be defined.

RSSI and CINR measurements and associated statistics  may be used for characterizing signal quality, assisting ABS in selection/assignment of frequency partition, HO and SCAN. As channel behavior is time variance, both mean and standard deviation are required to be defined.

16.3.14.2 RSSI mean and standard deviation

The method for calculation of the RSSI and its mean and standard deviation in IEEE 802.16m shall be the same as that described in 8.4.12.2 of IEEE Std 802.16-2009.

When collection of RSSI measurements is mandated by the ABS, an AMS shall obtain an RSSI measurement from the DL SA-preamble and mid-amble. From a succession of RSSI measurements, the AMS shall derived and update estimates of the mean and the standard deviation of the RSSI, and report them via AAI_FFR-REP, AAI_HO-REQ, AAI_SCN-REP messages.

Mean and standard deviation statistics for RSSI shall be reported in units of decibels relative to 1 mW  and decibels respectively, following the quantization requirement in each message.

The method used to estimate the RSSI of a single measurement is left to individual implementation, which can be derived from thePsignal[k]defined in session 16.3.14.3.

The (linear) mean RSSI statistics (in milliwatts), derived from a multiplicity of single measurement, shall be updated using Equation (x)

μRSSI[k]=R[0]k=0(1−αavg)n+1μRSSI[k−1]+(1−(1−αavg)n+1)R[k]k>0          mW           (x)

Where

k is the time index for the measurement; and n is the number of consecutive frames in which no measurement is made.

R[k] is the RSSI in mW measured during message k, and αavgis an averaging parameter specified by the ABS in AAI_XXX_XXX (to be defined).

μRSSIdBm[k]=10log(μRSSI[k])           dBm

To solve the standard deviation in dB, the expectation-squared statistic shall be updated using Equation (xx)

xRSSI2[k]=|R[0]|2k=0(1−αavg)n+1xRSSI2[k−1]+(1−(1−αavg)n+1)|R[k]|2k>0                 (xx)

σRSSIdB[k]=5log(xRSSI2[k]−μRSSI2[k])dB

16.3.14.3 CINR mean and standard deviation

The method for calculation of the CINR and its mean and standard deviation in IEEE 802.16m shall be the same as that described in 8.4.12.3 of IEEE Std 802.16-2009.

When CINR measurements are mandated by the ABS, an AMS shall obtain a CINR measurement from the DL PA-preamble, SA-preamble and mid-amble. From a succession of these measurements, the AMS shall derive and update estimates of the mean and standard deviation of the CINR, and report them via AAI_FFR-REP, AAI_HO-REQ, AAI_SCN-REP messages.

Mean and standard deviation statistics for CINR shall be reported in units of dB, following the quantization requirement in each message.

The method used to estimate the CINR of a single measurement is left to individual implementation. Possible method to estimate CINR is

CINRk=PsignalkPinterference[k]+Pnoise[k]

Where CINR[k] is the CINR for measurement k,

Psignal[k] is the signal power for measurement k,

Pinterference[k]is the interference power for measurement k.

Pnoise[k] is the noise power for measurement k.

rPA−preamblek,i, rSA−preamblek,iis the received signal vector for PA-preamble and SA-preamble in i-th subcarrier of measurement k.

rmidamblek,i is the Nr by Nt received signal matrix from midamble at i-th subcarrier of measurement k.

An example method to estimate the noise power using unoccupied subcarriers of the PA-preamble symbol is expressed by equation (1) as

Pnoise[k]=1Ni=0N−1rPA−preamblek,2⋅i2                           (1)

The Pinterference[k]andPsignal[k] can be calculated using either SA-preamble or mid-amble. An example method to estimate the interference power for segment 0 from mid-amble is described below. The reuse-1 partition interference power can be calculated as

Pinterference[k]=1Nall i∈T0Wr×ymidamble[k,i]−h[k,i]×Wt2+1Nall i∈T1Wr×ymidamble[k,i]×Wt2+1Nall i∈T2Wrymidamble[k,i]×Wt2−3Pnoise

where Tsis a set of measurement indices derived from the midamble tone indices of segment s; Wtis the non-adaptive precoding codeword selected from CDL,OL,SUNt,1,Nwbased on eqn (240); h[k,i]is theNr×Nt unbiased channel estimation based on mid-amble for sector 0; ymidamble[k,i]=12.18j=0Nt−1rmidamble(i+3j)is the combinedNr×Nt received signal, Wr is the implementation dependant receiving vector at AMS.

The interference power for reuse-3 partition can be calculated as

Pnoise+interference[k]=Pboost1Nall i∈T0Wr×ymidamble[k,i]−h[k,i]×Wt2−Pnoise2+Pde−boost21Nall i∈T1Wr×ymidamble[k,i]×Wt2−Pnoise+Pde−boost31Nall i∈T2Wrymidamble[k,i]×Wt2−Pnoise

where Pboostis FP1 power for segment 0, and Pde−boost2 FP2 power broadcasted in SFH or AAI_DL-IM, Pde−boost3 FP3 power broadcasted in SFH or AAI_DL-IM, and Wtis the non-adaptive precoding codeword selected from CDL,OL,SUNt,1,Nwbased on eqn (240);

The signaling power Psignal[k] for reuse-1 FP can be calculated as

Psignal[k]=1Nall i∈T0h[k,i]×WtF

where Wtis the non-adaptive precoding codeword selected from CDL,OL,SUNt,2,Nwbased on eqn (240);

The signaling power Psignal[k] for boosted reuse-3 FP can be calculated as

Psignal[k]=PboostNall i∈T0h[k,i]×WtF

where Wtis the non-adaptive precoding codeword selected from CDL,OL,SUNt,2,Nwbased on eqn (240);

An example method to estimate the interference and signal power from SA-preamble is described below.

The reuse-1 partition interference power can be calculated as

Pinterference[k]=1Nall i∈T0Wr×ySA−preamble[k,i]−h[k,i]2+1Nall i∈T1Wr×ySA−preamble[k,i]2+1Nall i∈T2WrySA−preamble[k,i]2−3Pnoise

where Tsis a set of subcarrier indices derived from the SA-preamble tone indices of segment s; h[k,i]is theNr×1 unbiased equivalent channel estimation based on SA-preamble for sector 0, ySA−preamble[k,i]=1BprSA−preamble[k,i]is the normarlizedNr×1 received signal where Bpis the amplitude boosting value for SA-preamble defined in Table 799,Wr is the implementation dependant receiving vector at AMS.

The interference power for reuse-3 partition can be calculated as

Pnoise+interference[k]=Pboost1Nall i∈T0Wr×ySApreamble[k,i]−h[k,i]2−Pnoise2+Pde−boost21Nall i∈T1Wr×ySApreamble[k,i]2−Pnoise+Pde−boost31Nall i∈T2WrySApreamble[k,i]2−Pnoise

where Pboostis FP1 power for segment 0, Pde−boost2 FP2 power broadcasted in SFH or AAI_DL-IM, Pde−boost3 FP3 power broadcasted in SFH or AAI_DL-IM.

The signaling power Psignal[k] in reuse-1 partition can be calculated as

Psignal[k]=1Nall i∈T0h[k,i]F.

The signaling power Psignal[k] in boosted reuse-3 partition can be calculated as

Psignal[k]=PboostNall i∈T0h[k,i]F.

The mean CINR statistics (in decibels) shall be derived from a multiplicity of single message using Equation (2).

μCINRdB[k]=10log(μCINRdB[k])              (2)

where

μCINRdB[k]=CINR[0]k=0(1−αavg)n+1μCINRdB[k−1]+(1−(1−αavg)n+1)CINR[k]k>0

αavgis an averaging parameter specified by the ABS in AAI_XXX_XXX (to be defined); and n is the number of consecutive frames in which no measurement is made. To solve the standard deviation, the expectation-squared statistic shall be updated using Equation (3)

xCINRdB2[k]=|CINR[0]|2k=0(1−αavg)n+1xCINRdB2[k−1]+(1−(1−αavg)n+1)|CINR[k]|2k>0

And

σCINRdB[k]=5log(xCINR2[k]−μCINRdB2[k])dB