| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized format for AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP messages (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngkyo Baek Hyeonjeong Kang Jaehyuk Jang Youngbin Jang Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.3.4” |  |
| Abstract | This contribution provides ASN.1 format for network entry related MAC control messages |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized format for AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP messages (16.2.3)  Youngkyo Baek, Hyeonjeong Kang, Jaehyuk Jang,Youngbin Jang

Samsung Electronics

# Introduction

All MAC control message should be converted to ASN.1 format. This contribution suggest ASN.1 format for the Network entity related MAC control messages. especially SBC and REG messages are suggested in this contribution.

# 1Proposed Text#1

Modify the sentences and table, line 32, page 70 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

16.2.3.4 AAI_SBC-REQ

An AAI_SBC-REQ message, to which HARQ operation is applied, is transmitted by AMS to negotiate basic capability during network entry.

The AAI_SBC-REQ message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed. In Table 679, the CAPABILITY_INDEX transmitted in the AAI_SBC-REQ message refers to the maximum "Capability Class" that the AMS can support. The maximum value of CAPABILITY_INDEX is denoted by [TBD] 8bits.

Table 679 - AAI_SBC-REQ message format Field Descriptions

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| ………….. | ……………………… | ……………… |
| …………… | ……… | ……….. |

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CAPABILITY_INDEX | 8 | It refers to the "Capability Class" that the AMS can support |  |

The following parameters(Table xxx)  may be included negotiated and parameter sets are mapped to capability index (the mapping is TBD):

•Authorization policy support

•-If Bit #0=0, EAP-based authorization is not supported

•-If Bit #0=1: EAP-based authorization is supported

•PN Window Size : Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks

•Auth type for EAP : Auth Type for EAP shall only be included when EAP-based authorization is supported.

•If Bit #0=0, device authentication

•If Bit #0=1, user authentication

Table xxx. AMS Capabililies to be trasmitted to ABS

| Nego parameters | value |
| --- | --- |
| Long TTI for UL | If Bit#0 =1,it supports |
| Long TTI for DL | If Bit#0 =1,it supports |
| UL sounding | If Bit#0 =1, UL sounding based MIMO supports If Bit#1=1, Unquantized MIMO feedback through Sounding supports |
| DL FFR | If Bit#0 =1,it supports |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |
| feedback support for DL | If Bit#0 =1, base mode supports If Bit#1 =1, trasnformation mode supports If Bit#2 =1, differential mode supports If Bit#3 =1, MIMO feedback mode 0 supports If Bit#4 =1, MIMO feedback mode 1 supports If Bit#5=1, MIMO feedback mode 2 supports If Bit#6 =1, MIMO feedback mode 3 supports If Bit#7 =1, MIMO feedback mode 4 supports If Bit#8 =1, MIMO feedback mode 5 supports If Bit#9 =1, MIMO feedback mode 6 supports If Bit#10 =1, MIMO feedback mode 7 supports If Bit#11=1, covariance matrix feedback supports |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |
| Inter-RAT Operation Mode | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 – 0b11 : reserved |
| Supported Inter-RAT type | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |
| MIH Capability Supported | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |

16.2.3.5 AAI_SBC-RSP

An AAI_SBC-RSP message, to which HARQ operation is applied, is transmitted by ABS in response to a received AAI_SBC-REQ during initialization.

The AAI_SBC-RSP message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed. In Table 680, the CAPABILITY_INDEX transmitted in the AAI_SBC-RSP message refers to the "Capability Class" that the ABS has allowed the AMS to use during the session. The value of CAPABILITY_INDEX signaled in the AAI_SBC-RSP message is numerically smaller than or equal to the CAPABILITY_INDEX signaled in the AAI_SBC-REQ message by the AMS.

Table 680 - AAI_SBC-RSP message format Field Descriptions

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| ………….. | ……………………… | ……………… |
| …………… | ……… | ……….. |

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CAPABILITY_INDEX | 8 | It refers to the "Capability Class" that the ABS has allowed the AMS to use. |  |

The following parameters(Table yyy) may be included negotiated and parameter sets are mapped to cabability index(the mapping is TBD):

•Authorization policy support

•If Bit #0=0, EAP-based authorization is not supported;

•If Bit #0=1: EAP-based authorization is supported

•PN Window Size : Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks

Table yyy. AMS Capabililities to be confirmed by ABS

| Nego parameters | value |
| --- | --- |
| Long TTI for UL | If Bit#0 =1,it supports |
| Long TTI for DL | If Bit#0 =1,it supports |
| UL sounding | If Bit#0 =1, UL sounding based MIMO supports If Bit#1=1, Unquantized MIMO feedback through Sounding supports |
| DL FFR | If Bit#0 =1,it supports |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |
| feedback support for DL | If Bit#0 =1, base mode supports If Bit#1 =1, trasnformation mode supports If Bit#2 =1, differential mode supports If Bit#3 =1, MIMO feedback mode 0 supports If Bit#4 =1, MIMO feedback mode 1 supports If Bit#5=1, MIMO feedback mode 2 supports If Bit#6 =1, MIMO feedback mode 3 supports If Bit#7 =1, MIMO feedback mode 4 supports If Bit#8 =1, MIMO feedback mode 5 supports If Bit#9 =1, MIMO feedback mode 6 supports If Bit#10 =1, MIMO feedback mode 7 supports If Bit#11=1, covariance matrix feedback supports |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |
| Supported Inter-RAT type | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |
| MIH Capability Supported | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# 2Proposed Text#2

Modify the sentences and table, line 46, page 72 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

16.2.3.7 AAI_REG-REQ

An AAI_REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

The AAI_REG-REQ message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

The following parameter is included in the AAI_REG-REQ message:

—AMS MAC address: This is used to derive security keys.

Table 682 - Parameters for AAI_REG-REQ message Field Descriptions

| Name | value |
| --- | --- |
| ………….. | ……………………… |
| …………… | ……… |

The following parameters may be included:

AMS Scanning Capability:

This field indicates properties of the AMS that the ABS needs to know for scanning purposes:

-Number of Center Frequencies AMS can scan during one scan iteration.

-Maximum number of preambles per Center Frequency AMS can scan during one iteration.

-Number of frames required to complete scanning of all preambles for a given frequency.

The length of the field is 3 Bytes, which are divided as follows:

•Bit 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame

•Bit 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed.

•0b000 = 0 sub-frames

•0b111 = 7 sub-frames)

•Bits 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned.

•0b000 = 0 sub-frames

•0b111 = 7 sub-frames)

•Bits 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data.

•0b000 = 0 sub-frames

•0b111 = 7 sub-frames)

•Bit 14-15: Minimum supported scan duration

•(0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames)

•Bit 16-23: Maximum supported scan duration

•(0x0 = no limit

•All other values: maximum scan duration in number of sub-frames))

| M/O | Attributes / Array of attributes |  |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | AMS MAC address |  |  |  |  | 48 | This is used to derive security keys |  |
| O | AMS capability negotiation parameters | ARQ parameters |  | ARQ_SN_MODULUS |  | 10 | the number of unique SN values |  |
| O |  |  |  | ARQ_WINDOW_SIZE |  | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O |  |  |  | ARQ_SUB_BLOCK_SIZE |  | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  |  | ARQ_BLOCK_LIFETIME |  | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  |  | ARQ_RX_PURGE_TIMEOUT |  | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O |  |  |  | MAXIMUM_ ARQ_BUFFER_SIZE |  | TBD | The maximum ARQ buffer size Depend on AMS’s memory |  |
| O |  | Multicarrier capabilities |  |  |  | 2 | 0b00: No MC modes 0b01: Multicarrier aggregation 0b10: Multicarrier switching 0b11: Both multicarrier aggregation and switching |  |
| O |  | LBS Capabilities | Capability for supporting A-GPS method |  |  | 1 | 0b0: no support 0b1: support |  |
| O |  | Capabilities for interference mitigation support | DL PMI coordination capability |  |  | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
|  |  |  | DL collaborative multi-BS MIMO capability |  |  | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
|  |  |  | DL closed-loop multi-BS macro diversity capability |  |  | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro di¬versity capable |  |
| O |  |  | UL PMI combination capa¬bility |  |  | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable |  |
| O |  | E-MBS capabilities |  |  |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O |  | channel BW and Cyclic prefix |  |  |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |  |
| O |  | frame configuration to support legacy(5MHz) |  |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | frame configuration to support legacy(10MHz) |  |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | MAX Tx Power |  |  |  | 24 | Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. |  |
| O |  | 3-step BR |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | dynamic ranging for HO |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Persistent Allocation support |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Group Resource Allocation support |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Co-located coexistence capability support |  |  |  | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |  |
| O |  | HO trigger metric |  |  |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |  |
| O |  | Capabilities for interference mitigation support |  |  | DL PMI coordination capability | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O |  |  |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O |  | AMS Scanning Capability |  |  |  | 24 | This field indicates properties of the AMS that the ABS needs to know for scanning purposes: - Number of Center Frequencies AMS can scan during one scan iteration. - Maximum number of preambles per Center Frequency AMS can scan during one iteration. - Number of frames required to complete scanning of all preambles for a given frequency The length of the field is 3 Bytes, which are divided as follows: Bit# 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame Bit# 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bits# 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bits# 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bit# 14-15: Minimum supported scan duration • (0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames) Bit# 16-23: Maximum supported scan duration • (0x0 = no limit • All other values: maximum scan duration in number of sub-frames)) |  |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  |  |  | TBD | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O |  | Maximum number of classification rules |  |  |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  |  |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  |  |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | Supported-IP-Versions IE |  |  |  | 2 | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4 and IPv6 support |  |
| O | Host configuration capabilities and parameters | Host-Configuration-Capability-Indicator IE |  |  |  | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |  |
| O |  | Requested-Host-Configurations IE |  |  |  | 1 | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |  |

16.2.3.8 AAI_REG-RSP

An AAI_REG-RSP message is transmitted by ABS in response to AAI_ REG-REQ message during initialization.

The AAI_REG-RSP message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

The following parameter is included in the AAI_REG-RSP message:

—STID: AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message.

—CRID (Context Retention Identifier): AMS identifier which the AMS has been assigned for coverage loss or DCR mode

The following parameters may be included:

Table 683 - Parameters for AAI_REG-RSPmessage Field Descriptions

| Name | value |
| --- | --- |
| ………….. | ……………………… |
| …………… | ……… |

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | STID |  |  | 48 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. |  |
| M | CRID |  |  | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode |  |
| O | AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | 10 | the number of unique SN values |  |
| O |  |  | ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O |  |  | ARQ_SUB_BLOCK_SIZE | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  | ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  | ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O |  |  | MAXIMUM_ ARQ_BUFFER_SIZE | TBD | The maximum ARQ buffer size Depend on AMS’s memory |  |
| O |  | Capabilities for interference mitigation support | DL PMI coordination capability | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable |  |
| O |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable |  |
| O |  |  | DL closed-loop multi-BS macro diversity capability | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable |  |
| o |  |  | UL PMI combination capability | 1 | 0: ABS is not UL PMI combination capable 1: ABS is UL PMI combination capable |  |
| O |  | LBS Capabilities | Capability for support A- GPS method | 1 | 0b0: no support 0b1: support |  |
| O |  | Multicarrier capabilities |  | 2 | 0b00: No MC modes 0b01: Multicarrier aggregation 0b10: Multicarrier switching 0b11: Both multicarrier aggregation and switching |  |
| O |  | E-MBS capabilities |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O |  | channel BW and Cyclic prefix |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |  |
| O |  | frame configuration to support legacy(5MHz) |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | frame configuration to support legacy(10MHz) |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | 3-step BR |  | 1 | If Bit#0 =1,it supports |  |
| O |  | dynamic ranging for HO |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Persistent Allocation support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Group Resource Allocation support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | HO trigger metric |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |  |
| O |  | Co-located coexistence capability support |  | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |  |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  | TBD | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O |  | Maximum number of classification rules |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | Resource_Retain_Time |  | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |
| O |  | IP-Service-Type IE |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved |  |
| O | Host configuration capabilities and its parameters | IPv4-Host-Address IE |  | 32 | The allocated IPv4 Host Address for the AMS. |  |
| O |  | IPv6-Home-Network-Prefix IE |  | 64 | The allocated IPv6 Home Network Prefix for the AMS. |  |
| O |  | Additional-Host-Configurations IE |  | TBD | Includes additional host configuration options in DHCP Options format |  |
| O | Redirection Info | ABSID, preamble index and center frequency for one or more neighbor ABS |  | TBD | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# References

- IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.