| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Emergency Alert Service (16.2.12) |  |
| Date Submitted | 2010-03-10 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) | hcy@iii.org.tw |
| Re: | In response to IEEE 802.16 Working Group Working Group Letter Ballot #31: Announcement |  |
| Abstract | This contribution proposes text for emergency alert service |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Emergency Alert Service (16.2.12)

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry (III)

Introduction

Emergency information is transmitted to AMSs when needed. For sleep AMSs that require AAI_TRF-IND, emergency alert indication can be transmitted in AAI_TRF-IND with a specific SLPID=1023. For sleep AMSs that do not require AAI_TRF-IND, emergency alert indication can be transmitted in SCEH with a specific sub-type=0b101. Having received the emergency alert indication, the AMS shall decode the A-A-MAP to obtain the emergency information.

The emergency information transmission time is a policy issue and thus it is out of scope of this standard.

Proposed text for the P80216m/D4:

-----------------------------------------------Start of the Text Proposal # 1---------------------------------------------

[Make the following changes to Section 16.2.12.7 in Page 264 beginning Line 51 in P802.16m/D4.]

#### 16.2.12.7 Emergency Alert Service

Emergency alert service is defined as a service that would provide the public with alerts on imminent emergency events, such as earthquake, storm, etc. The alerts would target subscribers in a specific geographical location. The emergency alert service includes the transmissions of emergency information. The emergency information is broadcasted through AAI_L2-XFER. The emergency information transmission time is a policy issue and thus it is out of scope of this standard.

-----------------------------------------------End of the Text Proposal # 1---------------------------------------------

-----------------------------------------------Start of the Text Proposal # 2---------------------------------------------

[Make the following changes to Section 16.2.2.2.5 in Page 53 beginning Line 36 in P802.16m/D4.]

#### 16.2.2.2.5 Sleep Control extended header (SCEH)

Table 669 – SCEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Sleep Control Extended Header Format () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC Signaling Header Type = 0b0011 |
| SCEH sub-type | 3 | 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b100 = Multi-Carrier Listening Window control 0b101 = emergency alert indication, when SCEH sub-type =0b101, the AMS shall decode the A-A-MAP to obtain the emergency information. 0b11001~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e. acknowledgment) to the request . |
| if (SCEH sub-type == Listening Window Control) { | - | - |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| if (Listening Window End or Extension == 1) { | - | - |
| Last frame of Extended Listening Window | 7 | LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } | - | - |
| } else if (SCEH sub-type == Resume Sleep Cycle Indication) { | - | - |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { | - | - |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 7 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 128). |
| } | - | - |
| } | - | - |
| else if (SCEH sub-type == Sleep Cycle configuration switch) { | - | - |
| SCID | 4 | SCID corresponds to the new sleep cycle setting to be switched. |
| Start Frame Offset for new sleep cycle configuration | 3 | Least Significant 3 bits of Frame Number in which sleep cycle setting is to be applied. |
| } | - | - |
| else if (SCEH sub-type == Sleep Cycle control) { | - | - |
| Next Sleep Cycle Flag (NSCF) | 2 | 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |
| if (NSCF == 0b10) | - | - |
| { | - | - |
| New Initial Sleep Cycle | 4 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. |
| } | - | - |
| } | - | - |
| else if (SCH sub-type == Multi- Carrier Listening Window control) { | - | - |
| Target Carrier Index Bitmap | 4 | If nth bit is set to 1, it indicates that DL data transmission on the secondary carrier of which logical carrier index is equal to (n+1) ends. |
| } | - | - |
| Padding | variable | For byte alignment |
| } | - | - |

-----------------------------------------------End of the Text Proposal # 2---------------------------------------------

-----------------------------------------------Start of the Text Proposal # 3---------------------------------------------

[Make the following changes to Section 16.2.3.25 in Page 111 beginning Line 6 in P802.16m/D4.]

#### 16.2.3.25 AAI_TRF-IND

Table 703 - AAI_TRF-IND message format

| Name | Value | Usage |
| --- | --- | --- |
| FRMT | 0 : It indicates the SLPID bitmap-based traffic indication 1: It indicates the SLPID-based traffic indication | This indicates type of Traffic Indication in AAI_TRF-IND message. |
| SLPID Group Indication Bitmap | 32bits N-th bit of SLPID-Group Indication Bitmap [MSB corresponds to N = 0] is allocated to SLPID Group that includes AMS with SLPID values from N*32 to N*32+31 0: There is no traffic for any of the 32 AMSs that belong to the SLPID-Group 1: There is traffic for at least one AMS in SLPID-Group. | This field appears when FMT is set to 0. It indicates the existence of each SLPID group |
| Traffic Indication Bitmap | Multiple of 32bits (i.e. 0 ~ 32*32bits) Each Traffic Indication bitmap comprises multiples of 32-bit long Traffic Indication units. A Traffic Indication unit for 32 SLPIDs is added to AAI_TRF-IND message whenever its SLPID Group is set to 1 32 bits of Traffic Indication Unit (starting from MSB) are allocated to AMS in the ascending order of their SLPID values: 0: Negative indication 1: Positive indication | This field appears when FMT is set to 0. It indicates the traffic indication for 32 AMSs in each SLPID group |
| Num_of_SLPIDs | 0~63 | This field appears when FMT is set to 1. It indicates the number of SLPID included in AAI_TRF-IND message |
| SLPID | 0~10223 1023 is used for emergency alert indication | This field appears when FMT is set to 1. Each SLPID is used to indicate the positive traffic indication for an AMS |

-----------------------------------------------End of the Text Proposal # 3---------------------------------------------

[1] IEEE P80216m/D4, “P802.16m/D4: Advanced Air Interface”