| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications for Sleep Cycle Operation Text (16.2.16) |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB31 / Area: Chapter 16.2.16.2.1 (Sleep Cycle operations) |  |
| Abstract | This contribution proposes changes to the Section 16.2.16 of IEEE P802.16m/D4 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications for Sleep Cycle Operation Text (16.2.16)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

The description for sleep cycle operation (16.2.16.2.1) is not clear enough for implementation. This contribution provides clean-up text to be adopted in D5.

- Proposed Text for IEEE P802.16m/D4

[Accept the following modifications marked in blue and red]

============================ Start of Proposed Text ==================================

16.2.16.2 Sleep Mode operation

16.2.16.2.1 Sleep Cycle operations

[Accept the following modification from line 40 on page 299]

The length of a Sleep Cycle may be changed implicitly. If there is negative indication in the traffic indication message or if there is no data traffic during the Listening Window or and if the NSCF is set to 0b01, the AMS and ABS shall update the length of the Sleep Cycle as follows:

Current Sleep Cycle = min( 2 x Previous Sleep Cycle, Final Sleep Cycle)…………..….(x)

The value of the Default Listening Window shall remain unchanged when Sleep Cycle is changed implicitly according to Equation (x).

[Accept the following modification from line 8 on page 300]

When Final Sleep Cycle is equal to or larger than 2 times the Initial Sleep Cycle, the length of Sleep Cycle exponentially doubles until the Final Sleep Cycle is reached. This Sleep Cycle operation is suitable for BE-traffic scenario. If the traffic indication message is positive for the AMS, then the length of the current Sleep Cycle shall be determined based on the value of NSCF which was included in the AAI_SLP-RSP or SCH or SCEH. The Sleep Cycle could be the different length according to the Next Sleep Cycle Flag (NSCF) within the AAI_SLP-REQ/RSP message, SCH or SCEH. If the NSCF is set to 0b00 then the Initial Sleep Cycle is always the same as the first Initial one. When the NSCF is set to 0b01 then current sleep cycle is doubled in previous sleep cycle.

When NSCF is set to 0b10, during sleep mode initiation the AMS negotiates with the ABS the new initial sleep cycle value to be used after positive traffic indicator.

The current Sleep Cycle shall be determined according to the rules listed in Table XXX.

Table XXX –Sleep Cycle Operation Rules.

|  | TIMF = 0 |  | TIMF = 1 |  |
| --- | --- | --- | --- | --- |
|  | No data during LW | Having data during LW | Negative indication | Positive indication |
| NSCF = 00 | Current Sleep Cycle = Initial Sleep Cycle | Current Sleep Cycle = Initial Sleep Cycle | Current Sleep Cycle = Initial Sleep Cycle | Current Sleep Cycle = Initial Sleep Cycle |
| NSCF = 01 | Follow Equation (x) | *Current Sleep Cycle = Initial Sleep Cycle | Follow Equation (x) | *Current Sleep Cycle = Initial Sleep Cycle |
| NSCF = 10 | Current Sleep Cycle = New Sleep Cycle | Current Sleep Cycle = New Sleep Cycle | Current Sleep Cycle = New Sleep Cycle | Current Sleep Cycle = New Sleep Cycle |

* If only MAC control messages or short messages are transmitted during Listening Window, the current Sleep Cycle should be updated by following Equation (x).

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.