| 2Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clarifications on S-SFH contents (D5 - 16.3.6.5.1.2) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution proposes some modification on section 16.3.6.5.1.2 for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clarifications on S-SFH contents (D5 - 16.3.6.5.1.2)

Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for clarifications for S-SFH contents in the IEEE 802.16m DRAFT Amendment [1].

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D5, page 529, line 1, make the following changes to subclause 16.3.6.5.1.2]

16.3.6.5.1.2 S-SFH IE

Table 837—S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Uplink AAI subframes for sounding | 3 | This value represents the number of uplink AAI subframes with sounding symbols. 0b000 – no sounding symbols 0b001 – 1 AAI subframe 0b010 – 2 AAI subframes 0b011 – 3 AAI subframes 0b100 – 4 AAI subframes 0b101-111 – reserved The sounding symbols shall be placed in AAI subframes in accordance to their type. First, sounding symbols shall be allocated in uplink AAI subframes of type 2 starting from the first in time AAI subframe of type 2. If the number of uplink AAI subframe of type 2 is less than the number of AAI subframes for sounding, sounding symbols shall be allocated in the AAI subframes of other types in the following order: type 1. For these types of uplink AAI subframes sounding symbols shall be allocated in the similar way as for type 2. Type 3 uplink AAI subframes are not used for sounding. |
| ABS EIRP | 6 | Unsigned integer from 1 to 64 in units of 1 dBm, where 0b000000=1 dBm and 0b111111=64 dBm. |
| Cell bar information | 1 | If Cell Bar bit = 0b1, this cell is not allowed for network entry or re-entry |
| UL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for UL HARQ 0b0: 4 0b1: 8 |
| DL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for DL HARQ 0b0: 4 0b1: 8 |
| TUL_Rx_Processing | 1 | Specifies the ABS's Rx processing time for UL HARQ for F= 8 in FDD or D + U = 8 in TDD 0b0: 3 AAI subframes 0b1: 4 AAI subframes |
| Femto Indicator | 1 | Indicates whether the ABS is a Femto ABS or not. 0b0: ABS is not a femto ABS. 0b1: ABS is a femto ABS. |

[On the IEEE P802.16m/D5, page 531, line 43, make the following changes to subclause 16.3.6.5.1.2]

Table 838—S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| SFH AAI MAC version | 4 | An unsigned 4-bit quantity equal to the value of the MAC version TLV (see 11.1.3), minus 10 |
| If (Duplexing mode == FDD) { |  | The duplexing mode is obtained from the frame configuration index set in S-SFH SP1 IE |
| UL to DL carrier frequency separation | 6 |  |
| UL bandwidth | 3 | The frequency spacing for UL channel is same with DL channel. 0b000: 512 FFT 0b001: 1024 FFT 0b010: 2048 FFT 0b011 to 0b111: reserved |
| } |  |  |
| MSB bytes of 48 bit ABS MAC ID | 36 | Specifies 36 MSB of BS ID Remaining bits of ABS MAC ID except LSB of 48 bit ABS MAC ID in S-SFH SP1 |
| Network Configuration | 1 | Indicates configuration of the ABS network 0b0: AAI network configuration 0b1: WirelessMAN-OFDMA network configuration |
| OLRegion | 1 | Provides indication about the structure of the MIMO OL Region. Further details in Section 16.3.5.3.1. |
| DSAC | 5/4/3 | Indicate the number of subbands KSB as defined in Table 807 to Table 809 in 16.3.5.2.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| DFPC | 4/3/3 | Indicate the frequency partition configuration as defined in Table 786810 to Table 812 in 16.3.5.2.3 For 2048 FFT size, 4 bits For 1024 FFT size, 3 bits For 512 FFT size, 3 bits |
| DFPSC | 3/2/1 | Indicate the number of subbands allocated to FPi, i>0 in 16.3.5.2.3 For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |
| USAC | 5/4/3 | Indicate the number of subbands KSB as defined in Table 899 to Table 901 in 16.3.8.2.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits For support of WirelessMAN-OFDMA with FDM-based UL PUSC Zone, all bits shall be set to zero. |
| UFPC | 4/3/3 | Indicate the frequency partition configuration as defined in Table 879902 to Table 904 in 16.3.8.2.3 For 2048 FFT size, 4 bits For 1024 FFT size, 3 bits For 512 FFT size, 3 bits For support of WirelessMAN-OFDMA with FDM-based UL PUSC Zone, all bits shall be set to zero. |
| UFPSC | 3/2/1 | Indicate the number of subbands allocated to FPi, i>0 in 16.3.8.2.3 For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits For support of WirelessMAN-OFDMA with FDM-based UL PUSC Zone, all bits shall be set to zero. |
| AMS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed AMS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| EIRxPIR,min | 5 | Unsigned integer from -133 to -102 in units of 1dBm, in units of 1dBm, where 0b00000=-133 dBm and 0b11111=-102 dBm. |
| reserved |  |  |
| } |  |  |

[On the IEEE P802.16m/D5, page 534, line 32, make the following changes to subclause 16.3.6.5.1.2]

Table 839—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL BW REQ channel information | 3 | Indicates the number and the location of UL AAI subframe where the UL BW REQ channel is allocated. The maximum number of UL BW REQ channels in a frame is four. 0b000: First UL AAI subframe in the first frame in every superframe 0b001: First UL AAI subframe in the first and second frame in every superframe 0b010: First UL AAI subframe in every frame 0b011: Every First 2 UL AAI subframes in every frame 0b100: First 4 UL AAI subframes in every frame 0b101~111: reserved |

-----------------------------------------------------------End of the Text---------------------------------------------------------