| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup for the AAI_LBS-IND MAC control message (16.8.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Hai Wang Beijing Samsung Telecom R&D Center Sangheon Kim, Ji-Yun Seol, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D6 |  |
| Abstract | Cleanup for the AAI_LBS-IND message to support U-TDOA on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup for the AAI_LBS-IND MAC control message (16.8.2)

Lei Zhou, Xufeng Zheng, Hai Wang,

Beijing Samsung Telecom R&D Center

Sangheon Kim, Ji-Yun Seol, Jaeweon Cho,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution clarifies the AAI_LBS-IND message in the IEEE 802.16m Amendment Draft Standard Error: Reference source not found to support U_TDOA clearly.

Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Adopt the following text modification started from line# 56 Pages 841 in P802.16m/D6]

16.8.2.4.2 LBS Measurement Message formats

The AAI defines AAI_SCN-REQ/RSP/REP messages to assign the unavailable interval for the AMS to measure its location by receiving the DL reference signals from the candidate ABSs.

The location measurement procedure may be initiated by either the AMS or the network, which is requested by the application associated with the AMS or the client attached at the core network.

An ABS may trigger a location measurement by sending AAI_SCN-RSP to the AMS. The AMS responds to this with AAI_SCN-REP with the parameters that enable location determination.

The AAI defines AAI_LBS-IND message for U-TDOA.

Table 978—parameters for AAI_ LBS-IND

| M/O | Name | Size(bits) | Value/Note | Condition |
| --- | --- | --- | --- | --- |
|  | Dedicated CDMA ranging code of neighbor ABS | 5 | Indicates the dedicated ranging code |  |
|  | Ranging opportunity index of neighbor ABS | 3 | Indicates the index of the allocated ranging opportunity |  |
|  | Subframe Index of neighbor ABS | 3 | Indicates the subframe index of the allocated ranging opportunity |  |
|  | Dedicated CDMA ranging code for the serving ABS | 5 | Indicates the dedicated ranging code. |  |
|  | Ranging opportunity index for the serving ABS | 3 | Indicates the index of the allocated ranging opportunity. |  |
|  | Subframe Index for the serving ABS | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| M | U-TDOA Mode | 1 | 0b0: Special U-TDOA 0b1: General U-TDOA |  |
| M | Frame identifier | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe |  |
| M | Dedicated CDMA ranging code for the serving ABS | 5 | Indicates the dedicated ranging code. |  |
| M | Ranging opportunity index for the serving ABS | 3 | Indicates the index of the allocated ranging opportunity. |  |
| M | Subframe Index for the serving ABS | 2 | Indicates the subframe index of the allocated ranging opportunity. |  |
| O | Neighbor ABS index | Variable 8xN | ABS index corresponds to the position of ABS in AAI_NBR-ADV message | Shall be included for each target ABS when U-TDOA Mode =0b01 |
| O | Action time | 8 | Action Time included in this message is the absolute frame number at the serving ABS. | Shall be included for each target ABS when U-TDOA Mode =0b01 |
| O | Dedicated CDMA ranging code of neighbor ABS | 5 | Indicates the dedicated ranging code | Shall be included for each target ABS when U-TDOA Mode=0b01. |
| O | Ranging opportunity index of neighbor ABS | 3 | Indicates the index of the allocated ranging opportunity | Shall be included for each target ABS when U-TDOA Mode=0b01. |
| O | Subframe Index of neighbor ABS | 2 | Indicates the subframe index of the allocated ranging opportunity | Shall be included for each target ABS when U-TDOA Mode=0b01. |

Figure NN Operation description on AAI for General U-TDOA

1. Serving BS requests neighbor BS to assign the dedicated ranging opportunity for the MS.

2. Neighbor BS confirms the allocation of the dedicated ranging opportunity for the MS and returns the related parameters used for dedicated ranging between the MS and the neighbor BS.

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

3. Serving BS allocates a CDMA code and dedicated ranging opportunity to MS.

4. AAI_LBS-IND message is sent to the MS to request the MS to initiate dedicated ranging signals for the serving BS and neighbors. The following parameters are included in this message:

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

5. MS sends dedicated ranging signals to the serving BS on the dedicated ranging opportunity.

6. Serving BS measures Timing Adjustment t1

7. MS sends dedicated ranging signal to the neighbor BS on the dedicated ranging opportunity.

8. Neighbor BS measures Timing Adjustment t2

9. Neighbor BS returns t2 to serving BS

10. Serving BS calculates the U-TDOA

△t=t1-t2

Figure NNN Operation description on AAI for Special U-TDOA

1. Serving BS and neighbor BS negotiate the allocation of a dedicated ranging region for the MS.

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

2. AAI_LBS-IND message is sent to the MS to request the MS to initiate dedicated ranging signals for the serving BS and neighbors. The following parameters are included in this message:

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

3. MS sends dedicated ranging signal to the serving BS on the dedicated ranging opportunity. At the same time, the neighbor BS shall make no allocations in that dedicated ranging region, and the neighbor BS will listen to the dedicated ranging signal from the MS.

4. Serving BS measures Timing Adjustment t1

5. Neighbor BS listens to the dedicated ranging signal from the MS, and then measures Timing Adjustment t2

6. Neighbor BS returns t2 to serving BS

7. Serving BS calculates the U-TDOA

△t=t1-t2

-----------------------------------------  Text End ---------------------------------------------------------------