| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup on AAI_RNG-ACK (16.2.3.3) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Yang Liu, Feng Xie ZTE Corporation | E-mail: { liu.yang8, xie.feng2,}@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16. 2.3.3” |  |
| Abstract | This contribution proposes to delete unicast indication attribute in AAI_RNG-ACK for clean up. |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup on AAI_RNG-ACK (16.2.3.3)

Yang Liu, Feng Xie,

ZTE Corporation

# 1. Introduction

# Since a unicast AAI_RNG-ACK can be identified by DL basic assignment A MAP IE, the unicast indication and related text should be removed from table 681.

# 2. Text Proposal

Revise table 681 at P.85 as follows.

============================== Start of proposed text ===============

Table 681 — AAI_RNG-ACK message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Unicast indication |  | 1 | 0b0: for broadcast 0b1: for unicast | N.A. |
| O | Number of frame identifiers (N_Frames) |  | 3 | The number of frame identifiers included in this AAI_RNG-ACK message | It shall be included only when the AAI_RNG-ACK message is a broadcast message. the unicast indication is 0b0. |
| O | Frame identifier |  | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe (ranging from 0b00 to 0b11). | It shall be included only when the AAI_RNG-ACK message is a broadcast message. the unicast indication is 0b0. |
| O | RNG-ACK Bitmap |  | 4 | Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. | It shall be included only when the AAI_RNG-ACK message is a broadcast message. the unicast indication is 0b0. |
| O | Number of received codes |  | 6 | The number of ranging preamble code indices detected in this corresponding ranging opportunity. | It shall be included only when the AAI_RNG-ACK message is a broadcast message. the unicast indication is 0b0. |
| … | … |  | … | … | … |
| O | Ranging responses [0..32] | Ranging Status | 2 | Used to indicate whether ranging preamble code is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue | It shall be included for each received ranging preamble code index. Or it shall be included for the specific user when the AAI_RNG-ACK message is a unicast message.Unicast indication =1 |
|  | Ranging response [0..32] | Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | It shall be included for each received ranging preamble code index when Ranging status is 0b00 or 0b10.Or it shall be included for the specific user when the AAI_RNG-ACK message is a unicast message Unicast indication =0b1 and Ranging status is 0b00 or 0b10. |
|  | Ranging responses [0..32] | Timing offset adjustment | 5 | Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs | It shall be included for each received ranging preamble code index when API bit #0 = 0b1 Or it shall be included for the specific user when the AAI_RNG-ACK message is a unicast message Unicast indication =0b1 and API bit #0 = 0b1 |
|  | Ranging responses [0..32] | Power level adjustment | 4 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the MS shall apply to its current transmission power for initial ranging. The bit size represents power level correction in the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. | It shall be included for the specific user when RNG-ACK bitmap=1 and number of received codes > 0 and and API bit #1 = 1and Ranging Status = 0b10. |
|  | Ranging responses [0..32] | Power level adjustment |  | Power level adjustment | It shall be included for the specific user when the AAI_RNG-ACK message is a unicast message Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #1 = 1 Or it shall be included for the specific user when the AAI_RNG-ACK message is a unicast message Unicast indication =1 and API bit #1 = 1 |
|  | Ranging responses [0..32] | Frequency offset adjustment | 9 | Frequency offset adjustment Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (f ) multiplied by the 9-bit number interpreted as a signed integer | It shall be included for the specific user when the AAI_RNG-ACK message is a unicast message Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #2 = 1 |

============================== End of proposed text ===============