| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Adaptive CP length to avoid Inter-symbol Interference in Macro diversity SFN |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Perumal Ramesh Babu, Youn Ok Park, Eunkyung Kim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: {ramesh.babu, parkyo, ekkim, csyoon}@etri.re.kr |
| Re: | Proposed text changes to P802.16m/D6 |  |
| Abstract | To avoid ISI in SFN |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Adaptive CP length to avoid Inter-symbol Interference in Macro diversity SFN

Perumal Ramesh Babu, Youn Ok Park, Eunkyung Kim, Chulsik Yoon

ETRI

# Introduction

In the Macro diversity case, consider BS1, BS2,…, BSn and MS. The distance between BS1 and MS is d1, distance between BS2 and MS is d2, distance between BSn and MS is dn. Let Dmax be maximum of (d1, d2,…,dn). And Dmin be minimum of (d1, d2,…, dn). To avoid inter symbol interference at the receiver,

Guard time should be proportional to ((Dmax – Dmin)/c). This means for a given geometry of the cells, the CP length should be adapted.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16.9.x. During cell planning, the maximum distance (which corresponds to a maximum delay) between any two base station in a given SFN should be calculated. The CP length to be used for this SFN by the transmitters should be fixed based on this maximum delay.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]