| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS Carrier Assignment (16.2.8.2.3.2 & 16.2.8.11.1) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D6,” in response to the IEEE 802.16 Working Group Sponsor Ballot |  |
| Abstract | E-MBS Carrier Assignment on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Carrier Assignment (16.2.8.2.3.2 & 16.2.8.11.1)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

This contribution provides clarification on the E-MBS Connection Establishment in IEEE 802.16m Amendment Draft Standard [1].

In the current IEEE 802.16m Amendment Draft Standard [1], physical carrier index shall be included in the AAI_DSA-REQ message when an ABS commences E-MBS service flow, which means E-MBS carrier may be assigned via AAI-DSx procedure. Furthermore, in the view of multicarrier operation (see Figure 418 in [1]), any AMS does not perform any MAC or PHY processing on an assigned carrier until such carrier is activated per ABS’s direction. Therefore, the carrier needs to be activated using MC-Specific messages in order to perform E-MBS operation in the particular carrier (i.e., E-MBS carrier).

However, any AMS, which has obtained E-MBS parameters during DSx procedure, may start receiving the E-MBS service without activating the E-MBS carrier.

In this contribution, we clarify the E-MBS operation in terms of E-MBS carrier assignment and activation as follows.

- Any assignment or activation for E-MBS carrier is not performed using any MC-specific message to distinguish typical MC operation with E-MBS operation.

- Any activation of secondary carrier (i.e., non E-MBS carrier) is performed using MC-specific message even though the carrier to activate has already been activated for E-MBS operation.

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

[Remedy1: Add the following text in the end of 16.2.8.2.3.2, line 11 in page 308 of 802.16m/D6]

To support E-MBS for an AMS, any carriers (i.e., mixed or dedicated carrier) used for E-MBS service are assigned using AAI_DSA described in 16.2.3.46.

[Remedy2: Add the following text in the end of 16.2.8.11.1, line 4 in page 318 of 802.16m/D6]

A carrier assigned for E-MBS is activated without AAI_CM-CMD/IND transaction after successful AAI_DSA procedure. If a carrier assigned for E-MBS is requested to be activated for non E-MBS traffic, then the carrier is activated through AAI_CM-CMD/IND transaction.

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” May 2010