| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS scanning of neighbor Femto ABSs (16.4.8.1.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Ying Li, Zhouyue Pi, Anshuman Nigam, Jung Je Son Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D6 |  |
| Abstract | This contribution is to propose clarification on AMS scanning. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# AMS Scanning of Neighbor Femto ABSs (16.4.8.1.2)

Ying Li, Zhouyue Pi, Anshuman Nigam, Jung Je Son

Samsung Electronics

# Introduction

- In the current draft D6, it describes some methods on how AMS performs cell detection, and how AMS scans its neighbor femto ABSs. However, some of these methods are not clearly described, or some of them could be further optimized, and some corresponding messages are not complete. Therefore, this contribution proposes some clarifications of the cell detection and AMS scanning.

# Proposed Text

Revise sections (section 16.4.8.1.2 and related messages modification) in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

[Modify texts as suggested below: Page 796, Line 9]

AMS may include information detected or based on whitelist, e.g., the FA, preambles, BSID, CSGIDs of the subscribed CSG femto BSs, location information, in the AAI_NBR-REQ message.

[Modify texts as suggested below: Page 796, Line 10]

In AAI_SCN-REQ, the AMS can indicate by setting ‘In whitelist indication’ being ‘1’ to indicate the detected Femto ABS is in its whitelist, ‘0’ to indicate other scenarios. It gives the ABS more information that AMS has detected.

When the serving BS receives AAI_NBR-REQ, AAI_SCN-REP/ AAI_SCN-REQ with the indicator of ‘NBR request” being ‘1’, it may respond with AAI_SCN-RSP with ‘response for the neighbor request’ indication bit as ‘1’.

If the indicator responding for the neighbor request is ‘1’, in AAI_SCN-RSP, it can contain both the optimized neighbor list and some scanning interval recommendations for some of the ABSs in the optimized neighbor list. If the indicator is ‘0’, it is a normal AAI_SCN-RSP.

16.2.3.13 AAI_SCN-REQ

Table 685—Parameters for AAI_SCN-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message Scan purpose bit map (2-bit): bit # 1: being ‘1’ indicating for BS in NBR list; bit#2: being ‘1’ indicating for BS not in NBR list. | If purpose bit #1 is ‘1’, including the fields related to the scanning based on NBR list If purpose bit #2 is ‘1’, including the fields related to the scanning not based on NBR list |
| Neighbor list request indication | ‘1’ requesting neighbor list ‘0’ not requesting neighbor list |  |
| Interleaving Interval. | The period of AMS's normal operation which is interleaved between Scan Durations.. |  |
| … | … | … |
| If scan purpose bit #1=’1’, |  |  |
| { |  |  |
| list of ABSs based on the AAI_NBR-ADV |  |  |
| … | … | … |
| } |  |  |
| If scan purpose bit #2=’1’, |  |  |
| { |  |  |
| In whitelist indication | ‘1’ ABS in AMS’s local whitelist ‘0’ others |  |
| Number of ABSs not on the AAI_NBR-ADV, but in the AMS’s local whitelist |  | Include when ‘In Whitelist indication’ is ‘1’ |
| List of ABSs not on the AAI_NBR-ADV, but in the AMS’s local whitelist |  | Include when ‘In Whitelist indication’ is ‘1’ |
| List of Other ABSs not on the AAI_NBR_ADV |  |  |
| MS Location information indicator | ‘1’ MS GPS information available ‘0’ MS GPS information not available | Include when Neighbor list request indication is ‘1’ |
| MS location information | Location information: Latitude Longitude Or Other coordinate system MS Speed? | Include when Neighbor list request indication is ‘1’ and MS location information indicator is ‘1’ |
| … | … | … |
| } |  |  |

16.2.3.14 AAI_SCN-RSP

Table 686—Parameters for AAI_SCN-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message Scan purpose bit map (2-bit): bit # 1: being ‘1’ indicating for BS in NBR list; bit#2: being ‘1’ indicating for BS not in NBR list. |  |
| Respond to neighbor list request indication | ‘1’ respond to neighbor request (AAI_NBR-REQ, or AAI_SCN-REP with neighbor list request indication as ‘1’, or AAI_SCN-REQ with neighbor list request indication as ‘1’) ‘0’ not a response to neighbor list request |  |
| … | … | … |
| If scan purpose bit #1=’1’, list of ABSs based on the AAI_NBR-ADV |  |  |
| … | … | … |
| If scan purpose bit #2=’1’, list of ABSs not on the AAI_NBR-ADV |  |  |
| … | … | … |

16.2.3.15 AAI_SCN-REP

Table 687—Parameters for AAI_SCN-REP message

| Name | Value | Usage |
| --- | --- | --- |
| Neighbor list request indication | ‘1’ requesting neighbor list ‘0’ not requesting neighbor list |  |
| MS Location information indicator | ‘1’ MS GPS information available ‘0’ MS GPS information not available | Include when Neighbor list request indication is ‘1’ |
| MS location information | Location information: Latitude Longitude Or Other coordinate system MS Speed? | Include when Neighbor list request indication is ‘1’ and MS location information indicator is ‘1’ |
| Report mode | Action code for an AMS's scan report of its measurement 0: Event-triggered report 1: Periodic report according to Scan report period of AAI_SCN-RSP |  |
| … | … | … |

16.2.3.36 AAI_NBR-REQ

Table xxx—Parameters for AAI_NBR-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| CSGID in the whitelist at the MS |  |  |
| MS Location information indicator | ‘1’ MS GPS information available ‘0’ MS GPS information not available |  |
| MS location information | Location information: Latitude Longitude Or Other coordinate system MS Speed | Include when Neighbor list request indication is ‘1’ and MS location information indicator is ‘1’ |
| Report detected ABSs |  |  |
| … | … | … |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------