| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for multicast operation in IEEE 802.16m (16.2.28) |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | Seho Kim, Sangheon Kim, Mingxia Xu, Jaehyuk Jang, Hyunkyu Yu, Youngbo Cho, Heewon Kang Samsung Electronics Zheng Yan-Xiu, Yu-Chuan Fang ITRI Roshni Srinivasan, Yi Hsuan, Hujun Yin Intel Kelvin Chou MediaTek Inc | E-mail: seho42.kim@samsung.com |
| Re: | Sponsor Ballot Recirc #1 Section 16.2.28 Support for Multicast Service |  |
| Abstract | The contribution provides the text for multicast operation in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Multicast operation in IEEE 802.16m (16.2.28)

Seho Kim, Sangheon Kim, Mingxia Xu, Jaehyuk Jang, Hyunkyu Yu, Youngbo Cho, Heewon Kang

Samsung Electronics Co., Ltd.

Zheng Yan-Xiu, Yu-Chuan Fang

ITRI

Roshni Srinivasan, Yi Hsuan, Hujun Yin

Intel

Kelvin Chou

MediaTek Inc

- Introduction

IEEE 802.16e support both multicast service and the multicast and broadcast services which utilize MBS specific features. Even if E-MBS service is supported in IEEE 802.16m, multicast service is also required to support simple multicasting service in an ABS.

The Multicast service supported in single ABS can be utilized in transmitting a message to multiple users belonged to a group. Multicast transmission for group consumes less traffic resources than unicast transmission for each user in a group. Multicast service can also be used in group call. In communication network for public protection and disaster relief, group call is considered as a key feature to send an order to multiple users and communicate with users in emergency circumstance. Since more than thousands of users can be joined in group call, unicast transmission for group call traffic seems to be inefficient and sometimes impossible in 16m systems.

In IEEE 16m, E-MBS can also support multicast service. E-MBS operates in E-MBS region that consists of subbands. In E-MBS region, E-MBS MAP and E-MBS bursts are transmitted based on E-MBS scheduling interval (MSI). The parameters of the E-MBS region and the burst size of the E-MBS MAP are transmitted in the AAI_E-MBS-CFG MAC control message which is advertised in every 16 superframes. MSI is updated by AAI_SCD message. Since E-MBS MAP is transmitted in every MSI to assign multicast traffics, E-MBS seems to be optimized to transmit periodic multicast traffic.

In case of non-periodic multicast traffic, it is hard to estimate the amount of traffic to be scheduled in frames. If non-periodic multicast traffics are mainly handled by E-MBS, there will be some variation in the amount of traffic transmitted in every frame so that E-MBS region should be decided large enough to accommodate the time-varying non-periodic multicast traffics. Although there is no non-periodic multicast traffic to send in DL, E-MAP MAP should be transmitted in every MSI, which wastes resource. In order to utilize resource efficiently in E-MBS region, E-MBS region and MSI can be changed by AAI_E-MBS-CFG MAC control message and AAI_SCD message, respectively, but it takes longer than 16 superframes to update the parameters relevant to E-MBS. The update time of E-MBS parameters is too long to chase the variation of non-periodic multicast traffics. Therefore, E-MBS seems to be inefficient to transmit non-periodic multicast traffics and more efficient transmission scheme to support non-periodic multicast traffics is required.

This contribution provides the description of multicast transmission operation for non-periodic multicast traffics in IEEE 802.16m. The following terms are added and modified to support multicast service in IEEE 802.16m.

- Multicast Service section is added to define the general Multicast operation, Multicast connection establishment, and multicast operation in connected state

- AAI_RNG_RSP messages are modified to update Multicast Group IDs and FIDs in handover to the target ABS.

- AAI_DSA-REQ/RSP messages are modified to assign Multicast Group IDs.

- AAI_DSC-REQ message are modified to change Multicast Group IDs.

- Multicast Assignment A-MAP IE is modified to add multicast assignment information.

- Multicast Identifier is added to describe ID of group that receives multicast burst.

- In order to reduce the burden in AMS side, ABS is restricted to assign 1 multicast burst (using Multicast Assignment A-MAP IE) per AAI subframe.

- Proposed Text Changes

Removed text

Added text

[Remedy #1, Insert the following text as a new section in page 435, line 20 as follows,]

16.2.28 Support for Multicast Service

Multicast Service provides an efficient method for concurrent transport of DL data common to a group of users, using a common Multicast Group ID and an FID in an ABS. Multicast service is associated with an ABS and is offered in the downlink only. Each multicast connection is associated with a service flow provisioned with the QoS and traffic parameters for that service flow. Service flows to carry multicast data are instantiated on individual AMSs participating in the service while in normal operation. During such instantiation, the AMS learns the parameters that identify the service and associated service flows.

The ABS shall use a combination of Multicast Group ID and FID to provide the multicast service. The Multicast Group ID is the ID of a group that receives multicast burst and is unique to an ABS. Multicast Group ID is assigned by AAI_DSA-REQ/RSP message and changed by AAI_DSC_REQ message.

16.2.28.1 Multicast Operation

An ABS may establish a DL multicast service by creating a multicast connection with each AMS to be associated with the service. Any available FID may be used for the multicast service (i.e., there are no dedicated FIDs for multicast transport connections). To ensure proper multicast operation, the FID used for the service shall be the same for all AMSs on the same channel that participate in the connection. The multicast connection shall be established using a combination of Multicast Group ID and FID assigned through AAI_DSA MAC control message. Since a multicast connection is associated with a service flow, it is associated with the QoS and traffic parameters of that service flow. For multicast connections, ARQ is not applicable, and the Null SAID is used as the target SAID field in DSA-REQ/RSP message.

16.2.28.2 Multicast Connection Establishment

When an AMS registers to receive multicast services, the serving ABS or the AMS may initiate the DSA procedure for multicast connections. The AMS’s discovery and registration of multicast services with the ABS through upper layer signaling are outside the scope of this standard.

The AAI_DSA, AAI_DSC and AAI_DSD messages are used to establish, change, and delete multicast service flows respectively. The ABS shall send the AAI_DSA-REQ/RSP to the AMS with the relevant multicast parameters including Multicast Group ID.

16.2.28.3 Multicast Operation in Connected State

When an AMS performs handover to a new ABS while in Active Mode or Sleep Mode, the AMS shall send AAI_RNG-REQ message with Ranging Purpose Indication = 0b0000 at the target ABS. Upon reception of the AAI_RNG-REQ message from the AMS, the ABS shall include Multicast Group ID in the AAI_RNG-RSP parameters to provide an updated Multicast Group ID, if any.

[Remedy #2, Insert the following text in Table 681, page 82, line 34, section 16.2.3.2 as follows,]

Table 681 – AAI_RNG-RSP message Field Descriptions

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the target ABS | Presented only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to be updated. |
| O | Current_Multicast Group ID and FID and New_Multicast Group ID and FID | Variable (32xM) | Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Present only if Num of Multicast Group ID and FID to be updated > 0 |

[Remedy #3, Insert the following text after line 65, page 188, section 16.2.3.46.1 as follows,]

When an ABS commences multicast service, the following parameters shall be included in the AAI_DSA-REQ message.

- Multicast Service: Indicates whether the multicast service is being requested or provided for the connection that is being setup.

- Multicast Group ID for the connection that is associated with the service flow in DSA-REQ.

[Remedy #4, Insert the following text in Table 740, page 199, line 51, section 16.2.3.46.1 as follows,]

Table 740 – AAI_DSA-REQ

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast SFID | 4 | Number of Multicast service flow to add | Present when Num of Multicast SFID > 0 |
| M | Multicast Group ID | 12 | ID of a group to which the flow is added | Present when Num of Multicast SFID > 0 |
| M | FID | 4 | FID of service flow created | Present when Num of Multicast SFID > 0 |

[Remedy #5, Insert the following text after line 51, page 200, section 16.2.3.46.2 as follows,]

When an AMS commences multicast service, the ABS shall include the following parameters in the AAI_DSA-RSP message.

- Multicast Service: Indicates whether the multicast service is being requested or provided for the connection that is being setup.

- Multicast Group ID for the connection that is associated with the service flow in DSA-RSP.

[Remedy #6, Insert the following text in Table 741, page 201, line 56, section 16.2.3.46.2 as follows,]

Table 741 – AAI_DSA-RSP

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast Group ID | 4 | Number of Multicast Group ID to add | Present when Num of Multicast Group ID > 0 |
| M | Multicast Group ID | 12 | ID of a group to which the flow is added | Present when Num of Multicast Group ID > 0 |

[Remedy #7, Insert the following text after line 65, page 202, section 16.2.3.46.3 as follows,]

When an ABS updates multicast service flow, the following parameters may be included in the AAI_DSC-REQ message.

- Multicast Service Flow Parameter: Mapping update of Multicast Group ID

[Remedy #8, Insert the following text in Table 743, page 210, line 39, section 16.2.3.46.3 as follows,]

Table 743 – AAI_DSC-REQ

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast Group ID to add | 4 | Number of Multicast Group IDs to add | Present when ABS initiates AAI_DSC-REQ Present only if Num of Multicast Group ID to be added > 0 |
| M | Multicast Group ID to be added | 12 | Multicast Group ID to be added | Present only if Num of Multicast Group ID to be added > 0 |
| O | Num of Multicast Group ID to be deleted | 4 | Number of Multicast Group IDs to be delete | Present when ABS initiates AAI_DSC-REQ Present only if Num of Multicast Group ID to be deleted > 0 |
| M | Multicast Group ID to be deleted | 12 | Multicast Group ID to be deleted | Present only if Num of Multicast Group ID to be deleted > 0 |

[Remedy #9, Modify the text in Table 862, page 617, section 16.3.5.5.2.4.13 as follows,]

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast or multicast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 865 describes the fields in a broadcast assignment A-MAP IE.

Table 862 – Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 12 | 00b00: This IE carries broadcast assignment information 10b01: This IE carriers ranging channel allocation information 0b10: This IE carriers multicast assignment information 0b11: reserved |
| If(Function Index == 00b00){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 1413 | Reserved bits |
| }else{ |  |  |
| Reserved | 1615 | Reserved bits |
| } |  |  |
| }else if{Function Index == 10b01}{ |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 2 | Opportunity index of the ranging channel |
| } |  |  |
| Reserved | 1110 | Reserved bits |
| } else if{Function Index == 0b10}{ |  |  |
| Multicast Group ID | 12 | ID of a group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 4 | Reserved bits |
| } |  |  |
| } |  |  |

* A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 846. If Function index == 0b00 or 0b01, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b001. If Function index == 0b10, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b010.

Multicast Group ID is the ID of a group that receives multicast burst and is unique to an ABS. Multicast Group ID is assigned by AAI_DSA-REQ/RSP message and changed by AAI_DSC_REQ message.

[Remedy #10, Add the following sentence before Table 846, page 567, section 16.3.5.5.2.4 as follows,]

The 16-bit masked CRC is constructed using a 1 bit masking prefix, a 3 bit message type indicator, and 12 bit Masking Code as described in Table 846.

[Remedy #11, Replace the Table 846, page 567, section 16.3.5.5.2.4 as follows,]

Table 846 – Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID |
|  | 0b001 | Refer to table 847 |
|  | 0b010 | 12 bit Masking Code (0xFFF in Hex) |
| 0b1 | 15 bit RAID: The RAID is derived from the AMS' random access attributes (i.e., superframe number (LSB 5bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)) as defined below: RAID = (LSB 5bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index) |  |

[Remedy #12, Insert the following text after line 46, page 43 as follows,]

16.2.1.2.6 Multicast Group Identifier

A 12-bit value that is used to uniquely indentify a multicast flow in the domain of the ABS.

[Remedy #13, modify the following text in page 442 as follows,]

ABS shall not allocate more than one broadcast data burst with time domain repetition per frame and 1, more than one broadcast data burst without time domain repetition per AAI subframe (using Broadcast Assignment A-MAP IE with the field "Transmission Format" indicating with or without time domain repetition) and 1, more than one E-MBS burst (using E-MBS A-MAP IE) per AAI subframe, and more than one multicast burst (using Broadcast Assignment A-MAP IE with Function Index = 0b10) per AAI subframe. Here, a long TTI burst shall be counted as one burst for each and every AAI subframe that the long TTI burst spans.