| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for adding Updating Capability of Power control Offset (16.3.8.4) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jeongho Park Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D7 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D8 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for adding Updating Capability of Power control Offset (16.3.8.4)

Jeongho Park

Samsung Electronics

- References

[1] IEEE P802.16m/D7

- Introduction

This contribution includes text proposal for adding another feature to deal with power adjustment message.

- Scope of Suggested Remedies

A. Adding ‘updating’ feature for Offset parameter (OffsetControl and OffsetData)

Currently Offset values in uplink power control (OffsetControl and OffsetData) are only possible to replace by newly received message of AAI_UL_POWER_ADJ.

However, if it is possible to update (increase) Offset values instead of replacing, it would be helpful for BS’s management of MS’s power.

For example, tracking all MS’s offsetControl values cannot be guaranteed by BS because PC-A-MAP IE transmission error rate is not zero. In this situation, BS might to be needy to update the value based on the received SINR. Then just updating instead of replacing would be appropriate.

- Proposed Text

Remedy #1: page 164, Modify Table 716 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

16.2.3.32 AAI_UL_POWER_ADJ message

An AAI_UL_POWER_ADJ message shall be transmitted by the ABS to control the transmit power level of AMS.

Table 716—AAI_UL_POWER_ADJ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | offsetData | 6 | offsetData is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | N.A. |
| M | offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | N.A. |
| M | offsetApplication | 1 | If 0b0, offsetData and offsetControl replace the previous value If 0b1, offsetData and offsetControl are added to the previous value | N.A. |
| O | channelIndex | 8 | Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between channel index and the AMS | Present when ABS want to change FF Channel Period by using AAI-POWER-ADJ message. (More details can be found in 16.3.6.5.2.4.5.) |
|  | shortTermFeedbackPeriod | 3 | Short-term feedback is transmitted on the FBCH every 2p frames, where p is the value specified by shortTermFeedbackPeriod |  |
|  | longTermFeedbackPeriod | 2 | Long-term feedback is transmitted on the FBCH every 2q short-term feedback reports are transmitted, where q is the value specified by longTermFeedbackPeriod. If q = 0b00, long-term feedback is not used |  |
|  | frameOffsetNumber | 2 | The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame number. If the current frame is specified, the AMS starts reporting in eight frames |  |
|  | subframeIndex | 3 | Indicates the UL AAI subframe index in the UL portion of the frame |  |
|  | allocationDuration | 3 | An FBCH is transmitted on the FBCH channels indexed by Channel Index for frames, where d is the value given by allocationDuration. If d = 0b000, the FBCH is deallocated. If d = 0b111, the AM reports until the ABS command for the AMS to stop |  |

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #2: page 756, Modify line 12~20 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

For a data channel transmission, the Offset in Equation (291) shall be set to the value OffsetData conveyed in an AAI_UL_POWER_ADJ message. The value used shall be taken from the most recent message which preceded the first uplink subframe of the frame by more than TDL_Rx_Processing. An AMS shall set initial OffsetData as OffsetInitial after the AMS succeeds for initial ranging process. Here OffsetInitial is calculated by an AMS during initial ranging process according to 16.3.8.4.4. When AMS receives AAI_UL_POWER_ADJUST message at i-th frame, the AMS’s OffsetData shall be replaced by the value indicated by the message from the (i+1)th frame if offsetApplication is 0b00, while OffsetData shall be increased by the value indicated by the message from the (i+1)th frame if offsetApplication is 0b01.

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #3: page 757, Modify line 43~47 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

When If AMS receives AAI_UL_POWER_ADJUST message at i-th frame, the AMS’s OffsetControl shall be replaced by the value indicated by the message from the (i+1)th frame if offsetApplication is 0b00, while OffsetControl shall be increased by the value indicated by the message from the (i+1)th frame if offsetApplication is 0b01. Also PC-A-MAP IE from (i+1)th frame is effective on this new value.

-------------------------------------------------Text Change END -----------------------------------------------------