| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Device class and related capability negotiation parameters during Network Entry/Re-entry Sections: 16.2.15, 16.2.3 |  |
| Date Submitted | 2010-07-05 |  |
| Source(s) | Shaocheng Wang Sassan Ahmadi Muthaiah Venkatachalam Xiangying Yang Intel Corporation | E-mail: shaocheng.wang@intel.com; sassan.ahmadi@intel.com; muthaiah.venkatachalam@intel.com; xiangying.yang@intel.com; |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “sections 16.2.15 and 16.2.3” |  |
| Abstract | This contribution proposes a new structure for capability negotiation parameters during network entry/re-entry to minimize the use of radio resources for transmission of unnecessary parameters that otherwise must be supported by default. We further categorize the parameters to be negotiated based on pre-authentication or post-authentication to ensure encryption and protection of those parameters that are directly pertained to the user or device identity and credentials. |  |
| Purpose | Adopt the proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Device class and related capability negotiation parameters during Network Entry/Re-entry

Shaocheng Wang, Sassan Ahmadi, Muthaiah Venkatachalam, Xiangying Yang

Intel Corporation

- Introduction

Since the large SBC-REQ/RSP MAC management message was a bottleneck in the reliability and accessibility of the legacy system, the goal of this proposal is to organize and reduce the number of configuration parameters that are negotiated between the base station and the mobile station in order to reduce unicast messaging overhead in the system.

WiMAX Forum Technical Working Group has recently initiated an effort to define device classes for release 2 of mobile WiMAX profiles. Depending on maximum downlink/uplink data rate (maximum encoding and decoding capability), MIMO and multi-carrier capabilities of the mobile station, different device classes are defined, where each device class targets a specific market or consumer hardware platform. While user equipment category concept has been around in cellular industry for some years, the concept of device class is new to mobile WiMAX and will affect the configuration parameters that are negotiated during network entry or re-entry. On the other hand, unlike its predecessor, IEEE 802.16m specifies several mandatory functionalities and default parameters that do not need to be negotiated and are required to be supported in all device classes.

A device class represents the hardware limitations of a client device implementation. The device class shall not be confused with the capability class concept defined in the standard. A capability class is defined as a unique set of functions, configuration parameters, air-interface protocol revision, and/or services that can uniquely describe a mobile station implementation or configuration while operating in the network. Each capability class is indicated by its corresponding CAPABILITY_INDEX. The AMS, by default, shall support the basic capabilities associated with Capability Class 0 (i.e., CAPABILITY_INDEX = 0).

In addition to the parameters that are set by the device class and capability class, there are additional parameters and configurations that are signaled via A-MAP IE and are not negotiable. The following is an example of device class mapping in Reference [1].

Table 1. Device Categorization from the perspective of User Peak Throughput (TDD)

| Device Category TDD | Total BW (MHz) | Aggregation | No of streams [Rx, Tx] | UL 64QAM | Max bits per subframe per device (See Assumption) |  | Peak System Throughput Mbps |  | User buffer size/max number of soft bits |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DL | UL | DL | UL | DL | UL |
| T1 | 20 | 20 | [2, 1] | No | 123648 | 43792 | 106.91 | 23.50 |  |  |
| T2 | 20 | 20 or (10+10) | [2, 2] | No | 123648 | 82432 | 106.91 | 44.24 |  |  |
| T3 | 20 | 20 | [2, 2] | No | 123648 | 82432 | 106.91 | 44.24 |  |  |
| T4 | 20 | 20 | [2, 2] | No | 123648 | 82432 | 106.91 | 44.24 |  |  |
| T5 | 20 | 20 or (10+10) | [4, 2] | Yes | 242880 | 123648 | 203.67 | 66.36 |  |  |
| T6 | 20 | 20 or (10+10) | [4, 2] | Yes | 242880 | 123648 | 203.67 | 66.36 |  |  |
| T7 | 20 | 20 | [4, 4] | Yes | 242880 | 242880 | 203.67 | 127.18 |  |  |
| T8 | 40 | 20+20 | [4, 4] | Yes | 242880x2 | 242880x2 | 203.67x2 | 127.18x2 |  |  |

Table 2. Device Categorization from the perspective of User Peak Throughput (FDD)

| Device Category FDD | Total BW | No of streams [Rx, Tx] | UL 64QAM | Max bits per subframe per device (See Assumption) |  | Peak System Throughput Mbps |  | User buffer size/max number of soft bits |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DL | UL | DL | UL | DL | UL |
| F1 | 2x20 MHz | [2, 1] | No |  |  | 168.66 | 62.67 |  |  |
| F2 | 2x20 MHz | [2, 2] | No |  |  | 168.66 | 117.96 |  |  |
| F3 | 2x20 MHz | [2, 2] | No |  |  | 168.66 | 117.96 |  |  |
| F4 | 2x20 MHz | [2, 2] | Yes |  |  | 168.66 | 176.95 |  |  |
| F5 | 2x20 MHz | [4, 4] | Yes |  |  | 322.56 | 339.15 |  |  |

As a result, the following is an example list of parameters that are specified once the device class of the mobile station is identified or are specified via A-MAP IE control signaling. This type of parameters shall be excluded from the negotiation list.

- Uplink modulation scheme (part of device class definition)

- # of RX/TX antenna (part of device class definition)

- Text Proposal

[Remedy #1]

--------------------------------Start of the proposed text-----------------------------------------

[Note to Editor] In section 16.2.3.4 under AAI_SBC-REQ modify Table 683 and 684 with following table (entries that are crossed out are existing parameters that we suggest to remove; entries in blue text are new parameters we suggest to add)

Table 683—AMS Capabilities to be transmitted to ABS

| Nego parameters | value |
| --- | --- |
| Modulation Scheme | If Bit#0 =1, DL 64 QAM supports If Bit#1 =1, UL 64 QAM supports |

Table 684—AAI_SBC-RSP Field Descriptions

Table 685—AMS Capabilities to be confirmed by ABS

| Nego parameters | value |
| --- | --- |
| Modulation Scheme | If Bit#0 =1, DL 64 QAM supports If Bit#1 =1, UL 64 QAM supports |

--------------------------------End of the proposed text-----------------------------------------