| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-NBR-ADV (16.2.3.13) |  |
| Date Submitted | 2010-9-8 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-NBR-ADV (16.2.3.13) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-NBR-ADV (16.2.3.13)

Kelvin Chou

MediaTek Inc.

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Since AAI-NBR-REQ has been removed and its functionality is replaced by AAI-SCN-REP, we also proposed to remove the ASN.1 code for this message from the Annex.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 126, line 57, replace Table 692 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 692—AAI-NBR-ADV parameters

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Change Count | 3 | AAI-NBR-ADV Change Count |  |
| Total number of cell types | 3 | The total number of cell types |  |
| Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101: R1 BS or LZone of ABS 0b110-0b111: reserved |  |
| Total Number of AAI-NBR-ADV Segments | 4 | Total number of segments of AAI-NBR-ADV for this cell type |  |
| AAI-NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type |  |
| Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI-NBR-ADV segment. If this is the first AAI-NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI-NBR-ADV segment has one Index which corresponds to the first ABS in that AAI-NBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index. |  |
| For (i=0; i<N-NBR-ABSs; i++) { |  | N-NBR-ABSs is the number of neighbor ABSs included in this message |  |
| BSID | 48 | Neighbor ABS ID |  |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with IEEE 802.16-2009 definition, with new MAC protocol version 10 defined for AAI. |  |
| CP time | 2 | CP time of the BS 0b00: 1/8 0b01: 1/16 0b10: 1/4 |  |
| For(j=0; j<N-Carrier-Info; j++) { | - | N-Carrier-Info is the number of carrier information listed here for the ABSi |  |
| SA-PREAMBLE index | 10 |  |  |
| A-PREAMBLE transmit power | 8 |  |  |
| Physical carrier index | 6 | Refer to the physical carrier index in AAI-Global-CFG message |  |
| Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in ABS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| PGID | 16 | Indicates paging group identifier where the carrier belongs |  |
| S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS |  |
| SFH encoding format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier which transmits this AAI-NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell ABS, the SFH encoding format shall be either 0b00 or 0b01 or 0b11. |  |
| If (SFH encoding format = 0b00) { | - |  |  |
| SFH Subpkt 1 | var | Depending on FFT size |  |
| SFH Subpkt 2 | var | Depending on FFT size |  |
| SFH Subpkt 3 | var |  |  |
| }else if (SFH encoding format = 0b01¦0b11) { | - |  |  |
| Delta information | var | Indicates the delta encoding, between the reference carrier and the current carrier where this message is transmitted if SFH encoding format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format=0b11. Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 836, Table 837, and table 838, respectively. Each delta information attribute is optional. |  |
| } | - |  |  |
| } | - |  |  |
| Neighbor-specific trigger | Var | Optional neighbor-specific triggers with encoding defined in Table 770—Trigger description | Optional |
| }//end of for N-NBR-ABSs |  |  |  |
| For (i=0; i<N-NBR-R1-BSs; i++) { | - | N-NBR-R1-BSs is the number of neighbor Release 1 BSs | Shall be presented if R1 NBR BS list is available |
| Neighbor BSID | 48 | Neighbor Base Station ID |  |
| Preamble index/Subchannel Index | 8 | Defines the PHY-specific preamble for each neighbor R1 BS. Bit 7: Indicates the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. Bit 6 to bit 0: Represent the preamble index of the neighbor R1 BS. |  |
| PHY mode ID | 16 | Specifies the PHY parameters of each neighbor R1 BS, including channel bandwidth, FFT size, cyclic prefix, and frame duration, defined in 11. 18. 2 |  |
| Channel bandwidth | 2 | Channel bandwidth of the neighbor R1 BS. R1 BS uses 5 msec frame duration, 1/8 CP 0b00: 5 MHz 0b01: 7 MHz 0b10: 8.75 MHz 0b11: 10MHz |  |
| R1 BS Physical Carrier Index | 6 | Carrier Index of the neighbor R1 BS | Presented when S-ABS carrier index is different from that of target R1 BS. |
| }//end of for N-NBR-R1-BSs |  |  |  |
| For(i=0; i<N-PHY-Carrier-Indices; i++) { | - | N-PHY-Carrier-Indices is the number of PHY Carrier Indices listed here 0 <= N-PHY-Carrier-Indices <= 64 | Optional. Presented only for Cell type whose system info are not included in AAI-NBR-ADV |
| PHY Carrier Index | 6 |  |  |
| } |  |  |  |
| IDCell range start | 10 |  |  |
| IDCell range end | 10 |  |  |
| NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting, in unit of frames. | Optional. Presented only if the serving ABS is a WirelessMAN-OFDMA Advanced co-existing System |
| NBR-ADV interval | 4 | The interval of MOB_NBR-ADV being sent in LZone, in unit of frames. |  |
| S-ABS LZone Preamble Index | 7 | S-ABS LZone Preamble Index for AMS to synchronize with S-ABS LZone to receive MOB_NBR-ADV |  |
| LDM Parameter - AI | 4 | The new AI for LDM | Shall be included when the LDM parameters are changed. These parameters will be broadcasted for a certain duration of time as decided by the network |
| LDM Parameter - UAI | 8 | The new UAI for LDM |  |
| LDM Parameter - SFO | 9 | The new SFO for LDM |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 921, line 12, remove the following ASN.1 code segment regarding AAI-NBR-REQ:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- Handover

aaiHoInd AAI-HO-IND,

aaiHoReq AAI-HO-REQ,

aaiHoCmd AAI-HO-CMD,

aaiNbrAdv AAI-NBR-ADV,

aaiNbrReq AAI-NBR-REQ,

aaiScnReq AAI-SCN-REQ,

aaiScnRsp AAI-SCN-RSP,

aaiScnRep AAI-SCN-REP,

--------------------------------------------------  Text End  -----------------------------------------------------------------

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D8, page 960, line 23, remove the following ASN.1 code segment regarding AAI-NBR-REQ:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Neighbor List Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-NBR-REQ ::= SEQUENCE {

lsbRequestedBSID 	INTEGER (0..4095),

requestBSType 		INTEGER {

csgFemtoABS (0),

osgFemtoABS (1)

} (0..3),

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------