| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up text of the Contention-based random access bandwidth request (D9 - 16.2.11.1.1) |  |
| Date Submitted | 2010-10-26 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | Sponsor Ballot on IEEE p802.16m D9 |  |
| Abstract | This contribution proposes clean-up text of the Contention-based random access bandwidth request. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up text of the Contention-based random access bandwidth request (D9 - 16.2.11.1.1)

Heejeong Cho, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the clean-up text for the Contention-based random access bandwidth request in the IEEE 802.16m DRAFT Amendment [1].

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[Remedy #1: Adopt the following modification in page 400, line 39, section 16.2.11.1.1]

16.2.11.1.1 Contention-based random access bandwidth request

Figure 432—Example of 5-step random access BR procedure

Otherwise, if the AMS receives a BR-ACK A-MAP IE in frame n + BR_ACK_Offset which indicates a successful reception of the BR preamble sequence, the AMS shall start a BR timer and set its expiration value to the fixed value defined in section 16.11 and Table 977. The AMS shall stop the BR timer upon receiving a UL grant for standalone BR header. After sending a standalone BR header, the BR procedure proceeds as the 5-step procedure in Figure 432, starting with step 3. Upon receiving a UL grant for standalone BR header, the AMS shall stop the BR timer and the BR procedure proceeds as the 5-step procedure in Figure 432, starting with step 3.

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

- References

[1] IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”