| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for CA-specific trigger conditions (16.2.8.2.11.1) |  |
| Date Submitted | 2010-10-26 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Kiseon Ryu LG Electronics | eunjong.lee@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D9 for Sponsor Ballot |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for CA-specific trigger conditions (16.2.8.2.11.1)

Eunjong Lee, Youngsoo Yuk and Kiseon Ryu

LG Electronics

- Introduction

The current 16m has defined the CA-specific trigger definitions for assigned secondary carriers of smaller coverage. However, according to the current description, it may lead to some unnecessary scan reporting procedure. So, we’d like to optimize the scan reporting of an AMS related to the CA-specific trigger and there are some clean-up texts.

Details are as follows:

- The scanning action of Table 771 was deleted in the last meeting. So we need to clean up the texts related to the CA-specific trigger actions and clarify the scanning action.

- If the channel quality of active carrier is bad, the AMS may need to transmit the scan report for the inactive assigned carrier(s) in order to switch to another carrier although the trigger conditions for inactive secondary carrier(s) are not met. So we propose to add one more metric in the CA-specific trigger.

- If there are one or more carriers having the same smaller coverage, these carriers would have the same CA-specific trigger definitions. In order to reduce the message overheads, therefore, we propose to define the CA-specific trigger definitions only for the lowest physical carrier index among the available carriers requiring the same trigger definitions and for the remaining carriers the ABS informs the AMS of this lowest physical carrier index (i.e., Reference Carrier in AAI_MC-ADV of proposal text) instead of the CA-specific trigger definitions.

- It seems to be inefficient that all of the inactive assigned carriers requiring the same CA-specific trigger definitions perform the reporting procedure. So, we propose that an AMS perform the reporting procedure only for the carrier corresponding to the lowest physical carrier index among them if all of the assigned carriers linked to Reference Carrier of AAI_MC-ADV are inactive. In addition, if one of the inactive assigned carriers linked to Reference Carrier is activated, the AMS doesn’t need to perform the reporting for the remaining inactive carriers and the ABS can refer to the channel quality of the activated carrier.

- References

[1] IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- Text proposal for the 802.16m DRAFT amendment

--------------------------------------------------- Start of the Text ------------------------------------------------------

[Remedy 1: Modify the texts in line 15 on page 385 as follows:]

The ABS may trigger the scanning of inactive secondary carrier(s) by transmitting AAI-SCN-RSP when deciding to activate assigned secondary carrier(s) for an AMS. The AMS receiving AAI-SCN-RSP shall perform the scanning for the inactive secondary carrier(s). The ABS may specify trigger parameters for both scanning and scan reporting of each inactive assigned secondary carrier in AAI-MC-ADV message. An AMS shall perform the scanning or scan reporting when the conditions on the triggers are met.

The ABS may define following trigger conditions for the each action:

1) Conditions that define when the AMS shall initiate scanning procedure for inactive assigned secondary carriers.

2)1) Conditions that define when the AMS shall report scanning measurement results to the ABS.

Carrier activation (CA)-specific Trigger definitions are encoded using the description in Table 770—.

[Remedy2: Modify the texts in Table 771 on page 386 as follows:]

Table 771—CA-specific Trigger; Type/Function/Action Description

| Name | Size (bits) | Value | Description |
| --- | --- | --- | --- |
| Type | 1(MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric |  |
| Function | 1 | Computation defining scanning and reporting trigger condition: 0x0: Metric of inactive assigned secondary carrier is greater than absolute value 0x1: reserved Metric of active secondary carrier is less than absolute value |  |
| Action | 1(LSB) | Action performed upon reaching trigger condition: 0b0: Respond on trigger with AAI_SCN-REP for inactive secondary carrier 0b1: reserved | Action 0b0 applies to only Function 0x0 and 0x1. |

[Remedy 3: Modify the texts in Table 749 on page 272 as follows:]

Table 749—AAI-MC-ADV Message Field Description

| Field | Size (bits) | Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Reference Carrier | 6 | Indicates the physical carrier index of the carrier that works as a reference carrier representing a group of carriers with a similar property and used for the CA specific trigger. The group of carriers represented by the Reference Carrier use the same CA-specific Trigger definitions as this reference carrier. If this value is present, CA-specific Trigger definitions for the corresponding carrier shall be omitted. | Optional Present for the carriers that belong to the carrier group represented by the Reference Carrier. Shall be omitted for the physical carrier index that is the Reference Carrier. |
| CA-specific Trigger definitions | Variable | Carrier Activation-specific triggers with encoding defined in Table 770 – CA-specific Trigger description. | Optional Present for the Reference Carrier when the corresponding carrier is required to be scanned for carrier activation. Shall be present only for the Reference Carrier. (that is, inclusion of this parameter is mutually exclusive with the parameter of Reference Carrier.) |
| PCC-specific Trigger definitions | Variable | Primary Carrier Change-specific triggers with encoding defined in Table 768 –Trigger description. | Optional Present when the corresponding carrier is required to apply different trigger condition from definitions of AAI-SCD message. |
| … |  |  |  |

[Remedy 4: Add the texts in line 19 on page 385 as follows:]

If all of the assigned secondary carriers linked to Reference Carrier are inactive, the AMS may perform the reporting procedure by the trigger conditions only for the carrier corresponding to the lowest physical carrier index among them until at least one of the assigned carriers gets activated. If any of inactive assigned carriers linked to Reference Carrier of AAI-MC-ADV is activated, the AMS may not perform the scanning/reporting procedure for the remaining inactive assigned carrier(s) and the ABS may refer to the channel quality of the activated carrier(s).

------------------------------------------------------------------End of the Text----------------------------------------------------------------