- (normative)

PCS proforma – Time-sensitive networking profile
for industrial automation

  - General

The supplier of an implementation that is claimed to conform to the profile specified in this document shall complete the corresponding Profile Conformance Statement (PCS) proforma, which is presented in a tabular format based on the format used for Protocol Implementation Conformance Statement (PICS) proformas.

The tables do not contain an exhaustive list of all requirements that are stated in the referenced standards; for example, if a row in a table asks whether the implementation is conformant to Standard X, and the answer "Yes" is chosen, then it is assumed that it is possible, for that implementation, to fill out the PCS proforma specified in Standard X to show that the implementation is conformant; however, the tables in this document only further refine those elements of conformance to Standard X where particular answers are required for the profiles specified here.

A completed PCS proforma is the PCS for the implementation in question. The PCS is a statement of which capabilities and options of the protocol have been implemented. The PCS can have several uses, including use by the following.

- Protocol implementer, as a checklist to reduce the risk of failure to conform to the document through oversight.

- Supplier and acquirer, or potential acquirer, of the implementation, as a detailed indication of the capabilities of the implementation, stated relative to the common basis for understanding provided by the standard PCS proforma.

- User, or potential user, of the implementation, as a basis for initially checking the possibility of interworking with another implementation.

NOTE While interworking can never be guaranteed, failure to interwork can often be predicted from incompatible PCS.

- Protocol tester, as the basis for selecting appropriate tests against which to assess the claim for conformance of the implementation.

- The user, to verify whether the IA-station, as described by the PCS, fulfills use-case requirements.

  - Abbreviated terms and special symbols

    - Status symbols

M	mandatory

O	optional

O.n	optional, but support of at least one of the group of options labeled by the same numeral n is required

X	prohibited

pred	conditional-item symbol, including predicate identification (see A.3.4)

¬	logical negation, applied to a conditional item's predicate

    - General abbreviated terms

N/A	not applicable

PCS	Profile Conformance Statement

  - Instructions for completing the PCS proforma

    - General structure of the PCS proforma

The first part of the PCS proforma, implementation identification and protocol summary, is to be completed as indicated with the information necessary to identify fully both the supplier and the implementation.

The main part of the PCS proforma is a fixed-format questionnaire, divided into several subclauses, each containing a number of individual items. Answers to the questionnaire items are to be provided in the rightmost column, either by simply marking an answer to indicate a restricted choice (usually Yes or No) or by entering a value or a set or range of values. There are some items where two or more choices from a set of possible answers can apply; all relevant choices are to be marked. Each item is identified by an item reference in the first column. The second column contains the question to be answered; the third column records the status of the item – whether support is mandatory, optional, or conditional (see also A.3.4). The fourth column contains the reference or references to the material that specifies the item in the main body of this document, and the fifth column provides the space for the answers.

The PCS indicates support of one of the conformance classes, ccA or ccB, per bridge and end-station component, specified in this profile.

A single IA-station can incorporate the functionality of one or more of the functions listed in this PCS. For example, an IA-station could have both an end station component and a Bridge component.

A supplier can also provide (or be required to provide) further information, categorized as either additional information (see A.3.2) or exception information (see A.3.3). When present, each kind of further information is to be provided in a further subclause of items labeled Ai or Xi, respectively, for cross-referencing purposes, where (i) is any unambiguous identification for the item (for example, simply a numeral). There are no other restrictions on its format and presentation.

A completed PCS proforma, including any Additional Information and Exception Information, is the Protocol Implementation Conformation Statement for the implementation in question.

NOTE Where an implementation is capable of being configured in more than one way, a single PCS can be used to describe all such configurations. However, the supplier has the choice of providing more than one PCS, each covering some subset of the implementation's configuration capabilities, in case that makes for easier and clearer presentation of the information.

    - Additional information

Items of Additional Information allow a supplier to provide further information intended to assist the interpretation of the PCS. It is not intended or expected that a large quantity be supplied, and a PCS can be considered complete without any such information. Examples can be an outline of the ways in which a (single) implementation can be set up to operate in a variety of environments and configurations, or information about aspects of the implementation that are outside the scope of this document but that have a bearing on the answers to some items.

References to items of Additional Information can be entered next to any answer in the questionnaire and can be included in items of Exception Information.

    - Exception information

It can occasionally happen that a supplier wishes to answer an item with mandatory status (after any conditions have been applied) in a way that conflicts with the indicated requirement. No preprinted answer will be found in the Support column for this item. Instead, the supplier shall write the missing answer into the Support column, together with an Xi reference to an item of Exception Information and shall provide the appropriate rationale in the Exception item itself.

An implementation for which an Exception item is required in this way does not conform to this document.

NOTE A possible reason for the situation described previously is that a defect in this document has been reported, a correction for which is expected to change the requirement not met by the implementation.

    - Conditional status

      - Conditional items

The PCS proforma contains a number of conditional items. These are items for which both the applicability of the item itself, and its status if it does apply (mandatory or optional) are dependent on whether certain other items are supported.

Where a group of items is subject to the same condition for applicability, a separate preliminary question about the condition appears at the head of the group, with an instruction to skip to a later point in the questionnaire if the "Not Applicable" (N/A) answer is selected. Otherwise, individual conditional items are indicated by a conditional symbol in the Status column.

A conditional symbol is of the form "pred: S" where pred is a predicate as described in A.3.4.2, and S is a status symbol, M or O.

If the value of the predicate is true (see A.3.4.2), the conditional item is applicable, and its status is indicated by the status symbol following the predicate: the answer column is to be marked in the usual way. If the value of the predicate is false, the "Not Applicable" (N/A) answer is to be marked.

      - Predicates

A predicate is one of the following:

- an item-reference for an item in the PCS proforma: the value of the predicate is true if the item is marked as supported and is false otherwise,

a predicate-name, for a predicate defined as a Boolean expression constructed by combining item-references using the Boolean operator OR: the value of the predicate is true if one or more of the items is marked as supported, or

the logical negation symbol "¬" prefixed to an item-reference or predicate-name: the value of the predicate is true if the value of the predicate formed by omitting the "¬" symbol is false, and vice versa.

Each item whose reference is used in a predicate or predicate definition, or in a preliminary question for grouped conditional items, is indicated by an asterisk in the Item column.

  - Common requirements

Instructions

One instance of Clause A.4 shall be filled out per IA-station.

    - Implementation identification

The entire PCS proforma is a form that shall be filled out by a supplier according to Table A.1.

Table A.1 – Implementation identification template

| Supplier |  |
| --- | --- |
| Contact point for queries about the PCS |  |
| Implementation Name(s) and Version(s) |  |
| Other information necessary for full identification, for example, name(s) and version(s) of machines and operating system names |  |

Only the first three items are required for all implementations; other information can be completed as appropriate in meeting the requirement for full identification. The terms "Name" and "Version" should be interpreted appropriately to correspond with a supplier's terminology (for example, Type, Series, Model).

    - Profile summary

Table A.2 shows the profile summary template.

Table A.2 – Profile summary template

| Identification of profile specification | IEC/IEEE 60802 – Time-Sensitive Networking profile for industrial automation |  |  |  |
| --- | --- | --- | --- | --- |
| Identification of amendments (Amd) and corrigenda (Corr) to the PCS proforma that have been completed as part of the PCS | Amd Amd | : : | Corr Corr | : : |
| Have any Exception items been required? (See A.3.3: the answer "Yes" means that the implementation does not conform to IEC/IEEE 60802) | No | [ ] | Yes | [ ] |
| Date of Statement |  |  |  |  |

    - Implementation summary

The form in Table A.3 is used to indicate the type of system that the PCS describes.

Table A.3 – Implementation type

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| BC-CCA-N | State the number of Conformance Class A bridge components implemented by the IA-station. | O | 5.7.2, 5.8.2 | Number _____ |
| BC-CCB-N | State the number of Conformance Class B bridge components implemented by the IA-station. | O | 5.7.3, 5.8.3 | Number _____ |
| ESC-CCA-N | State the number of Conformance Class A end station components implemented by the IA-station. | O.1 | 5.9.2, 5.10.2 | Number _____ |
| ESC-CCB-N | State the number of Conformance Class B end station components implemented by the IA-station. | O.1 | 5.9.3, 5.10.3 | Number _____ |
| CNC | Does the IA-station include a CNC? | O | 5.5.4.1 m) | Yes [ ] No [ ] |
| CUC | Does the IA-station include a CUC? | O | 5.5.4.1 n) | Yes [ ] No [ ] |
| PRX | Does the IA-station support proxy functionality? | O | 5.5.4.1 o) | Yes [ ] No [ ] |

IA-station requirements and options

Instructions

One instance of Clause A.5 shall be filled out for an IA-station.

IA-station requirements

The form in Table A.4 is used to indicate the IA-station requirements.

Table A.4 – IA-station requirements

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| IASTA-1 | Does the IA-station support PHY and MAC requirements for external ports? | M | 5.5.1 | Yes [ ] |
| IASTA-2 | Does the IA-station support topology discovery requirements? | M | 5.5.2 | Yes [ ] |
| IASTA-3 | Does the IA-station support requirements for time synchronization? | M | 5.5.3 | Yes [ ] |
| IASTA-4 | Does the IA-station provide a digital data sheet? | M | 5.5.4.3 | Yes [ ] |

Management Requirements for non-proxied IA-stations

The form in Table A.5 is used to indicate management requirements for IA-stations containing conformance class A components

Table A.5 – Additional requirements for non-proxied IA-stations

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| IASTA-5 | Does the IA-station support requirements for Secure management exchanges? | M | 5.5.4.1 a) to l) | Yes [ ] |
| IASTA-6 | Number of of Dynamic Subscriptions to YANG Events and Datastores over NETCONF | M | 5.5.4.1 h) | Number _____ |
| IASTA-7 | Does the IA-station support management YANG modules? | M | 5.5.4.2 | Yes [ ] |

IA-station PHY and MAC options for external ports

The form in Table A.6 is used to indicate PHY and MAC options for external ports.

Table A.6 – IA-station PHY and MAC options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| DOT3-1 | Does the IA-station support PoE over 2 pairs? | O | 5.6.1 a) | Yes [ ] No [ ] N/A [ ] |
| DOT3-2 | Does the IA-station support Power Interfaces? | O | 5.6.1 b) | Yes [ ] No [ ] N/A [ ] |
| DOT3-3 | Does the IA-station support PoE? | O | 5.6.1 c) | Yes [ ] No [ ] N/A [ ] |

IA-station options for time synchronization

The form in Table A.7 is used to indicate options for time synchronization.

Table A.7 – IA-station time synchronization options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| PTP-1 | Does the IA-station support media-independent TimeTransmitter capability according to IEEE Std 802.1AS-2020, 5.4.2 item b) as amended by IEEE Std 802.1ASdr-2024? | O | 5.6.2 a) | Yes [ ] No [ ] |
| PTP-2 | Does the IA-station support Grandmaster PTP Instance capability Grandmaster PTP Instance capability according to IEEE Std 802.1AS-2020 as amended by IEEE Std 802.1ASdr-2024 and IEEE Std 802.1ASdm-2024, 5.4.2 item c)? | O | 5.6.2 b) | Yes [ ] No [ ] |
| PTP-3 | Does the IA-station support more than one PTP port as a PTP Relay Instance according to IEEE Std 802.1AS-2020, 5.4.2 item d)? | O | 5.6.2 c) | Yes [ ] No [ ] |
| PTP-4 | Does the IA-station support hot standby redundancy requirements according to IEEE Std 802.1AS-2020 as amended by IEEE Std 802.1ASdm-2024, 5.4.2, item m)? | O | 5.6.2 d) | Yes [ ] No [ ] |

IA-station secure management exchange options

The form in Table A.8 is used to indicate options for secure management exchange.

Table A.8 – IA-station secure management exchange options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| SECMGMT-5 | Does the IA-station support Writable-Running capability? | O | 5.6.3 a) | Yes [ ] No [ ] |
| SECMGMT-6 | Does the IA-station support Confirmed Commit capability? | O | 5.6.3 b) | Yes [ ] No [ ] |
| SECMGMT-7 | Does the IA-station support Distinct Startup capability? | O | 5.6.3 c) | Yes [ ] No [ ] |
| SECMGMT-8 | Does the IA-station support URL capability? | O | 5.6.3 d) | Yes [ ] No [ ] |
| SECMGMT-9 | Does the IA-station support XPath capability? | O | 5.6.3 e) | Yes [ ] No [ ] |
| SECMGMT-10 | NETCONF-over-TLS server supporting TLS version 1.2? | O | 5.6.3 f) | Yes [ ] No [ ] |
| SECMGMT-11 | Does the IA-station support NETCONF-over-TLS server supporting TLS version 1.3? | O | 5.6.3 g) | Yes [ ] No [ ] |
| SECMGMT-12 | Does the IA-station support PKIX? | O | 5.6.3 h) | Yes [ ] No [ ] |

    - Proxy target requirements

The form in Table A.9 is used to indicate requirements for proxy targets.

Table A.9 – Proxy target requirements

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| PT-1 | Does the IA-station support Secure management exchanges? | O | 5.6.4 a) | Yes [ ] No [ ] |
| PT-1 | Does the IA-station support Secure management exchanges using a management proxy? | O | 5.6.4 b) | Yes [ ] No [ ] |

Bridge component

Bridge component requirements

The form in Table A.10 is used to indicate bridge component requirements.

Table A.10 – Bridge component requirements

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| BC-1 | Does the bridge component support the common bridge component requirements? | M | 5.7.1 | Yes [ ] |
| BC-2 | Does the bridge component support ccA bridge component requirements? | O.2 | 5.7.2 | Yes [ ] No [ ] |
| BC-3 | Does the bridge component support ccB bridge component requirements? | O.2 | 5.7.3 | Yes [ ] No [ ] |

Instructions

One instance of Clause A.6 shall be filled out per bridge component implemented by an IA-station.

Bridge Common bridge component options

The form in Table A.11 is used to indicate bridge component options.

Table A.11 – Common bridge component options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| BC-4 | Does the bridge component support the operation of the credit-based shaper algorithm? | O | 5.8.1 | Yes [ ] No [ ] |

Conformance class A bridge component options

The form in Table A.12 is used to indicate options for bridge components conforming to conformance class A.

Table A.12 – Conformance class A bridge component options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| CCA-BC-1 | Does the bridge component support any of the common bridge component options? | O | 5.8.2 a) | Yes [ ] No [ ] N/A [ ] |
| CCA-BC-2 | Does the bridge component support more than 2 PTP instances? | O | 5.8.2 b) | Yes [ ] No [ ] N/A [ ] |
| CCA-BC-3 | State the number of PTP instances supported by the bridge component. | CCA-BC-2:M | 5.8.2 b) | Number ____ |
| CCA-BC-4 | Does the bridge component support enhancements for scheduled traffic for the 10 Mb/s, 2,5 Gb/s, 5 Gb/s, or 10 Gb/s data rates? | O | 5.8.2 c) | Yes [ ] No [ ] N/A [ ] |
| CCA-BC-5 | Does the bridge component support frame preemption for the 10 Mb/s, 2,5 Gb/s, 5 Gb/s, or 10 Gb/s data rates? | O | 5.8.2 d) | Yes [ ] No [ ] N/A [ ] |

Conformance class B bridge component options

The form in Table A.13 is used to indicate options for bridge components conforming to conformance class B.

Table A.13 – Conformance class B bridge component options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| CCB-BC-1 | Does the bridge component support any of the common bridge component options? | O | 5.8.3 a) | Yes [ ] No [ ] N/A [ ] |
| CCB-BC-2 | Does the bridge component support more than 4 but not more than 8 egress queues? | O | 5.8.3 b) | Yes [ ] No [ ] N/A [ ] |
| CCB-BC-3 | State the number of egress queues supported by the bridge component. | CCB-BC-2:M | 5.8.3 b) | Number ____ |
| CCB-BC-4 | Does the bridge component support more than 1 PTP Instance? | O | 5.8.3 c) | Yes [ ] No [ ] N/A [ ] |
| CCB-BC-5 | State the number of PTP Instances supported by the bridge component. | CCB-BC-4:M | 5.8.3 c) | Number ____ |
| CCB-BC-6 | Does the bridge component support enhancements for scheduled traffic? | O | 5.8.3 d) | Yes [ ] No [ ] N/A [ ] |
| CCB-BC-7 | Does the bridge component support frame preemption? | O | 5.8.3 e) | Yes [ ] No [ ] N/A [ ] |

End station component

Instructions

One instance of Clause A.7 shall be filled out per end station component implemented by an IA-station.

Common end station component requirements

The form in Table A.14 is used to indicate common requirements for end station components.

Table A.14 – Common end station component requirements

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| ESC-1 | Does the end station component support the common end station component requirements? | M | 5.9.1 | Yes [ ] |
| ESC-2 | Does the end station component support the ccA end station component requirements? | O.3 | 5.9.2 | Yes [ ] No [ ] |
| ESC-3 | Does the end station component support the ccB end station component requirements? | O.3 | 5.9.3 | Yes [ ] No [ ] |

Common end station component options

The form in Table A.15 is used to indicate options for end station components.

Table A.15 – Common end station component options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| ESC-4 | Does the end station component support the operation of the credit-based shaper? | O | 5.10.1 a) | Yes [ ] No [ ] |
| ESC-5 | Does the end station component support talker end system behaviors? | O | 5.10.1 b) | Yes [ ] No [ ] |
| ESC-6 | Does the end station component support listener end system behaviors? | O | 5.10.1 c) | Yes [ ] No [ ] |

Conformance class A end station component options

The form in Table A.16 is used to indicate options for end station components conforming to conformance class A.

Table A.16 – Conformance class A end station component options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| CCA-ESC-1 | Does the end station component support any of the common end station component options? | O | 5.10.2 a) | Yes [ ] No [ ] N/A [ ] |
| CCA-ESC-2 | Does the end station component support more than 2 PTP Instances? | O | 5.10.2 b) | Yes [ ] No [ ] N/A [ ] |
| CCA-ESC-3 | State the number of PTP Instances supported by the end-station component. | CCA-ESC-2:M | 5.10.2 b) | Number ____ |
| CCA-ESC-4 | Does the end station component support enhancements for scheduled traffic for data rates 10 Mb/s, 2,5 Gb/s, 5 Gb/s, or 10 Gb/s? | O | 5.10.2 c) | Yes [ ] No [ ] N/A [ ] |
| CCA-ESC-5 | Does the end station component support requirements for frame preemption for data rates 10 Mb/s, 2,5 Gb/s, 5 Gb/s, or 10 Gb/s? | O | 5.10.2 d) | Yes [ ] No [ ] N/A [ ] |

Conformance class B end station component options

The form in Table A.17 is used to indicate options for end station components conforming to conformance class B.

Table A.17 – Conformance class B end station component options

| Item | Feature | Status | References | Support |
| --- | --- | --- | --- | --- |
| CCB-ESC-1 | Does the end station component support any of the common end station component options? | O | 5.10.3 a) | Yes [ ] No [ ] N/A [ ] |
| CCB-ESC-2 | Does the end station component support one or more PTP Instances? | O | 5.10.3 b) | Yes [ ] No [ ] N/A [ ] |
| CCB-ESC-3 | State the number of PTP Instances supported by the end-station component. | CCB-ESC-2:M | 5.10.3 b) | Number ____ |
| CCB-ESC-4 | Does the end station component support enhancements for scheduled traffic? | O | 5.10.3 c) | Yes [ ] No [ ] N/A [ ] |
| CCB-ESC-5 | Does the end station component support requirements for frame preemption? | O | 5.10.3 d) | Yes [ ] No [ ] N/A [ ] |

___________

Copyright release for the PCS: Users of this document may freely reproduce the PCS contained in this document so that they can be used for their intended purpose.