IEEE P802.11

Wireless LANs

Consideration on

Collision Detection Method

for WLAN

July 9, 1991

Toshiba Corporation

Hideaki Haruyama

Three CD methods discussed at 802.3 for COAX based Broadband Network

. beat-signal level detection

. Bit comparison & Col. Enhancement

. Random pulse

* Beat Signal level detection

level control & Freq. Modulation

Collision  Zero or double level

"Listen while talk capability

* Bit comparison & Col Enhancement

Compare Sending & Receiveing Data bit by bit

"Listen while Talk"

* Random Pulse

send two pulses with random interval

"Deteroation of throughput"

Not necessarily requires "Listen while Talk"

Occupied Bandwidth

"Random Pulse

1/2 "Beat signal"

<1/2 "Bit Comparison"

Direct Transmission (no AP)

robustness

Design Example

Collision Window:	n-pulse

Tx pulse number:	k

Collision Missing Probability

1/nCk

n=33  k=16    nCk>109

Pulse width: 	300 ns

Delay Spread margin: 	200 ns

Col. window = 16.5 ms

< 57.6 ms

(Minimum Packet Length)

Throughput Deterioration is little

Conclusion

Random Pulse method requires

* minimum Bandwidth

* no Access Point

Applying to Wireless LAN

Throughput Deterioration is little