Accommodating a Range of Intelligence in the Physical Layer

A MAC/PHY Interface Issue

Jim Schuessler

National Semiconductor Corporation

408-721-6802

jim@berlioz.nsc.com

Assumptions:

·	There will be one MAC (I hope)

·	There will be multiple PHYs.

·	There will be separate documents for the MAC, each PHY, the Management Functions and perhaps other interfaces/functions (ref. John Cory and Nathan Silberman: doc. 92/56)

·	PHYs will address different speed/power/cost trade-offs.

·	P802.11 must accommodate these trade-offs (or become/remain irrelevant in mkt.)

Goal:

·	A single MAC/PHY interface which does not preclude proper market driven decisions about the MAC or the PHY, whatever type it may be.

Some take a top down, logical (as opposed to physical) approach to the MAC/PHY interface question.  This results in a "services" oriented interface with SAPs and logical functions to be supported.  Others take a bottoms up, physical approach to the MAC/PHY interface question.   They know this approach has resulted in the lowest cost, most efficient interface in the past, at the penalty of reuse and "orthogonality". (elegance?)  I contend we can and must accommodate both approaches.

A Structure for the Solution:

(Idea of a media dependent and media independent MAC/PHY interface was proposed by Bob Crowder in email dated 6/20/92.  Is this what you mean Bob?)

µ §

Media Independent MAC/PHY Interface:

This is the "classic" MAC/PHY interface.  It is defined in terms of logical functions, services, data and messages.  It is important in terms of what the MAC can expect from the PHY, especially with respect to data integrity and security.  It answers the question, "What services does the MAC/PHY interface support?"

Media Dependent MAC/PHY Interface:

This is the efficient or expedient MAC/PHY interface.  It is defined in terms of signals and lines expressing exact hardware functionality and no more.  It answers the question, "How can I make the lowest cost (dirt cheap..), most efficient MAC/PHY interface?"

Who's Responsibility?:

µ §

µ §

Problems with this approach:

1.	Limitations imposed at the Media Dependent interface may make implementation of functions required at the Media Independent interface impossible.  Is this surmountable?

2.	More work.  (Yes, but worth it.)

The Dave Bagby Questions:  (email 6/19/92)

1.	Is the MAC/PHY interface the place where MAC/PHY independence is achieved?  Answer: NO for Media Dependent interface, YES for Media Independent interface.

2.	What level of intelligence should exist on either side of the MAC/PHY interface?  Answer: As little as possible given the agreed trade-offs of a specific PHY.  Contention may still arise if (when?) members cannot target a specific PHY to a well defined market segment.  At least with this approach, we have some freedom to address our competing market requirements, and still produce a single viable standard, albeit with multiple parts.  i would hope we could avoid, for instance, multiple ISM 2.4GHz. PHYs., or multiple IR PHYs.

Appendix

Appendix