IEEE 802.11
Wireless Access Methods and Physical Layer Specifications

Draft Proposal for a Frequency Hopping Spread Spectrum PHY Standard

Nathan Silberman

California Microwave Inc.

985 Almanor Ave.

Sunnyvale, CA 94086

Introduction

This contribution is intended to provide a framework for the definition of the  IEEE802.11  FHSS PHY standard.  It also outlines some of the criteria used for defining this PHY.  First PHY will be defined at 2.4 GHz, other frequencies will follow. Some parameters in the specifications require inputs from the MAC group and an agreed channel model.  Committee members are invited to provide inputs to the completion of this document.  The specification was put in a table format for as long as it is a "live" document .  Once finalized, the spec. will be converted to the std. IEEE 802 format.

Requirements outline:

·	Compliance with Regulatory Agencies for unlicensed operation

·	Compliance with 802.11 PAR (Data Rate at least 1 Mbps, etc.)

·	Operation in a multinetwork environment (multiple collocated networks)

·	Minimum Area coverage

·	Suitable for low power consumption implementations

·	Cost effective

·	Ensure Interoperability between conformant FHSS stations.

·	Modes of operation: 
- peer to peer with no prior knowledge
- node to AP and AP to node

·	Support asynchronous and time deterministic connectivity.

·	Support a specified number of stations per cell (Access Point)

·	Suitable for small size implementation

·	Robust operation in narrow band and partial band interference as well as multipath fading.

·	Graceful degradation under load and interference.

Specifications

The following table represents a template for Frequency Hopping PHY specification. Several blanks were left for those items that will be determined after the channel model is agreed  and after PHY MAC interface is determined;  Other parameters have to be worked out between PHY and MAC groups.

|  | Parameter | Proposed Spec. | Comments/Notes |
| --- | --- | --- | --- |

| 1. | Frequency Range | 2.4 to 2.4835 GHz | Other frequency bands will follow. |
| --- | --- | --- | --- |
| 2. | Minimum number of hopping channels / set | 75 | per FCC part 15.247 |
| 3. | No of hops per sec. | 2.5 to 10(?) | propose 4 |
| 4. | Transmitted power levels [mW] | a. 1000 / 100 b. 250,100, 50 mW (optional) levels | a. default US/default Europe b. optional levels |
| 5. | Optional Transmitted power control | Four discrete levels as above or continuous |  |
| 6. | Max. Radiated EIRP | a. 6 dBW in US ? dBW in Europe | Total radiated power including antenna gain As defined by regulatory agencies in each country. |
| 7. | Transmitted power variation (tolerance) | +0 /-3 dB max. (total) | & |
| 8. | Frequency deviation | DFmax= 300KHz(?) | & |
| 9. | Spurious Output: transmit and receive In band (2.4 -2.4835 GHz) at more than 2 MHz from Fc Out of Band | 85 dBc 65 dBC |  |
| 10. | Minimum Receiver Sensitivity | -80 to -90 dBm [TBD] | & Final number TBD after channel and link model agreed |
| 11. | Max. Input signal level at antenna connector with no performance degradation | 0 dBm | & |
| 12. | Adjacent Channel selectivity | -25 dB at channel boundary -65 dB @Df=2 MHz |  |
| 13. | Channel bandwidth (allocated) | 1 MHz |  |
| 14. | Occupied channel bandwidth (spectrum shape) | 30 dBc @ Df=.5 MHz 86 dBc @ Df=2 MHz |  |
| 15. | Receiver center frequency acceptance range | +/- TBD [Hz] | For interoperability purposes |
| 16. | Modulation | 4 level CPFSK |  |
| 17. | Channel Data Rate | 1.25 Mbps | Higher data rates not recommended because of delay spread and bandwidth constraints |
| 18. | Fallback data rate | a. 250 Kbps b. TBD (optional) |  |
| 19. | Phy supplied Clock Jitter | 0.0625 microsec. |  |
| 20. | Adjacent channel rejection margin: Maximum power of a signal with random data at Fc+/- 2 MHz for 10E-5 BER in a signal at Fc with -85 dBm input power level | -20 dB |  |
| 21. | Preamble length | a. 32 bit times b. Variable PHY preamble length | & TBD |
| 22. | Clock recovery | withstands patterns up to (7)continuous 1's or (7) 0's with no degradation in output signal to noise ratio and bit error rate |  |
| 23. | Carrier (energy)detect response time | TBD | & |
| 24. | Spurious emissions in band ( @ Df³1 MHz from Fc) | -80 dBC @ ?W TX power |  |
| 25. | Spurious emissions out of band | a. -20 dBW in US b. - 60 dBW/100 KHz @ 1 to 10 GHz c. -66 dBW/100 KHz @ 30 MHz to 1 GHz | country dependent |
| 26. | Switching time TX to RX | TBD | Time from full power transmission to full sensitivity receiver availability. This parameter should include any preamble time used for receiver synchronization. |
| 27. | Switching time RX to TX | TBD | time from full sensitivity reception to full power transmitter availability |
| 28. | Channel switching time (hop settling time) | typically 100 to 200 mS | Elapsed time from receipt of hop command until unit frequency settles within +/- DF= Receiver Acceptance range or TX frequency tolerance (whichever is tighter) |
| 29. |  |  | deleted |
| 30. | BER at specified Eb/No | 10 exp. -6 @ Eb/No=17 dB | Includes modem implementation margin |
| 31. | Channel availability | 99.5 % | Could also be specified as probability of outage. With no interference. |
| 32. | TX Frequency Stability | 15 ppm |  |
| 33. | Data Line / Clock input / output Jitter | TBD | & Includes static and dynamic Jitter (see 802.3 definition) dependent on MAC requirements. |
| 34. |  |  | deleted |
| 35. |  |  | deleted |
| 36. | Antenna port impedance | 50 ohms |  |
| 37. | VSWR | Devices shall stand 0£VSWR£¥ with no damage. -Operational VSWR =TBD |  |
| 38. |  |  | deleted |
| 39. |  |  | deleted |
| 40. | Interface lines to Convergence layer (when exposed) | RX Data TX Data RX/TX clock Data valid Control line Status line Ctl./Sta clock | & Timing and levels TBD. |
| 41. | PHY-MAC Net Management info./control variables | a. Signal Quality b. Loop back c. No. of channels available d. Channel in use (sequence) e. No. of diversity channels f. Which Diversity in use g. Alternate Diversity use h. Lock i. No. of RX signal strength levels j. RX signal strength levels k. RX Signal Strength l. PHY type m. Channel quality n. Channel available (for LBT) o. No. of TX power levels p. Read TX output level q. Set TX output level r. Hop [?] s. Transmit t. Receive u. Sleep mode v. Wake-up w. Standby mode (low power) x. Data rate -indication - command y. Jabber control function z. Jabber indication | &,* Most signals are bi-directional |
| 42. | Other PHY-MAC Net Management info./control variables | a. PHY initialize b. PHY specific tally collection c. Adaptive power control by learning reception statistics. | * |
| 43. | Safety Requirements | Compliance with applicable Safety Agencies requirements [TBD] |  |
| 44. | DTE/DCE Interface | TBD | * |
| 45. | ACK protocol support | TBD | * |

Notes:

& indicates dependency on the channel model.

*  indicates inputs from MAC group.

Status of the receive signal/on a per frame basis

Status of the PHY control parameter /on a per frame basis

1. Coding not addressed yet.

2. Issue: how to deal with diversity? Should this be covered by  the "channel"

.

·	MAC-PHY Paired exchanges

PHY_data.request sequence=start_ of_ activity +consecutive activity_ requests specifying data +end_of_ data

PHY_data.request _completion=PHY_data.confirmation

·	PHYE reports
PHY_data.indication
-start of activity
-consecutive data activity indicators
-single end of data