IEEE 802.11

Wireless Access Method and Physical Specification

Title:	802.11 MAC Requirements and Comparison Criteria.

Presented by:

Wim Diepstraten

NCR WCND-Utrecht

NCR/AT&T Network Product Group

Nieuwegein	The Netherlands

Tel:	(31)-3402-76482

Fax:	(31)-3402-39125

Email:	Wim.Diepstraten@utrecht.ncr.com

Abstract:	This paper discusses the "User Requirements" for a Wireless MAC, and arrives to a list of comparison criteria, on which existing and future 802.11 MAC proposals should be judged.

Introduction:

This paper intends to give a list of the requirements that an 802.11 MAC should support, together with related Comparison Criteria. It can be used as a checklist to guide in the comparison of different protocols.

# Required MAC services:

The following services need to be provided:

-	An asynchronous Bursty traffic service.

802 LLC services should be provided

.	Unicast transfer mode.

.	Broadcast/Multicast transfer mode.

.	Meet the reliability requirements (with possible exception for Broadcasts)

Note:	Different service levels may be specified.

-	An Isochronous (Time Bounded Service)

.	Should be designed to be an option.

.	Should be dimensioned to support Voice.

Related Questions:

.	Different service levels may be specified.

.	What are the limitations applicable to support the Isochronous service.

.	Does the MAC offer mixed Isochronous/Asynchronous capability.

.	What is the burden on Asynchronous implementation complexity.

# Support Infrastructure based Multiple Cell Networks

The connectivity options that are possible within the total architecture should be made clear. Global connectivity requirements are:

-	Peer-to-Peer within the BSA.

-	Peer-to-Peer between different BSA's.

-	Connectivity with existing wired network implementations.

Related questions:

-	Is Direct Peer-to-Peer supported in Infrastructure mode.

-	Can an Ad-Hoc network overlap in the same channel.

## Infrastructure considerations:

-	Can any 802 compatible LAN be used as the Distribution system.

-	What are the supported configurations.

-	What are the unique functions needed from the Distribution system.

-	What are the limitations for different supported configurations.

-	What are the provisions to support "Re-association" across Routers and Bridges and Gateways.

-	What is the throughput performance per station across the Distribution system, and what are the factors that determine this.

-	What is the required infrastructure for the Time Bounded Services

## MAC should be able to operate in a multiple BSA configurations in a single channel environment.

-	Can the MAC handle overlapping BSA's using a single channel.

-	What are the provisions for spatial re-use of the spectrum.

-	What is the overhead involved to coordinate this.

-	What is the impact of overlapping networks on throughput.

-	What are the service limitations for this configuration.

-	What is the "Re-associate" algorithm, and what is the impact on response time and throughput.

-	What are the "Re-associate" provisions for Time Bounded Services.

## MAC should be able to operate in a multichannel environment.

-	What is the overhead involved to coordinate this.

-	Is there a limit (max or min) on the number of channels.

-	What is the "Re-associate" algorithm, and how seamless is it.

-	How are the channel re-use limitations aspects handled in the MAC.

-	What channel management functions are needed.

# Support of Ad-Hoc Networks

-	Can the MAC support infrastructure-less (ad-Hoc) networks.

-	Can the Ad-Hoc networks overlap with infrastructure networks, and what is the mutual impact.

-	Can a Station be connected to an Ad-Hoc and Infrastructure Network at the same time.

-	What is the procedure to set up an Ad-Hoc network.

-	Is there a Power saving mode supported in Ad-Hoc.

-	What security services are available.

-	Can multiple Ad-Hoc networks overlap (on the same channel)

# MAC must support low power operations.

-	What are the Power Management provisions supported by the MAC.

-	Does the MAC support extreme low power stations that need battery life of months.

-	What are the Power Management provisions for Ad-Hoc operation.

-	What are the different service levels associated with different Power Management modes.

-	What is the impact of the "Re-associate" scenario on the power consumption.

-	What is the effect on the performance.

.	Station throughput performance.

.	Response time

# MAC need to support Multiple PHY's.

-	Support at least a 1-20 Mbps bitrate.

-	Need to support the regulatory requirements in the different bands.

.	DSSS in 915 MHz, 2.4 GHz and 5.8 GHz ISM bands

.	FHopper in 2.4 GHz and 5.8 GHz ISM bands

.	Narrow Band (1.9 GHz including Etiquette)

.	IRed

.	Wide-Band (ETSI 5.2 GHz)

-	What are the generic provisions to support the different PHY's.

-	What is the method to include PHY dependent MAC functions.

# MAC Access Function Requirements.

-	What is the default Coordination Function for Ad-Hoc and Infrastructure mode operation.

-	What is the limit for the number of Stations that can be supported by the Coordination Function.

-	How fair is the access method.

-	What is the stability of the access method during high load.

-	What is the throughput capacity of the access method.

-	How robust is the access method for interference.

.	ISM band interference.

.	Co-channel interference.

-	What are the medium sharing characteristics in case of a overlap between BSA's that use the same channel.

-	What is the overhead associated with the access method.

-	What is the method to support mixed Isochronous/Asynchronous traffic.

-	What is the Bursty traffic performance of a Station

.	MAC to MAC.

.	End to End.

-	What is the effect of Isochronous traffic on Bursty traffic performance.

-	What is the Isochronous robustness.

# Access Method Independent Features

-	What are the security provisions provided by the MAC.

.	What is the impact on the existing infrastructure when security is required.

.	What are the different levels of security supported.

-	Does the MAC support mixed bitrate operation.

-	How does the MAC support the defined DSS functions.

-	What are the extra requirements for other 802 standards, imposed by this MAC and proposed architecture.

-	What are the parameters affecting inter Access Point interoperability.

-	What are the managed objects of a MAC

Conclusion:

The MAC requirements have been listed in major categories. Each of the catagories provides a checklist that can be used to compare different MAC proposals against.

I propose to use this checklist as a basis for the comparison, and as a guideline to develop comparison criteria.