# CSMA/CA Based Protocol

A Distributed Access Protocol Proposal

## Supporting Time Bounded Services

By: Wim Diepstraten

# Wireless Network Architecture

µ §

## *	Infrastructure mode "Base Station oriented".

Default all traffic goes via the AP.

# Supported Services:

## *	Asynchronous Data Service:

-	Short response time with high instantaneous Throughput.

-	Suitable for Bursty traffic.

## *	Time Bounded Service (Optional):

-	Time Bounded Service dimensioned for Voice.

-	Allows mixed Voice/Data.

-	Requires sufficient BSS isolation, (TDMA type access protocols have similar requirements).

# Basic Access Protocol:

## *	Use Distributed Access Protocol for efficient medium sharing.

## *	Robust for interference.

-	CSMA/CA + Ack for unicast frames.

-	With MAC level recovery

-	CSMA/CA for Broadcast frames.

## *	Supports Ad-Hoc operation seamlessly

# CSMA/CA explained:

µ §

## *	Reduce collision probability where mostly needed.

## *	Implement different priority levels.

(to allow immediate Ack and Isochronous coexistence)

# CSMA/CA+Ack Access Protocol

µ §

*	Exponential Backoff with Access Retry limit.

*	Retransmission after Random delay when no Ack received.

# Async service Characteristics:

## *	Short response time, and high throughput efficiency similar to 802.3 CSMA/CD.

## *	Lost packets due to collisions and/or interference are recovered at the MAC level.

## *	Stable under high Load conditions.

## *	Efficient Medium sharing without added control overhead.

## *	Includes Coexistence Provisions for Time Bounded Services.

# Async/Time Bounded model:

µ §

## *	Data, Voice or mixed implementations possible.

## *	Time Bounded Service uses Point Coordination Function.

## *	Time Bounded capability does not burden the Async service implementation.

# Time Bounded Characteristics:

## *	Built on Asynchronous Access method.

## *	Uses CSMA/CA + (Ack) with highest priority.

-	With limited Asynchronous recovery.

## *	Dimensioned to support mixed Voice/Data.

## *	Video support possible at higher  PHY rates.

*	Isochronous Framing Period is PHY speed dependent IFP= 20-25 msec 	for a 2 Mbps PHY.

## *	Unused reserved Isochronous Bandwidth can be used for Asynchronous traffic.

# Time Bounded Characteristics:

## *	Can support different PHY speeds (1-20 Mbps)

## *	Support variable packet size on a per frame basis, without control overhead.

-	Can take full advantage of "Talk Spurt" characteristics of Voice.

-	Allows flexible congestion control.

## *	Includes provisions for Power Consumption Management.

## *	Includes basic re-association provisions.

# Time Bounded Services (Cont.):

µ §

*	Time Synchronization assumed by regular Beacon.

# Performance Example:

## *	Assumption:	32Kbps ADPCM Voice

15 Byte MAC + Wavelan PHY Overhead

## *	Example: @ 2 Mbps Modem speed:

Voice only:					28 FDX (using talkspurt)

Voice/Data   576 Byte:		23 FDX (using talkspurt)

Voice/Data 1500 Byte:		18 FDX (using talkspurt)

Voice/Data 1500 Byte:		  9  FDX (100% load)

Async Data throughput:     >  75 KByte/sec (assuming max Isoc load)

Async Data throughput:     >130 KByte/sec (assuming 9 FDX Talkspurt channels)

Async Data throughput:     >200 KByte/sec (no Isoc connection active)

# Conclusion:

*	CSMA/CA + Ack is an efficient distributed access method with good medium sharing characteristics.

*	A very flexible Time Bounded Capability has been demonstrated, built on top of CSMA/CA.

*	Added advantage over TDMA is its frame size flexability and optimum Asynchronous and Time Bounded sharing characteristic.

# Service required from the PHY

µ §

*	Fast CS function with approx. 10 dB more sensitivity than the acceptable Data quality level.

*	Fast Rx-to-Tx Turnaround time.