IEEE 802.11
Wireless Access Methods and Physical Layer Specifications

Draft Proposal for a Frequency Hopping and Direct Sequence Spread Spectrum PHY Standard

Editor

Nathan Silberman

California Microwave Inc.

985 Almanor Ave.

Sunnyvale, CA 94086

This is a revised version of document IEEE P802.11/92-127r1 that integrates Frequency hopping and Direct Sequence proposed specs.  Direct sequence spec is as proposed by Jan Boer.  It also incorporates agreed changes madeof comments received in theJanuary 11, 1993 interim meeting and the March 1993 plenary meeting.  For the previous version please refer to document  802.11/ 92-127r1.

Introduction

This contribution is intended to provide a framework for the definition of the  IEEE802.11  FHSS  and DSSS PHY standard.  It also outlines some of the criteria used for defining this PHY.  First PHY will be defined at 2.4 GHz, other frequencies will follow.  Some parameters in the specifications require inputs from the MAC group and an agreed channel model.  Committee members are invited to provide inputs to the completion of this document.  The specification was put in a table format for as long as it is a "live" document .  Once finalized, the spec. will be converted to the std. IEEE 802 format.

Requirements outline:

·	Compliance with Regulatory Agencies for unlicensed operation

·	Compliance with 802.11 PAR (Data Rate at least 1 Mbps, etc.)

·	Operation in a multinetwork environment (multiple collocated networks)

·	Minimum Area coverage

·	Suitable for low power consumption implementations

·	Cost effective

·	Ensure Interoperability between conformant FHSS stations.

·	Modes of operation: 
- peer to peer with no prior knowledge
- node to AP and AP to node

·	Support asynchronous and time deterministic connectivity.

·	Support a specified number of stations per cell (Access Point)

·	Suitable for small size implementation

·	Robust operation in narrow band and partial band interference as well as multipath fading.

·	Graceful degradation under load and interference.

Specifications

The following table represents a template for Frequency Hopping PHY specification. Several blanks were left for those items that will be determined after the channel model is agreed  and after PHY MAC interface is determined;  Other parameters have to be worked out between PHY and MAC groups.

|  |  |  | Frequency Hopping | Direct Sequence | Comments |
| --- | --- | --- | --- | --- | --- |
| 1. | Tx & Rx | Frequency Range | 2.4 to 2.4835 GHz U.S.A 2.471 to 2.497 Ghz Japan 2.4 to 2.485 Ghz Europe |  | Other frequency bands will follow. |
| 2a. | Tx & Rx | Minimum number of channels / set | 75 |  | per FCC part 15.247 |
| 2b. |  | Minimum number of channels / set |  | 3 in USA 1 in Japan |  |
| 3. | Tx & Rx | No of hops per sec. | 2.5 to 200 |  |  |
| 3b. | Tx & Rx | Spreading sequence |  | 11 chips Barker sequence (1,-1,1,1,-1,1,1,1,-1,-1,-1) |  |
| 4. | Tx | Transmitted power levels [mW] | a. max. 1000 / 100 b. 250,100,50,10 mW (optional) levels | a. max. 1000 / 100 b. 250,100,50,10 mW (optional) levels | a. US/Europe, Japan b. optional levels |
| 4a. | Tx | Minimum transmitted power level | 10 mW 1 mW for battery operated equipment (?) | 10 mW 1 mW for battery operated equipment (?) | Required for conformance testing. |
| 5. | Tx | Optional Transmitted power control | Four discrete levels as above or continuous, required above 100 mW. | 3 to 7discrete levels as aboveous, required above 100 mW. | per PHY group vote on 1/11/93 |
| 6. | Tx | Max. Radiated EIRP | per FCC part 15.247 in US per ETSI res 02-09 in Europe per TBD in Japan | per FCC part 15.247 in US per ETSI res 02-09 in Europe per TBD in Japan | Total radiated power including antenna gain As defined by regulatory agencies in each country/. for reference only |
| 7. |  |  |  |  | Deleted |
| 8. | Tx | Modulation Frequency deviation | DFmax= 300KHz |  | & Required for interoperability |
| 9. |  |  |  |  | Deleted and merged with 25. |
| 10. | Rx | Receiver Minimum input level sensitivity | - 80 dBm @ 10-5 BER | - 76 dBm @ 10-5 BER | reinstated |
| 11. | Rx | Receiver maximum input level | - 20 dBm | - 20 dBm [?] | reinstated |
| 12. | Rx | Alternate channel interference tolerance | 45 dB at 10-5 measured by the following method: input an in-channel receive signal level that provides 10-5 BER, and increase this signal level by 1 dB; an alternate channel signal modulated in the same fashion is increased in level until BER is 10-5. The difference between the desired and undesired signal levels is greater than 45 dB; all measured in an AWGN channel | NA | To allow specification of the transmitted spectrum mask; To facilitate interoperability |
| 12 a. | Rx | Adjacent channel rejection | NA | 50 dB |  |
| 13. | Tx & Rx | Channel bandwidth (allocated) @20 dB | 1 MHz per FCC part 15.247 | 22 MHz | For reference only; will be deleted . |
| 14. | Tx | Occupied channel bandwidth (spectrum shape) | 20 dBc @ Df=+0.5 Mhz 45 dBc @ Df=+ 2Mhz 60 dBc @ DF=+ 30 MHz | 30 dBc @ Df = +11 MHz 55 dBc @Df = +0.5Mhz | Defines transmitted spectrum mask. Required for coexistence of multiple networks. -20 dBc at channel boundary per FCC part 15.247 for FH |
| 15. | Tx & Rx | Receiver center frequency acceptance range | +/- TBD [Hz] |  | For interoperability purposes |
| 16. | Tx & Rx | Modulation | 4 level CPFSK with baseband shaping TBD . | DQPSK | shaping TBD after channel model agreed. |
| 17. | Tx & Rx | Channel Data Rate | 1.25 Mbps | 2 Mbps |  |
| 18. | Tx & Rx | Fallback data rate | 800 Kbps, 500 Kbps, 250 Kbps | 1 Mbps | per Mc Donald and Le Maut |
| 19. |  | Phy supplied Clock Jitter | 0.0625 microsec. |  |  |
| 19 a. |  | Bit Clock Accuracy (baseband) |  | < 50 ppm |  |
| 20. |  |  |  |  | Deleted per McDonald and |
| 21. | Tx & Rx | Preamble length | a. 32 bit times b. Variable PHY preamble length | 146 symbols (146 mS ) | & TBD |
| 22. | Tx & Rx | Clock recovery | withstands patterns up to (7)continuous 1's or (7) 0's with no degradation in output signal to noise ratio and bit error rate. Scrambling polinomial : 1+x-4+x-7. | withstands patterns up to (7)continuous 1's or (7) 0's with no degradation in output signal to noise ratio and bit error rate. Scrambling polinomial : 1+x-4+x-7. | Implies use of a scrambler. Polinomial TBD. |
| 23. | Rx | Carrier (energy)detect response time | TBD | < 30 mS | & Required for upper layers decision making. |
| 24. | Tx & Rx | Spurious emissions in the frequency band ( @ Df³+4MHz from Fc) | -64dBc |  | Chadwicks proposal |
| 25. | Tx & Rx | Spurious emissions out of band | per FCC part 15.247,15.205 and 15.209in USA per ETSI RES 02-09 in Europe. | per FCC part 15.247,15.205 and 15.209in USA per ETSI RES 02-09 in Europe. | for reference only |
| 26. | Tx & Rx | Switching time TX to RX | 100 microsec. | 10 mS | FH:per Le Maut Time from full power transmission to full sensitivity receiver availability. It should include any preamble time used for receiver synchronization. DS:In a CSMA/CA like protocol a interframe space is defined. Rx function ( carrier sense) must be in beefore new frame may arrive. |
| 27. | Tx & Rx | Switching time RX to TX | TBD | 10 ms | time from full sensitivity reception to full power transmitter availability |
| 28. | Tx & Rx | Channel switching time (hop settling time) | 300mS max. | NA | Elapsed time from receipt of hop command until unit frequency settles within +/- DF= Receiver Acceptance range or TX frequency tolerance (whichever is tighter) |
| 29. |  | SIR for frame to frame reception |  | 8 dB | deleted |
| 30. | Rx | BER at specified Eb/No | 10 exp. -5 (previously 10 exp. -6 ) @ Eb/No=16 dB | 10 exp. -5 (?10 exp. -6 ) @ Eb/No=16 dB | -Includes modem implementation margin -This is a MAC requirement. DS at thermal noise - 174 dBm/Hz and man made noise 18 dB above thermal noise, receiver sensitivity is better than -76 dBm at this BER. |
| 31. | Tx & Rx | Channel availability | 99.5 % | 99.5 % | Could also be specified as probability of outage. With no interference. from the PAR. |
| 32. | Tx | TX Frequency Stability | 20 ppm | 50 ppm | including aging for 4 years and over +10 to + 40 deg. C |
| 33. |  | Data Line / Clock input / output Jitter | TBD | TBD | & Includes static and dynamic Jitter (see 802.3 definition) dependent on MAC requirements. |
| 34. |  |  |  |  | deleted |
| 35. |  |  |  |  | deleted |
| 36. | Tx & Rx | Antenna port impedance (if exposed) | 50 ohms | 50 ohms | for interoperability at antenna port. |
| 37. | Tx & Rx | VSWR | Devices shall stand 0£VSWR£¥ with no damage. -Operational VSWR =TBD | Devices shall stand 0£VSWR£¥ with no damage. -Operational VSWR =TBD | for conformance testing |
| 38. |  |  |  |  | deleted |
| 39. |  |  |  |  | deleted |
| 40. |  | Interface lines to Convergence layer (when exposed) | RX Data TX Data RX/TX clock Data valid Control line Status line Ctl./Sta clock | RX Data TX Data RX/TX clock RTS/CTS Carrier Sense MDA (MAC data available) Control line Status line Ctl./Sta clock | & Timing and levels TBD. |
| 41. |  | PHY-MAC Net Management info./control variables | a. Signal Quality b. Loop back c. No. of channels available d. Channel in use (sequence) e. No. of diversity channels f. Which Diversity in use g. Alternate Diversity use h. Lock i. No. of RX signal strength levels j. RX signal strength levels k. RX Signal Strength l. PHY type m. Channel quality n. Channel available (for LBT) o. No. of TX power levels p. Read TX output level q. Set TX output level r. Hop [?] s. Transmit t. Receive u. Sleep mode v. Wake-up w. Standby mode (low power) x. Data rate -indication - command y. Jabber control function z. Jabber indication |  | &,* Most signals are bi-directional |
| 42. |  | Other PHY-MAC Net Management info./control variables | a. PHY initialize b. PHY specific tally collection c. Adaptive power control by learning reception statistics. |  | * |
| 43. |  | Safety Requirements | Compliance with applicable Safety Agencies requirements [TBD] |  |  |
| 44. |  | DTE/DCE Interface | TBD |  | * |
| 45. |  | ACK protocol support | TBD |  | * |

Notes:

& indicates dependency on the channel model.

*  indicates inputs from MAC group.

Status of the receive signal/on a per frame basis

Status of the PHY control parameter /on a per frame basis