IEEE 802.11

Wireless Access Method and Physical Specification

Draft Proposal for a Direct Sequence Spread Spectrum PHY standard

edited by Jan Boer, AT&T

This document is a second revised version of document IEEE P802.11-232 for the direct sequence proposed specification. It incorporates the changes agreed upon in the Vancouver March 1994 meeting.  Cells in the table, that have been changed  compared to the r1 revision are shaded.

|  |  | Parameter | Proposed Spec Direct sequence | Comments/Notes | Status |
| --- | --- | --- | --- | --- | --- |

| 1 | T&R | Frequency range | 2.4 to 2.4835 GHz (USA) 2.4-2.5 GHz (Europe) 2.471-2.497 GHz (Japan) | Other frequency bands will follow | closed |
| --- | --- | --- | --- | --- | --- |
| 2b | T&R | Number of channels | 3+2 | See appendix for the definition of the channels | closed |
| 3b | T&R | Spreading sequence | 11 chips Barker sequence (1,-1,1,1,-1,1,1,1,-1,-1,-1) |  | closed |
| 3c | T&R | Data scrambler | polynomial 1+x-4+x-7 | ALL bits transmitted by the PHY shall be scrambled. See appendix for a description of the scrambler | closed |
| 4a | Tx | Transmitted power levels [mW] | Max 1000/100 | US/Europe,Japan | closed |
| 4b | Tx | Minimum transmitted power level | 10 mW | Required for conformance testing | closed |
| 5 | Tx | Transmitted power control | up to a maximum of 4 levels required above 100mW |  | closed |
| 6 | Tx | Max. Radiated EIRP | Per FCC part 15.247 in US Per ETSI res 02-09 in Europe Per TBD in Japan | total radiated power including antenna gain, as defined by regulatory agencies in each country. For reference only. | closed |
| 8 | Rx | Receiver minimum input level sensitivity | -80 dBm @ 10-5 BER | At 2 Mb/s, 11 chip code rate at the antenna connector | closed |
| 9 | Rx | Receiver maximum input level | - 4 dBm |  | closed |
| 10b | Rx | Adjacent channel rejection | 37 dB at 10-5 measured by the following method: input an inchannel receive signal level 2dB above the level specified in item 8; set a adjacent channel signal modulated in the same fashion to a level 39dB above the level specified in item 8. The BER shall be better than 10-5. | adjacent channel is defined per group (see 2b) | closed |
| 12 | Tx | Channel spectrum mask | -30 dBc @ Df³±11Mhz -50 dBc @ Df³±22Mhz | includes all transmit spectral products | closed |
| 13a | Tx | Transmitter Centre Frequency tolerance | ±25 ppm Type I: 0 to 40 oC Type II: -30 to 70 oC |  | closed |
| 14 | T&R | Modulation | DBPSK at 1 Mb/s DQPSK at 2 Mb/s | see appendix for specifications | closed |
| 14a |  |  |  |  | deleted |
| 15 | T&R | Channel data rate | 1 Mb/s and 2 Mb/s |  | closed |
| 17a | Tx | Chip clock frequency accuracy | £ 50 ppm |  | closed |
| 18 | Tx | Preamble length | 192 bits sent at 1 Mb/s |  | closed |
| 18a | Tx | Preamble content | A: synchronization field: 128 bits of 'ones' B: unique word: 16 bits (Octal 2717) C: 802.11 signal bits: 11000000 1Mb/s DBPSK 00110000 2Mb/s DQPSK D:Service bits: 8 bits all 'zeros' for 802.11 E:16 bit Length (in octets) F:16 bit CRC applied over C,D,E polynomial 1+ x5+x12+x16 (CCITT CRC16) |  | closed |
| 18b |  |  |  |  | deleted |
| 18c | T&R | Rate change | All symbols following the PHY preamble shall be at the rate indicated in 802.11 signal bits. |  | closed |
| 20 | Rx | Energy detect time | £ 20 µs | Maximum response time from energy incident upon receiver antenna to receiver signal level crossing the threshold (20a) | closed |
| 20a | Rx | Energy detection threshold | TBD |  | open |
| 22 | T&R | Out and in band spurious emissions | Per FCC part 15.247,15.205 and 15.209 in USA per ETSI RES 02-09 in Europe |  | closed |
| 22a |  |  |  |  | deleted |
| 23 | Tx | Transmit power off and transmit power on | £ 2 µs | Tx power off ramp from 90% to 10% and Tx power ramp on from 10% to 90% | closed |
| 23a |  |  |  |  | deleted |
| 23b | T&R | Transmit to Receive turnaround time | £ 25 µs | Measured from transition of exposed Rx/Tx control line | closed |
| 24 | T&R | Switching time Rx to Tx | £10 µs | Time from transition of exposed Rx/Tx control line till 90% full power measured at antenna | closed |
| 24a |  |  |  |  | deleted |
| 26 | Rx | Frame capture ratio | 10 dB | Given a continuous interferer at -50 dBm, an error rate of 10-5 shall be maintained from a desired packetized source of -40 dBm. Both signals shall be of DS modulation. | closed |
| 27 | Rx | BER at specified Eb/No | 10-5 @ Eb/No = 17 dB |  | closed |
| 28 | T&R | Channel availability | 99.5% | Could also be specified as probability of outage; with no interference. From the PAR. | deleted |
| 29 |  | Data Line/clock input/output jitter |  | MAC/PHY interface issue | deleted |
| 30 | T&R | Antenna port impedance (if exposed) | 50 ohms |  | closed |
| 31 | T&R | VSWR | Devices shall withstand no damage and remain unconditional stable over the range 0£VSWR£¥. |  | closed |
| 32 |  | Interface lines to convergence layer (when exposed) |  | refer to doc: IEEE P802.11-94/61 | open |
| 33 |  | PHY-MAC Net management info/control variables |  | refer to doc: IEEE P802.11-94/61 | open |
| 34 |  | Other PHY-MAC Net management info/control variables |  | refer to doc: IEEE P802.11-94/61 | open |
| 35 |  | Safety Requirements |  |  | open |
| 36 |  | DTE/DCE Interface |  | refer to doc: IEEE P802.11-94/61 | open |

Appendix

This appendix gives additional descriptions of the items in the template.

# ITEM 2B Channelization

5 different channels are supported  divided in two groups.

The centre frequencies of these channels are:

group 1:

channelnumber 1a: 2412 MHz

channelnumber 1b: 2442 MHz

channelnumber 1c: 2472 MHz

group 2:

channelnumber 2a: 2427 MHz

channelnumber 2b: 2457 MHz

A group 1 channel  can be used simultaneously with any other group 1 channel with no channel interference in adjacent cells.  A group 2 channel can be used simultaneously with the other group 2 channel.

Group 1 and group 2 channels may be used simultaneously in a cellular frequency plan where interference effects have been factored into a cell plan.

ITEM 3C  Data Scrambler

Data Scrambler implementation :

µ §

Figure 1 : Scrambler implementation; polynomial 1+x-4+x-7

# ITEM 14 Modulation parameters for DQPSK and DBPSK

| dibit pattern (d0,d1) (d0, first in time) | Phase jump |
| --- | --- |
| 0, 0 | 0 |
| 0 ,1 | p/2 |
| 1 ,1 | p |
| 1 ,0 | -p/2 |

Table 1. Mapping for DQPSK

Data dibit to differential phase mapping

| bit | Phase jump |
| --- | --- |
| 0 | 0 |
| 1 | p |

Table 2. Mapping for DBPSK

Bit to differential phase mapping