# Knobs, Dials and Sliders

- or -

the Management Information Base

Bob O'Hara

Advanced Micro Devices, Inc.

I/O and Networks Division

One AMD Place,  MS 70

Sunnyvale, CA  94088

Tel: +1 408 987 2421

Fax: +1 408 987 2814

Email: bob.ohara@amd.com

Abstract:  Al description of the structure and definition of the Management Information Base is presented in the format defined in ISO 10165-4, Guideline for the Definition of Managed Objects.

Action:  Adopt motion the contents of this submission as partial definition of the MIB and place it in the draft standard.

# 1	Managment Information Definitions

## 2	MIB Summary

The following sections summarize the 802.11 Management Information Base (MIB).  Each group, attribute, action and notification is listed.  This summary is for information purposes only.  If any errors exist, the formal definitions have precedence.

### 3	Station Management Attributes

#### 4	agStation_Config_grp

aActing_as_AP_Status,

aAssociated_State,

aBeacon_Period,

aPower_Mgt_State,

aPower_Mgt_Capability;

#### 5	agAuthentication_grp

aAuthentication_Algortihms,

aSelected_Authentication_Algorithm,

aAuthentication_Handshake_State,

aAuthentication_State,

aMin_Authentication_Required;

#### 6	agPrivacy_grp

aPrivacy_Algortihms,

aSelected_Privacy_Algorithm,

aPrivacy_Handshake_State,

aPrivacy_State,

aMin_Privacy_Required;

#### 7	Not Grouped

aStation_ID

aCurrent_BSS_ID

aCurrent_ESS_ID

aKnown_APs

### 8	MAC Attributes

#### 9	agAddress_grp

aMAC_Address,

aGroup_Addresses;

#### 10	agOperation_grp

aNAV,

aNAV_max,

aRate_Factor,

aHandshake_Overhead,

aSIFS,

aPIFS,

aDIFS,

aRTS_Threshold,

aSlot_Time,

aCW_max,

aCW_min,

aCTS_Time,

aACK_Time,

aRetry_max,

aMax_Frame_Length;

#### 11	agCounters_grp

aTransmitted_Frame_Count,

aOctets_Transmitted_Count,

aMulticast_Transmitted_Frame_Count,

aBroadcast_Transmitted_Frame_Count,

aFailed_Count,

aCollision_Count,

aSingle_Collision_Count,

aMultiple_Collision_Count,

aReceived_Frame_Count,

aOctets_Received_Count,

aMulticast_Received_Count,

aBroadcast_Received_Count,

aError_Count,

aFCS_Error,Count,

aLength_Mismatch_Count,

aFrame_Too_Long_Count,

aTotal_Backoff_Time;

#### 12	agStatus_grp

aMAC_Enable_Status,

aTransmit_Enable_Status,

aPromiscuous_Status;

#### 13	Not Grouped

aManufacturer_ID

aProduct_ID

### 14	PHY Attributes

#### 15	agConfig_grp

#### 16	agOperation_grp

#### 17	agError_Counters_grp

#### 18	agBER_grp

#### 19	agStatus_grp

### 20	ResourceTypeID Attributes

#### 21	Not Grouped

aResourceTypeIDName

aResourceInfo

### 22	Actions

#### 23	SMT Actions

acStation_init

acStation_reset

#### 24	MAC Actions

acMAC_init

acMAC_reset

#### 25	PHY Actions

acPHY_init

acPHY_reset

### 26	Notifications

#### 27	SMT Notifications

nAssociate

nDissociate

#### 28	MAC Notifications

nFrame_Error_Rate_Exceeded

#### 29	PHY Notifications

nBER_Exceeded

## 30	Managed Object Class Templates

### 31	SMT Object Class

#### 32	oSMT

SMT MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pSMT_base	PACKAGE

BEHAVIOUR

bSMT_base BEHAVIOUR

DEFINED AS "The SMT object class provides the necessary support at the station to manage the processes in the station such that the station may work cooperatively as a part of an 802.11 network.";

ATTRIBUTES

aStation_ID	GET,

aActing_as_AP_Status	GET,

aCurrent_AP_MAC_Address	GET,

aCurrent_BSS_ID	GET,

aCurrent_ESS_ID	GET,

aKnown_APs	GET,

aAuthentication_Algorithms	GET,

aPrivacy_Algorithms	GET,

aSelected_Authentication_Algorithm	GET,

aSelected_Privacy_Algorithm	GET,

aAuthentication_Handshake_State	GET,

aPrivacy_Handshake_State	GET,

aAuthentication_State	GET,

aPrivacy_State	GET,

aMin_Authentication_Required	GET,

aMin_Privacy_Required	GET,

aAssociated_State	GET,

aBeacon_Period	GET-REPLACE,

aPower_Mgt_State	GET,

aPower_Mgt_Capability	GET;

ATTRIBUTE GROUPS

agStation_Config_grp,

agAuthentication_grp,

agPrivacy_grp;

ACTIONS

acSMT_init,

acSMT_reset;

NOTIFICATIONS

nAssociate,

nDissociate;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) smt(0) };

### 33	MAC Object Class

#### 34	oMAC

MAC MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pMAC_base	PACKAGE

BEHAVIOUR

bMAC_base BEHAVIOUR

DEFINED AS "The MAC object class provides the necessary support for the access control, generation and verification of frame check sequences, and proper delivery of valid data to upper layers.";

ATTRIBUTES

aMAC_Address	GET,

aGroup_Addresses	GET-REPLACE,

aPromiscuous_Status	GET,

aTransmitted_Frame_Count	GET-REPLACE,

aOctets_Transmitted_Count	GET-REPLACE,

aMulticast_Transmitted_Frame_Count	GET-REPLACE,

aBroadcast_Frame_Count	GET-REPLACE,

aFailed_Count	GET-REPLACE,

aCollision_Count	GET-REPLACE,

aSingle_Collision_Count	GET-REPLACE,

aMultiple_Collision_Count	GET-REPLACE,

aReceived_Frame_Count	GET-REPLACE,

aOctets_Received_Count	GET-REPLACE,

aMulticast_Received_Frame_Count	GET-REPLACE,

aBroadcast_Received_Frame_Count	GET-REPLACE,

aError_Count	GET-REPLACE,

aFCS_Error_Count	GET-REPLACE,

aLength_Mismatch_Count	GET-REPLACE,

aFrame_Too_Long_Count	GET-REPLACE,

aMAC_Enable_Status	GET,

aTransmit_Enable_Status	GET,

aNAV	GET,

aNAV_max	GET,

aRate_Factor	GET,

aHandshake_Overhead	GET,

aSIFS	GET,

aPIFS	GET,

aDIFS	GET,

aRTS_Threshold	GET-REPLACE,

aTotal_Backoff_Time	GET-REPLACE,

aSlot_Time	GET,

CW_max	GET-REPLACE,

aCW_min	GET-REPLACE,

aCTS_Time	GET,

aACK_Time	GET,

aRetry_max	GET-REPLACE,

aMax_Frame_Length	GET,

aManufacturer_ID	GET,

aProduct_ID	GET;

ATTRIBUTE GROUPS

agCapabilities_grp,

agConfig_grp,

agAddress_grp,

agOperation_grp,

agCounters_grp,

agFrame_Error_Condition_grp,

agStatus_grp;

ACTIONS

acMAC_init,

acMAC_reset;

NOTIFICATIONS

nFrame_Error_Rate_Exceeded;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) mac(1) };

### 35	PHY Object Class

#### 36	oPHY

PHY MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pPHY_base	PACKAGE

BEHAVIOUR

bPHY_base BEHAVIOUR

DEFINED AS "The PHY object class provides the necessary support at the station to manage the physical layer.";

ATTRIBUTES

aPHY_Type	GET,

aPHY_Data_Rate	GET,

aChannel_Capability	GET,

aCurrent_Channel	GET,

aChannel_List	GET,

aDiversity_Capability	GET,

aBER_Estimate	GET,

aPHY_Turnaround_Time	GET,

aManufacturer_ID	GET,

aProduct_ID	GET;

ATTRIBUTE GROUPS

agConfig_grp,

agOperation_grp,

agBER_grp,

agStatus_grp;

ACTIONS

acPHY_init,

acPHY_reset;

NOTIFICATIONS

nBER_Exceeded;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(2) };

### 37	Resource Type Object Class

#### 38	oResourceTypeID

ResourceTypeID MANAGED OBJECT CLASS

DERIVED FROM IEEE802CommonDefinitions.oResourceTypeID;

CHARACTERIZED BY

pResourceTypeID	PACKAGE

ATTRIBUTES

aResourceTypeIDName	GET,

aResourceInfo	GET;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) resourcetypeid(3) };

## 39	Attribute Group Templates

### 40	Station Management Attribute Group Templates

#### 41	agStation_Config_grp

Station_Config_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aActing_as_AP_Status,

aAssociated_State,

aBeacon_Period,

aPower_Mgt_State,

aPower_Mgt_Capability;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) smt(0) station_config_grp(0) };

#### 42	agAuthentication_grp

Authentication_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aAuthentication_Algortihms,

aSelected_Authentication_Algorithm,

aAuthentication_Handshake_State,

aAuthentication_State,

aMin_Authentication_Required;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) smt(0) authentication_grp(1) };

#### 43	agPrivacy_grp

Privacy_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aPrivacy_Algortihms,

aSelected_Privacy_Algorithm,

aPrivacy_Handshake_State,

aPrivacy_State,

aMin_Privacy_Required;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) smt(0) privacy_grp(2) };

### 44	MAC Attribute Group Templates

#### 45	agAddress_grp

Address_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aMAC_Address,

aGroup_Addresses;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) mac(0) address_grp(0) };

#### 46	agOperation_grp

Operation_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aNAV,

aNAV_max,

aRate_Factor,

aHandshake_Overhead,

aSIFS,

aPIFS,

aDIFS,

aRTS_Threshold,

aSlot_Time,

aCW_max,

aCW_min,

aCTS_Time,

aACK_Time,

aRetry_max,

aMax_Frame_Length;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) mac(0) operation_grp(1) };

#### 47	agCounters_grp

Counters_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aTransmitted_Frame_Count,

aOctets_Transmitted_Count,

aMulticast_Transmitted_Frame_Count,

aBroadcast_Transmitted_Frame_Count,

aFailed_Count,

aCollision_Count,

aSingle_Collision_Count,

aMultiple_Collision_Count,

aReceived_Frame_Count,

aOctets_Received_Count,

aMulticast_Received_Count,

aBroadcast_Received_Count,

aError_Count,

aFCS_Error,Count,

aLength_Mismatch_Count,

aFrame_Too_Long_Count,

aTotal_Backoff_Time;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) mac(0) counters_grp(2) };

#### 48	agStatus_grp

Status_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aMAC_Enable_Status,

aTransmit_Enable_Status,

aPromiscuous_Status;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) mac(0) status_grp(3) };

### 49	PHY Attribute Group Templates

#### 50	agConfig_grp

Config_grp ATTRIBUTE GROUP

GROUP ELEMENTS

a;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(0) config_grp(0) };

#### 51	agOperation_grp

Operation_grp ATTRIBUTE GROUP

GROUP ELEMENTS

a;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(0) operation_grp(1) };

#### 52	agError_Counters_grp

Error_Counters_grp ATTRIBUTE GROUP

GROUP ELEMENTS

a;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(0) error_counters_grp(2) };

#### 53	agBER_grp

BER_grp ATTRIBUTE GROUP

GROUP ELEMENTS

a;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(0) BER_grp(3) };

#### 54	agStatus_grp

Status_grp ATTRIBUTE GROUP

GROUP ELEMENTS

a;

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(0) status_grp(4) };

## 55	Attribute Templates

### 56	SMT Attribute Templates

#### 57	aStation_ID

Station_ID ATTRIBUTE

DERIVED FROM

IEEE802CommonDefinitions.MACAddress;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) station_id(0) };

#### 58	aActing_as_AP_Status

Acting_as_AP_Status ATTRIBUTE

WITH APPROPRIATE SYNTAX

boolean;

BEHAVIOUR DEFINED AS

"True if this station is acting as an access point, false otherwise.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) acting_as_ap_status(4) };

#### 59	aCurrent_AP_MAC_Address

Current_AP_MAC_Address ATTRIBUTE

DERIVED FROM

IEEE802CommonDefinitions.MACAddress;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) ap_address(5) };

#### 60	aCurrent_BSS_ID

Current_BSS_ID ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute shall identify the basic service set (BSS) with which the station is currently associated.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) current_bss_id(6) };

#### 61	aCurrent_ESS_ID

Current_ESS_ID ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute shall identify the extended service set (ESS) with which the station is associated, if any.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) current_ess_id(7) };

#### 62	aKnown_APs

Known_APs ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of AP_ID.type;

[ BOB- A sequence of 32 data structures that including the MAC address, BSS_ID, ESS_ID, signal strength, age, detach reason, BER + history, FER + history, Missed beacons, "cost" and PHY specific information required to identify the most recent 32 access points encountered.  This data type needs to be defined in the ASN.1]

BEHAVIOUR DEFINED AS

"This attribute shall be a set of the identities of the most recently known Access Points.  The Access Point with which the station is currently associated, if any, shall always be the first element of the set.  Access Points may be included in this list even if the station did not associate with them.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) known_aps(8) };

#### 63	aAuthentication_Algorithms

Authentication_Algorithms ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOUR DEFINED AS

"This attribute shall be a set of all the authentication algorithms supported by the stations.  The values of the numbers in the list are as defined in IEEE Standard 802.10.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) authentication_algorithms(9) };

#### 64	aPrivacy_Algorithms

Privacy_Algorithms ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of integer;

BEHAVIOUR DEFINED AS

"This attribute shall be a set all of the privacy algorithms supported by the stations.  The values of the numbers in the list are as defined in IEEE Standard 802.10.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) privacy_algorithms(10) };

#### 65	aSelected_Authentication_Algorithm

Selected_Authentication_Algorithm ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute shall indicate the authentication algorithm identifer selected during the authentication negotiation.  The value of this attribute shall be selected from the set in the aAuthentication_Algorithms attribute.  The value of this attribute shall reference one of the authentication algorithm identifiers defined in IEEE Standard 802.10.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) selected_authentication_algorithm(11) };

#### 66	aSelected_Privacy_Algorithm

Selected_Privacy_Algorithm ATTRIBUTE

WITH APPOPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute shall indicate the privacy algorithm identifer selected during the privacy negotiation.  The value of this attribute shall be selected from the set in the aPrivacy_Algorithms attribute.  The value of this attribute shall reference one of the privacy algorithm identifiers defined in IEEE Standard 802.10.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) selected_privacy_algorithm(12) };

#### 67	aAuthentication_Handshake_State

Authentication_Handshake_State ATTRIBUTE

WITH APPROPRIATE SYNTAX

authentication_handshake.type

[BOB: this data type needs to be defined in the ASN.1 to enumerate the handshake states in the authentication process.]

BEHAVIOUR DEFINED AS

"This attribute shall identify the current state of the station in the authentication process.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) authentication_handshake_state(13) };

#### 68	aPrivacy_Handshake_State

Privacy_Handshake_State ATTRIBUTE

WITH APPROPRIATE SYNTAX

privacy_hanshake.type;

[BOB: this data type needs to be defined in the ASN.1 to enumerate the handshake states in the privacy process.]

BEHAVIOUR DEFINED AS

"This attribute shall identify the current state of the station in the privacy negotiation process.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) privacy_handshake_state(14) };

#### 69	aAuthentication_State

Authentication_State ATTRIBUTE

WITH APPROPRIATE SYNTAX

authentication_state.type;

[BOB: this data type needs to be defined in the ASN.1]

BEHAVIOUR DEFINED AS

"This attribute shall indicate the authentication state.";

[BOB: between what?]

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) authentication_state(15) };

#### 70	aPrivacy_State

Privacy_State ATTRIBUTE

WITH APPROPRIATE SYNTAX

privacy_state.type;

[BOB: this one needs to be defined as well.]

BEHAVIOUR DEFINED AS

"This attribute shall indicate the current privacy state.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) privacy_state(16) };

#### 71	aMin_Authentication_Required

Min_Authentication_Required ATTRIBUTE

WITH APPROPRIATE SYNTAX

Authentication_Required.type;

[BOB: this type needs to be defined in the ASN.1.  Should be an enumerated type with none, me, you, both]

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) min_authentication_required(17) };

#### 72	aMin_Privacy_Required

Min_Privacy_Required ATTRIBUTE

WITH APPROPRIATE SYNTAX

Privacy_Required.type;

[BOB: this data type needs to be defined in the ASN.1.  Should be an enumerated tyoe with none, me, you, both.]

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) min_privacy_required(18) };

#### 73	aAssociated_State

Associated_State ATTRIBUTE

WITH APPROPRIATE SYNTAX

Associated_State.type;

[BOB: this data type needs to be defined in the ASN.1.]

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) associated_state(19) };

#### 74	aBeacon_Period

Beacon_Period ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR

"The beacon period shall indicate the time, in nanoseconds, between the transmission of beacon frames if the station is acting as an Access Point.  If the station is not an Access Point but is associated with one, the beacon period shall indicate the time, in nanoseconds, between the expected arrival of beacon frames.  If the station is not an Access Point and is not associated with one, the beacon period shall indicate the time, in nanoseconds, between the transmission of beacon frames.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) beacon_period(20) };

#### 75	aPower_Mgt_State

Power_Mgt_State  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Power_Mgt_State.type;

[This attribute needs an enumerated data type that includes all possible power management states.  It must be defined in the ASN.1 - Bob]

BEHAVIOUR DEFINED AS

"An enumerated type that describes the current power management state of the station.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) power_mgt_state(21) };

#### 76	aPower_Mgt_Capability

Power_Mgt_Capability  ATTRIBUTE

WITH APPROPRIATE SYNTAX

set of Power_Mgt_State.type;

BEHAVIOUR DEFINED AS

"An enumerated type that describes all the possible power management states of which the station is capable.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) SMT(0) attribute(7) power_mgt_capability(22) };

### 77	MAC Attribute Templates

#### 78	aMAC_Address

MAC_Address ATTRIBUTE

DERVIVED FROM

IEEE802CommonDefinitions.MACAddress;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) mac_address(0) };

#### 79	aGroup_Addresses

Group_Addresses ATTRIBUTE

WITH APPROPRIATE SYNTAX

set-of IEEE802CommonDefinitions.MACAddress;

BEHAVIOUR DEFINED AS

"A set of xxx MAC_Addresses identifying the multicast addresses for which this station will receive frames."

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) group_addresses(1) };

#### 80	aPromiscuous_Status

Promiscuous_Status ATTRIBUTE

WITH APPROPRIATE SYNTAX

boolean;

BEHAVIOUR DEFINED AS

"This attribute is true when the station is enabled to receive all frames promiscuously.  It is false otherwise.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) promiscuous_status(2) };

#### 81	aTransmitted_Frame_Count

Transmitted_Frame_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":pdusSentCounter;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) transmitted_frame_count(3) };

#### 82	aOctets_Transmitted_Count

Octets_Transmitted_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":octetsSentCounter;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) octets_transmitted_count(4) };

#### 83	aMulticast_Transmitted_Frame_Count

Multicast_Transmitted_Frame_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":pdusSentCounter;

BEHAVIOUR DEFINED AS

"This counter shall increment only when the multicast/broadcast bit is set in the destination MAC address and the destination MAC address is not the broadcast address.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) multicast_transmitted_frame _count(5) };

#### 84	aBroadcast_Transmitted_Frame_Count

Broadcast_Transmitted_Frame_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":pdusSentCounter;

BEHAVIOUR DEFINED AS

"This counter shall increment only when the destination MAC address is the broadcast address.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) broadcast_transmitted_frame_count(6) };

#### 85	aFailed_Count

Failed_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":counter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a frame is not transmitted due to the number of transmit attempts exceeding the retry_max value.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) failed_count(7) };

#### 86	aCollision_count

Collision_count ATTRIBUTE

DERIVED FROM;

"ISO/IEC 10165-2":counter

BEHAVIOUR DEFINED AS

"This counter shall increment when a collision is detected.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) collision_count(8) };

#### 87	aSingle_Collision_Count

Single_Collision_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":counter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a frame is successfully transmitted after a single collision.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) single_collision_count(9) };

#### 88	aMultiple_Collision_Count

Multiple_Collision_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":counter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a frame is successfully transmitted after more than one collision.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) multiple_collision_count(10) };

#### 89	aReceived_Frame_Count

Received_Frame_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":pdusReceivedCounter;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) received_frame_count(11) };

#### 90	aOctets_Received_Count

Octets_Received_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":octetsReceivedCounter;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) octets_received_count(12) };

#### 91	aMulticast_Received_Frame_Count

Multicast_Received_Frame_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":pdusReceivedCounter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a frame is received with the multicast/broadcast bit set in the destination MAC address, the destnation MAC address is not the broadcast address and the destination address is in the set of Group_Addresses.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) multicast_received_frame_count(13) };

#### 92	aBroadcast_Received_Frame_Count

Broadcast_Received_Frame_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":pdusReceivedCounter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a frame is received with the destination MAC address equal to the broadcast address.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) broadcast_received_frame_count(14) };

#### 93	aError_Count

Error_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":courruptedPDUsReceivedCounter;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) error_count(15) };

#### 94	aFCS_Error_Count

FCS_Error_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":counter;

BEHAVIOUR DEFINED AS

"This counter shall increment when an FCS error is detected in a received frame.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) fcs_error_count(16) };

#### 95	aLength_Mismatch_Count

Length_Mismatch_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":counter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a frame is received and the number of bytes in the frame does not equal the value in the length field of the frame.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) length_mismatch_count(17) };

#### 96	aFrame_Too_Long_Count

Frame_Too_Long_Count ATTRIBUTE

DERIVED FROM

"ISO/IEC 10165-2":counter;

BEHAVIOUR DEFINED AS

"This counter shall increment when a received frame that exceeds Max_Frame_Length is detected.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) frame_too_long_count(18) };

#### 97	aMAC_Enable_Status

MAC_Enable_Status ATTRIBUTE

WITH APPROPRIATE SYNTAX

boolean;

BEHAVIOUR DEFINED AS

"This attribute is true when the MAC sublayer is enabled.  It is false otherwise.  Setting this attribute true causes the MAC to become operational in the idle state.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) mac_enable_status(19) };

#### 98	aTransmit_Enable_Status

Transmit_Enable_Status ATTRIBUTE

WITH APPROPRIATE SYNTAX

boolean;

BEHAVIOUR DEFINED AS

"This attribute is true when transmission is enabled.  It is false otherwise.  Setting this attribute to true allows the MAC to transmit frames.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) transmit_enable_status(20) };

#### 99	aNAV

NAV ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the amount of time remaining that the station will consider the medium to be in use by another station.  This attribute is updated whenever there is a change in the MAC network allocation vector.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) nav(21) };

#### 100	aNAV_max

NAV_max ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This is the maximum allowable value for the NAV.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) nav_max(22) };

#### 101	aRate_Factor

Rate_Factor ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the current rate (in bytes per second) at which data is transferred across the medium.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) rate_factor(23) };

#### 102	aHandshake_overhead

Handshake_overhead ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute is the amount of time required to complete an RTS/CTS handshake.  This value, along with the Rate_Factor, may be used to determine the desirable setting of the RTS_Threshold to maximize data throughput.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) handshake_overhead(24) };

#### 103	aSIFS

SIFS ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the length of the short interframe space.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) sifs(25) };

#### 104	aPIFS

PIFS ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the length of the priority interframe space.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) pifs(26) };

#### 105	aDIFS

DIFS ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the length of the distributed interframe space.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) difs(27) };

#### 106	aRTS_Threshold

RTS_Threshold ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the number of bytes in an MSDU, below which an RTS/CTS handshake will not be performed.  An RTS/CTS handshake shall be performed for all frames where the length of the MSDU is larger than this threshold.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) rts_threshold(28) };

#### 107	aTotal_Backoff_Time

Total_Backoff_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the length of time, in number of slots, the MAC has spent in a backoff condition.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) total_backoff_time(29) };

#### 108	aSlot_time

Slot_time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute is the length of a single slot, in nanoseconds.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) slot_time(30) };

#### 109	aCW_max

CW_max ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the maximum size of the contention window, in slots.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) cw_max(31) };

#### 110	aCW_min

CW_min ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the minimum size of the contention window, in slots.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) cw_min(32) };

#### 111	aCTS_Time

CTS_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the length of time it takes to transmit a CTS frame.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) cts_time(33) };

#### 112	aACK_Time

ACK_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the length of time it takes to transmit an ACK frame.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) rts_time(34) };

#### 113	aRetry_max

Retry_max ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates tha maximum number of transmission attempts that will be made before a failure condition is indicated.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) retry_max(35) };

#### 114	aMax_Frame_Length

Max_Frame_Length ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute specifies the maximum MSDU length that will be accepted for transmission.  If a frame is received with a length that exceeds this value, a Frame_Too_Long error will be reported.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) max_frame_length(36) };

#### 115	aManufacturer_ID

Manufacturer_ID ATTRIBUTE

WITH APPROPRIATE SYNTAX

octet string;

BEHAVIOUR DEFINED AS

"The Manufacturer_ID shall include, at a minimum, the name of the manufacturer.  It may include additional information at the manufacturer's discretion.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) manufacturer_id(37) };

#### 116	aProduct_ID

Product_ID ATTRIBUTE

WITH APPROPRIATE SYNTAX

octet string;

BEHAVIOUR DEFINED AS

"The Product_ID shall include, at a minimum, an identifier that is unique to the manufacturer.  It may include additional information at the manufacturer's discretion.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) MAC(1) attribute(7) product_id(38) };

### 117	PHY Attribute Templates

#### 118	aPHY_Type

PHY_Type ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"The PHY_Type shall be uniquely assigne to each different PHY.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) phy_type(0) };

#### 119	aPHY_Data_Rate

PHY_Data_Rate ATTRIBUTE

WITH APPROPRIATE SYNTAX

integer;

BEHAVIOUR DEFINED AS

"This attribute indicates the physical rate at which bits are transmitted by the PHY.  The rate is given in bits per second.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) phy_data_rate(1) };

#### 120	aChannel_Capability

Channel_Capability ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) channel_capability(2) };

#### 121	aCurrent_Channel

Current_Channel ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) curretn_channel(3) };

#### 122	aChannel_List

Channel_List ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) channel_list(4) };

#### 123	aDiversity_Capability

Diversity_Capability ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) diversity_capabililty(5) };

#### 124	aBER_Estimate

BER_Estimate ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) ber_estimate(7) };

#### 125	aPHY_Turnaround_Time

PHY_Turnaround_Time ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) phy_turnaround_time(8) };

#### 126	a(other stuff from Michael)

(other stuff from Michael) ATTRIBUTE

WITH APPROPRIATE SYNTAX

;

BEHAVIOUR DEFINED AS

;

REGISTERED AS

{  };

#### 127	aManufacturer_ID

Manufacturer_ID ATTRIBUTE

WITH APPROPRIATE SYNTAX

octet string;

BEHAVIOUR DEFINED AS

"The Manufacturer_ID shall include, at a minimum, the name of the manufacturer.  It may include additional information at the manufacturer's discretion.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) manufacturer_id(10) };

#### 128	aProduct_ID

Product_ID ATTRIBUTE

WITH APPROPRIATE SYNTAX

octet string;

BEHAVIOUR DEFINED AS

"The Product_ID shall include, at a minimum, an identifier that is unique to the manufacturer and appropriate regulatory body registration information.  It may include additional information at the manufacturer's discretion.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) product_id(11) };

### 129	Resource Type Attribute Templates

#### 130	aResourceTypeIDName

ResourceTypeIDName ATTRIBUTE

DERIVED FROM

IEEE802CommonDefinitions.ResourceTypeIDName;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) ResourceTypeID(3) attribute(7) resourcetypeidname(0) };

#### 131	aResourceInfo

ResourceInfo ATTRIBUTE

DERIVED FROM

IEEE802CommonDefinitions.ResourceInfo;

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) ResourceTypeID(3) attribute(7) resourceinfo(1) };

## 132	Action Templates

### 133	SMT Action Templates

#### 134	acInitialize_SMT

Initialize_SMT ACTION

### 135	MAC Action Templates

#### 136	acInitialize_MAC

#### 137	acAdd_Group_Address

#### 138	acDelete_Group_Address

#### 139	acExecute_Self_Test

### 140	PHY Action Templates

#### 141	acInitialize_PHY

#### 142	acExecute_Self_Test

## 143	Notification Templates

### 144	SMT Notification Templates

#### 145	nAssociate

#### 146	nDisassociate

### 147	MAC Notification Templates

#### 148	nFrame_Error_Rate_Exceeded

### 149	PHY Notification Templates

#### 150	nBER_Exceeded

## 151	ASN.1 Definitions

### 152	Common Definitions

### 153	SMT Definitions

### 154	MAC Definitions

### 155	PHY Definitions

## 156	Name Binding

### 157	MAC Naming

### 158	PHY Naming

Motion: I move that the definition of the MIB as described in this submission be adopted and placed in the draft standard.