IEEE 802.11

WIRELESS ACCESS METHODS AND PHYSICAL LAYER SPECIFICATIONS

Title		Infrared Components Characteristics for

EXIRLAN-FQPSK and other Wireless Systems

Authors	Helmut Kindl, Werner Just		Larry Sisken, Jonathan Wafer

Siemens, Semiconductor Div.		Siemens Components, Inc.

Munich, Germany				Optoelectronics Division

Cupertino, CA - USA

1. Summary

For the EXIRLAN-Expandable Infrared Local Area Network [ref 1 - 5], currently being standardized by the IEEE 802.11 Wireless Local Area Networks committee (Ref Docu-

ment IEEE P802.11-94/65R-Template, editor P. Blomeyer March 1994) as well as

considered by other European and international organizations we describe the characteristics

of typical readily available, low cost and power efficient Infrared transmit and receive diodes suitable for implementation of these FQPSK systems [ref 6 - 17] in the 1 Mb/s to 10 Mb/s

range and could be used even for higher data rates.

As stated in Siemens submission IEEE 802.11-94/71 of March 1994 [ref 18] currently Siemens

is considering even a higher level of system and component integration than the components/

characteristics described in this paper. This ongoing development program is going to be further intensified and speeded up as soon as the committees finalize the FQPSK based Exirlan standards.

Low cost/high power Infrared transmit diodes in the DC to 30 MHz range are the most practical for the first generations of robust implementations.

2. 	Characteristics of INFRARED COMPONENTS

2.1. 	Description of  typical available transmitter diodes

IR emitters proposed for IR-wireless applications differ by chip and by package. The chip

determines wavelength, speed, power etc. whereas the package may influence light ex-

traction efficiency and power dissipation/max. current rating. Due to its high extraction

efficiency and lower cost we have chosen the standard T 1 3/4 (=5 mm) plastic package

for a comparison of various emitter chip types. Table 1 shows three high-speed emitters,

compared by some key data:

SFH 477 is a typical GaAlAs Double Hetero junction diode on a transparent substrate

with 880 nm wavelength and 25 mW/@100 mA powerlevel and a beam width of less than

20 degrees (see fig. 1), which will suffice for transmission up to about 10 meters, depen-

ding on receiver design. 100 mA forward current is the max. permissible DC-value, given

by the thermal limitations of 165 mW maximum power dissipation of this package,

whereas pulsed operation up to 2 amps peak is permitted. It should work well in EXIRLAN

applications up to about 12 MHz. Its wavelength fits well to the maximum of the spectral

response curve of silicon photodiodes (see fig. 2).

Fig. 2: Relative sensitivity of different light-sensitive detectors

Some detailed data can be seen in table 2. Except power, wavelength, speed and current

handling capability, the other important factors are forward voltage VF and beam angle resp.

radiant intensity Ie. A low VF is important with respect to power dissipations of the emitter

itself and in the drive circuitry. As long as point to point setups of the communication partners are desired, a narrow beam angle should be preferred and consequently a high radiant intensity Ie

thereby obtaining high signal to noise ratio at large distances and for high interference immunity.

The price for this is a fixed setup that needs some adjustment and that should not be interrupted

by obstacles. For "room filling" applications, a wider beam angle is desirable and can be pro-

vided by a lensless package containing the same chip, e.g. PL-CC2 see SFH 423/417 in fig 3.

The price there is a low radiant intensity of 4 mW/sr with all the other data being the same. As

with all small high power optical emitters, care should be taken in the practitcal implementation of a system, to comply with the safety requirements of IEC 825 which make basically no difference between stimulated emitters (lasers) and spontaneous emitters.

SFH 495 P is a stimulated emitter with excellent speed, good for up to 30 MHz, high power

efficiency and low forward voltage at high currents, making it an excellent choice for pulsed high

speed baseband systems, but only for low duty cycles since its threshold of about 100 mA forbids any use in a CW-mode with reasonable power output. Therefore it will not be con-

sidered for EXIRLAN in this paper.

F 237 A is a GaAlAs Double hetero junction on a transparent substrate, like SFH 477/423/427 but more optimized for high power output at moderate currents (e.g. 100 mA), but with some compromise in  speed, making it an optimum solution for EXIRLAN applications up to about 10 MHz. The high forward voltage at 1 A is a drawback only in high-current pulsed applications whereas at moderate currents, it does not exceed the limits set by 5 V circuitry and by its thermal dissipation capability.

2.2.	Future developments of IR-Emitters

GaAlAs hetero junction devices, in the 800 - 900 nm range, seem to have some future potential in speed a well as in power. Experimental devices with rise/fall times tr/tf of 10 ns at 20 mW have been implemented, and commercial devices may be expected in due course of time. Nevertheless the trade off between maximum power efficiency and highest speed seems to be permanent in this class of diodes.

Another direction of future progress may be the reduction of threshold currents in stimulated emitters, as a spin-off from optical fiber communication developments, possibly leading to more efficient and higher speed devices to support EXIRLAN. Eventually this may be enhanced by the

application of packages with higher thermal conductivity (higher priced, though). The third aspect for future IR-LANs will be the application of various wavelengths e.g. 880 - 1300 - 1500 nm. It may be expected that, particularly for point to point communication, efficient and low cost filtering techniques will become available, thereby expanding the effective available handwidth.

REFERENCES

1] 	K. Feher: "Infrared EXIRLAN FQPSK proposed flexible standard," IEEE 802.11-	94/55,  	March, 1994.

[2] 	P. Blomeyer: "Revised version of the combined baseband and multichannel IR-PHY 	EXIRLAN," IEEE P.802.11-94/62.

[3]	P. Blomeyer: "Implementation of EXIRLAN multichanneI IR PHY using existing 	commodity components," IEEE P.802.11 -94/63.

[4]	P. Blomeyer: "Compatibility issues between existing IR techniques and present/future 	requirements," IEEE P.802.11-94/64.

[5]	P.Blomeyer: "EXlRLAN Template, "lEEEP.802.11-94/65, March 1994.

[6]	K. Feher: "JTC Modulation Standard Group FQPSK Consortium: Spectrum utilisation 	with compatible/expandable GMSK, QSPK, and FQPSK," JTC TR 46.3.3/TIPI.4 	Telecommunications Industry Association.

[7]	K. Feher: "HS-FH and IR FQPSK based proposed standards for 1.4 Mbit/s to 4.2 	Mbit/s," IEEE 8()2.11-94/51, March, 1994.

[8]	K. Feher: "Filter." U.S: PATENT No. 4,339,724. Issued July 13, 1982. Canada No. 	1130871, August 31, 1982.

[9]	S. Kato, K. Feher: "Correlated Signal Processor," U.S. Patent No. 4,567,602. Issued

January 28, 1986. Canada No. 1211-517. Issued September 16, 1986.

[10]	Feher, K., Ed.: "Advanced Digital Communications: Systems and Signal Processing 	Techniques," Prentice-Hall, Inc., Englewood Cliffs, New Jersey 07632, 1987.

[11]	S. Kato et al, NTT: "Performance of OQPSK and equivalent FQPSK-KF for the .DS-SS 	system. IEEE P802.11 93/189. November 1993.

[12]	Z. Wan K Feher: "Modulation specifications for 2 Mb/s. DS-SS system," IEEE 802 11 	94/02. Wireless Access and Physical Layer Specifications. Jan. 1994

[13]	H. Mehdi. K Feher: "Compatible power efficient NLA technique (I Watt) for DS-SS,'' 	IEEE 802.11-94/04. Wireless Access and Physical Layer Specifications. Jan. 1994.

[14]	Y. Guo. H Yan, K Feher- "Proposed modulation and data rate for higher speed 	frequency hopped spread spectrum (HS-FH SS) standard," IEEE 802.11-94/03. 	Wireless Access and Physical Layer Specifications. Jan. 1994

[15]	M. Soderstrand et al.: (with Feher)- "DS-SS and higher speed FH-SS modem VLSI 	implementation." IEEE 8()2.11 -94/06, Wireless Access and Physical Layer 	Specifications. Jan. 1994.

[16]	S. Kato (NTT-Japan) et al.: 'Implementation architecture and suggested preambles: 	VLSI implementation of FQPSK and offset QPSK-WLAN, IEEE 802.11-93/137, 	Wireless Access and Physical Layer Specifications. Jan 1994.

[17]	 K. Feher. " FQPSK,GMSK and QPSK Compatible Proposed Air Interface Standards 	for TDMA,FDMA and CDMA" Document: JTC(AlR)/94.01.19-035,January 1994

[18]	W. Just et al: "FQPSK-Siemens Components for EXIRLAN Infrared (IR), Wireless

IEEE 802.11 Radio, other European, American and International Standards"

IEEE 802.11-94/71, Wireless Access Methods and Physical Layer Specifications,

March 1994.

Fast emitter

important data

|  | SFH 477 | SFH 495 P | F 237 A |  |
| --- | --- | --- | --- | --- |
| tr / tf | 20 | 7 | 30 | ns |
| phie (100 mA) | 25 | 5 | 50 | mW |
| phie (1 A) | 250 | 700 | -- | mW |
| lambda | 880 | 945 | 830 | nm |
| Vf (1 A) | 3 | 2 | 10 | V |
| Vf (100 mA) | 1,5 | N.A. | 2,4 | V |

Table 1

Figure 1

Table 2

µ §

Figure 1a

Figure 2A

Figure 3