IEEE 802.11

Wireless Access Method and Physical Layer Specification

Title:		IR PHY Baseband Template

Editors:	Rui Valadas, University of Aveiro, Portugal

Barry Dobyns, Photonics Corporation

General Specifications

Parameter

Value

Comments

Spectrum Occupancy

DC to 5 MHz

Baseband

Data Rate

1 and 2 Mbps

Wavelength

850 - 950 nm

Modulation Method

PPM

16-PPM for 1 Mbps

4-PPM for 2 Mbps

Number of Channels

1

Propagation Mode

Diffuse

Non aimed transceivers

Transmitter Specifications

Parameter

Value

Comments

Output Peak Power

2.0 W ± 20%

@900 nm

Pulse Format

See fig. 3 doc:94/96

• tw (pulse width)

250 ns ± 10 ns

1 and 2 Mbps

• tr (rise time)

40 ns (max)

10 to 90%

• tf (fall time)

40 ns (max)

10 to 90%

• tj (jitter)

10 ns (max)

absolute deviation

Emitter Radiation Pattern

TBD

Receiver Specifications

Parameter

Value

Comments

Sensitivity (BER=10-9)

-47 dBm/cm2 (1 Mbps)

-10 dBm/cm2 ambient

(max)

-41 dBm/cm2 (2 Mbps)

light

Minimum Dynamic Range

30 dB

On the irradiance at the receiver detector

Minimum Field of View

150_

At the physical limit

Frame Error Rate (FER)

£ 4 ¥ 10-5

MAC frame = 512 octets

Carrier Sense out to MAC

£ 12 µs

After the preamble start

TX-RX turnaround time

TBD

RX-TX turnaround time

TBD

IR silence to Carrier Sense deassert

16 µs

In case of EFD failure

Frame Specifications I

Parameter

Value

Comments

Preamble

57 - 73 time slots

See "Frame specs. II"

Start of Frame Delimiter

4 time slots

See "Frame specs. II"

Data Rate Field

3 time slots

See "Frame specs. II"

DC Level Adjustment

32 time slots

See "Frame specs. II"

MAC Frame

TBD

Integer number of octets

End of Frame Delimiter

16 time slots

See "Frame specs. II"

Frame Specifications II

Parameter

Format

Comments

Preamble Format

010101...0101010

1 and 2 Mbps

SFD Format

1001

1 and 2 Mbps

DC Level Adjustment

2 ¥ symbol '8':

1 Mbps / 16-PPM

Field Format

8 ¥ symbol '2':

2 Mbps / 4-PPM

Data Rate Field Format

000

1 Mbps

001

2 Mbps

other formats:

TBD

EFD Format

0000011011011011

1 and 2 Mbps

PPM Mapping Table

4-PPM

16-PPM

PPM Symbol

00

0000

1 - 0000000000000001

01

0001

2 - 0000000000000010

10

0010

3 - 0000000000000100

11

0011

4 - 0000000000001000

0100

5 - 0000000000010000

0101

6 - 0000000000100000

0110

7 - 0000000001000000

0111

8 - 0000000010000000

1000

9 - 0000000100000000

1001

10 - 0000001000000000

1010

11 - 0000010000000000

1011

12 - 0000100000000000

1100

13 - 0001000000000000

1101

14 - 0010000000000000

1110

15 - 0100000000000000

1111

16 - 1000000000000000