Summary Of 1994 Motions of the Frequency Hop Group

Jim McDonald

Motorola Inc.

50 E Commerce Dr.

Schaumburg, IL 60173

708 576 3169

jim_mcdonald@wes.mot.com

Abstract:

This submission is a summary of the technical motions passed in the Frequency Hop Group during 1994. The listing is taken directly from the published minutes. It is hoped that this listing will be useful to the members of the Frequency Hop Group.

Jan 94

.......................................................................................................................................................................................

Jerry Socci: : If your received power is below some threshold, you can go ahead and blast.  If your received power is above the threshold, you can also blast but only if you can't detect a clock (no bits are present).  That is, you defer iff power is above the threshold AND your clock recovery circuits indicate bit sync.

MOTION :       We accept Jerry's proposal [above] as a baseline and call for submissions on CCA

Moved: T. Blaney, Seconded: E. Geiger

The chair ruled Motion 1 passed unanimously.

............................................................................................................................................................................................

MOTION 	The FH PHY group accepts IBM's proposed hopping sequences, in document 93/@ for 802.11-compatible FH WLANs.

Moved: J. McKown, Seconded: W. Moyers

VOTE ON MOTION :   Yes=16, No=0, Abstain=2.  Motion 1 passes.

............................................................................................................................................................................................

MOTION    We shall remove from 93/161 all reference to the subject matter of line 16 of 93/83r2 (fall back data rates below 1 Mbps).

Moved: J. McKown, Seconded: J. Renfro

VOTE ON MOTION 1:  Yes=12, No=0, Abstain=5

MOTION   We shall remove from 93/161 all reference to the subject matter of lines 17 and 17a of 93/83r2 (baseband bit jitter & clock accuracy).

Moved:  J. McKown, Seconded: M. Traynor

VOTE ON MOTION :  Yes=4, No=1, Abstain=2.  Motion 3 passes.

...............................................................................................................................................................

MOTION    In-band spurious emissions shall be -55 dBc.

Moved: P. Chadwick, seconded R. Jellicoe

VOTE ON MOTION :  Yes=9, No=0, Abstain=5.  Motion 4 passes.

.............................................................................................................................................................................................

March 94

MOTION :  Editing of the FH draft spec will proceed by simple majority and will be confirmed in the FH ad hoc group by 75% majority of the voting members.  Moved: E. Geiger. Seconded: C. Zegelin.

VOTE ON MOTION :  Motion passes unanimously by acclaim.

...........................................................................................................................................................................

MOTION :  The FH group accepts 94/68 as the FH working draft standard.  Moved: P. Chadwick.  Seconded:  D. Kawaguchi.

VOTE ON MOTION :  for=12, against=1, abstentions=0.  The motion passes.

...........................................................................................................................................................................

July 94

Motion:

After discussion, proposed Dean Kawaguchi, seconded Stuart Kerry, that CCA must be evaluated  with an antenna that has essentially the same coverage and loss as the transmit antenna.

Question called Jerry Loraine, seconded John McKown

In favour 10    Opposed 2  Abstention  1

...........................................................................................................................................................................

Motion:

Dean Kawaguchi proposed  to amend  to read [90%] probability of detection for preamble, and a [70%]  probability for random data. Seconded by Wayne.

Question on acceptance for the amendment called by Jerry Loraine, seconded Jim Renfro. In favour: 14. Against: 2. Abstentions: 2.

Motion:

Ed Geiger moved to amend to remove the section "or any signal greater than [-50dBm]."

Question called Ed Geiger seconded John McKown

In favour: 13     Opposed:  4       Abstentions: 1

.................................................................................................................................................................

Motion:

Question called by Ed Geiger, seconded by John McKown, on the motion which reads

"In the presence of any 802.11 compliant FH PHY signal above [-80]dBm, the PHY must signal busy within [16]us at [90]% probability of detection for preamble and a [70]% probability detection for random data. Note: [] = TBD"

In favour 13, Against 3, Abstentions 3.

...........................................................................................................................................................................

Motion:

Moved by Dean Kawaguchi that the CCA threshold as defined in the proceeding motions above be -85dBm. Seconded Stuart Kerry.

Question called by Jim Renfro, seconded Jerry Loraine

Question called.

In favour  13   Opposed  0  Abstentions 2

...........................................................................................................................................................................

Motion:

After discussion, proposed by Jerry Loraine, seconded Dean Kawaguchi, that the receiver sensitivity be -84dBm midband, -82dBm band edge. Friendly amendment to -80dBm, by Jim Renfro, seconded Peter Chadwick. Moved to call the motion Dean Kawaguchi, seconded Jerry Loraine,

Unanimous 13,0,0 voting.

MOTION CARRIES

.........................................................................................................................................................................

Motion:

Moved to define that the above figure applies at a 1 in 10*5 BER moved by Dean Kawaguchi, seconded Jim Renfro. Question called by Jerry Loraine, seconded Dean Kawaguchi. Question called,  11 in favour, 2 abstentions.

MOTION CARRIES

Motion:

Proposed Jerry Loraine, seconded Dean Kawaguchi, that the IM performance in 94/78 be accepted, except the level be -30dB. Question called by Wayne Moyers, seconded Dean Kawaguchi. On the calling of the question, voting, 7 in favour, 1 against, 1 abstention. Voting on the motion: 7 in favour, 2 against, 0 abstenti

ons.

MOTION CARRIES

...................................................................................................................................................................

Motion:

Proposed Jerry Loraine, that the desense test in 94/78 be accepted, except that at 2MHz, the figure be 30dB, and at 3MHz 40dB, with the desired signal at -3 dB relative to sensitivity. Friendly amendment by Peter Chadwick to amend sensitivity to -80dBm. Accepted. Peter Chadwick raised point of clarification regarding the interfere signal - is it amplitude modulated?

AGREED that the signal is NOT amplitude modulated.

Move to call the question: Peter Chadwick, seconded Jerry Loraine, passed.

Voting on the motion: In favour 7, against 1 , abstentions 4.

MOTION CARRIES.

........................................................................................................................................................................

Motion:

Moved Ed Geiger, seconded Wayne Moyers that the document (94/159) be accepted, with the proviso that some figures are TBD.

Question called by: Peter Chadwick         seconded: Jim Renfro

Question called unanimously

Voting on the motion: for:  11   against:  1     abstentions:  1

MOTION PASSES.

......................................................................................................................................................................

Motion:

Proposed to enter the contents of 94/156 into 94/68. proposed Dean Kawaguchi, seconded Wayne Moyers.  On the motion, accepted 8,0,4.

..................................................................................................................................................................

Motion:

Naftali Chayat presented 94/206, proposing a better unique word, and moved  to adopt 5555 0CBD. Question called Dean  Kawaguchi, seconded Wayne Moyers.

Question called. Motion passes 7, 0, 2.

Aug/Sept 94

.....................................................................................................................................................................

Motion to remove min column  from 9.5.7.2

by JimR and second by EdG.

No discussion

result: 6,0,0 passed.

......................................................................................................................................................................

Motion:

From 94/111:

"The occupied channel bandwidth for the PMD is 1.0 MHz wide.  This 1.0 MHz must contain 99% of the emitted energy.  The FCC may impose a further restriction on transmitted bandwidth requiring the 20 dB bandwidth, as measured with a spectrum analyzer and referenced to the magnitude at the center of the transmitted bandwidth to be less than 1 MHz.

The transmitter center frequency shall be within +-25ppm of one of the specified operating center frequencies listed in section 9.5.7.3.  The following diagram (Fig. 9-11 of 94/068r3) illustrates the relationship of the operating transmitter center frequency to the occupied channel bandwidth."

EdG calls the question John M 2nd.

result: 6, 0,0 question called.

vote on motion

result: 4, 1,1 passed.

..........................................................................................................................................................................

MOTION:

"The absolute mid-symbol peak deviation throughout the packet shall be at least 110 kHz with respect to the center frequency during the last 8 bits before the unique word."

2nd by JimR

Discussions on the definition and the measurement of center frequency.

LarryZ: proposes to define the center frequency to be the average of the difference of the average of maximum and minimum deviations.

JL: call the question, JohnM: 2nd, passes.

Vote on motion: 10,1,1 passes.

......................................................................................................................................................................

MOTION:

Zero Crossing error shall be less than +-1/8 of a symbol, per diagram.

This motion will include a picture by EdG.

Moved by CJ. 2nd by EdG.

Vote on Motion: 9,1,2 passes

Motion:

Set Time from MAC command to start a transmission of the first bit of 1,0 	preamble is 20 usec, Maximum.

based on the following

R/Tt  as defined in 94/113 < 10 usec

Xmit Delay, TDt < 1 usec

Ramp Allocation = 8 usec

MAC State Machine delay < 1 usec

moved by JimM, 2nd by JL

Vote: 9,1,2 passed

..........................................................................................................................................................

Motion:

TBD microsecond in 9.5.7.11 of 94/068r3 is 224 usec.  The frequency accuracy 	TBD kHz is changed to 60 kHz.

Moved by JimM, 2nd by JohnM

Vote on Motion: 6,1,0 the motion passed.

.....................................................................................................................................................................

Motion:

9.5.7.15 VSWR

Move to delete this spec.

by JL, 2nd by EdG

Vote: on Motion: 8,0,0 the motion passes

.........................................................................................................................................................................

Motion:

Slot Time is 50 usec.

The interval from the MAC command to transmit to the appearance of the first bit 	of the preamble at the RF antenna (also called collision time) is a maximum of 20 	usec.  The channel shall be assessed for at least 30 usec.  The interval from the 	start of the preamble at the antenna to the time when the ch_busy line goes high is 	a maximum of 16 usec.

Moved by JimM, 2nd by EdG

JohnM calls the question, JL second

Vote: 8,0,0 the question is called

Vote on motion: 8,0,1 the motion passes.

................................................................................................................................................................

Motion: Frame Length of 94/69 proposal is 32 bits

by EdG, 2nd Stuart

Vote on Motion: 5,1,1 motion passes.

Motion: Define CCA Threshold as a function of the intended RF power level.

CCA Threshold  = -65 dBm - Transmit Power in dBm.

by LarryZ, 2nd by JimR

JL call the question

the question is called.

Vote on Motion: 4,0,4 the motion passes.

......................................................................................................................................................................

Motion:

JL: propose a change in the motion text

The 802.11 compliant frequency hopping transmitters shall be labeled in four 	classes according to their maximum nominal EIRP

Class 1 up to 10 mW

Class 2 up to 100 mW

Class 3 up to 500 mW

Class 4 up to 1 W.

2nd by Wayne Moyers

Stuart: Call the question, Wayne 2nd

Vote: 9,0,0 the question is called.

Vote on Motion: 7,2,2 the motion passes.

.................................................................................................................................................................

Motion:

WayneM: the last word in the text of 9.5.8.2 should be 1000mW rather than 100mW.

Motion:

Change last word in 9.5.8.2 to 1000mW.

CJ 2nd.

Vote on Motion: 7,0,3 the motion passes.

......................................................................................................................................

Motion:

The transmit spectrum mask shall be measured under dynamic conditions such 	that the power generated in a 1 MHz Channel, for a given carrier offset, shall be 	less than the values in the table below:

Channel Offset (MHz)	Specification Limit (dBc)

+/- 2				-40

>= +/-3			-60

The radio shall be set to alternatively xmit and rcv with nominal duty cycle ratio 	of 1 to 1.  And the xmit packet length shall be greater than 300 usec and less than 	2 msec.

moved by JL, JimR 2nd.

Vote on Motion: 10,0,2 the motion passes.

JohnM:

Motion: Delete the table and replace the last sentence to

It shall maintain this stability over the stated operating temperature range.

2nd by CJ

Vote on motion: 10,0,0 the motion passes.

................................................................................................................................................................

Nov 94

(Paragraph numbers in this section in general refer to 58r5.)

Motion:	The FHSS PLCP preamble and header per figure 9-3 shall be at PLCP_ rate.  The PLCP_PDU may run at 1,2 Mbps & higher rates.

Moved: 	Pablo			2nd:	Wayne			Vote:	17/1/2

..........................................................................................................................................................................

Motion:	Accept frame coding per 94/287

Moved: 	Mike			2nd:	Wayne			Vote:	16/2/2

.........................................................................................................................

Motion:	Add the international specs back into 068/r6.  The numbers are subject to the regulatory authorities.

Moved: 	Dean			2nd:	Kieth			Vote:	17/0/1

............................................................................................................................

Motion:	Change +/-25ppm to +/-60khz in 9.6.5

Moved: 	Jim			2nd:	Nathan		Vote:	consensus

..................................................................................................................................

Motion:	The default dwell time for FH will be 20ms.

Moved: 	Ed			2nd:	John M			Vote:	12/0/1

Motion:	Approve Changes to 9.6.1 to 9.6.7

Moved: 	Ed			2nd:	Mike			Vote:	14/0/0

..............................................................................................................................

Motion:	Add Equivalent Isotropically Radiated Power and reference to 							regulatory authorities in text and delete table.

Moved: 	John Sonnenberg		2nd:	Wayne	Vote:	15/0/1

............................................................................................................................

- 9.6.18 Transmit power level control

Motion:	Delete section 9.6.18

Moved: 	Stuart			2nd:	Peter			Vote:	12/1/1

................................................................................................................................................

Motion:	Strike section clock recovery time (9.6.28 in 68r5)

Moved: 	Ed			2nd:	John M		Vote:	12/0/0

- 9.6.21 Transmit center frequency tolerance

.discussion on temp operating range and tolerance spec in Hz vs ppm.

Motion:	Accept the text as defined in 068r6 sect 9.6.21

Moved: 	Ed			2nd:	Keith			Vote:	11/0/2

.....................................................................................................................................................

- 9.6.22 PMD receiver specs OK

- 9.6.23 Input dynamic range

.discussion on

Motion:	Modify title to Input dynamic range

Moved: 	Peter			2nd:	Dean			Vote:	10/0/0

Motion:	Accept the text as defined in 068r6 sect 9.6.23

Moved: 	Peter			2nd:	Dean			Vote:	13/1/1

................................................................................................................................................................

- 9.6.24 Selectivity

Motion:	Delete section

Moved: 	Peter			2nd:	Dean			Vote:	11/0/2

...................................................................................................................................................................

- 9.6.25 Channel BER

Motion:	Delete section

Moved: 	Jerry			2nd:	Peter			Vote:	11/2/1

.................................................................................................................................................................

- 9.6.26 Rx center freq acceptance range

Motion:	Modify title to above and reflect changes as in 068r6 sect 9.6.21

Moved: 	Ed			2nd:	Charlie			Vote:	11/0/0

..............................................................................................................................................................

- 9.6.27 CCA response time

Motion:	Ed and Dean to capture spirit of wording from previous meeting

Moved: 	Peter		2nd:	Dean			Vote:	consensus

................................................................................................................................................

- 9.6.29 Rx data jitter tolerance

Motion:	Delete section as interface is not exposed

Moved: 	Ed		2nd:	Jerry				Vote:	12/0/0

.................................................................................................................................................

- 9.6.30 Ramp up period

Motion:	Move to table pending submission of mods

Moved: 	Ed		2nd:	John				Vote:	consensus

.....................................................................................................................................................

- 9.6.xx _ Rx data DC offset & Max run length

Motion:	Delete these sections

Moved: 	Ed		2nd:	John				Vote:	consensus

..............................................................................................................................................

- 9.7.1	Introduction: FHSS Phy Medium Dependent sublayer 2.0M bit

Motion:	Accept text as in 068r6

Moved: 	Ed		2nd:	Dean				Vote:	consensus

- 9.7.2	thr 9.7.9 _ OK

Motion:	Accept text as in 068r6 [similar to 1Mbit sections]

Moved: 	Ed		2nd:	Dean				Vote:	consensus

.......................................................................................................................................................

- 9.7.11	Channel Data rate

Motion:	Accept text as in 068r6

Moved: 	Ed		2nd:	Jerry				Vote:	consensus

..........................................................................................................................................................

- 9.7.12 to 9.7.24_OK as 1M Phy except for 9.7.23 below

- 9.7.23_Input dynamic range

Motion:	Accept text as in 068r6

Moved: 	Ed		2nd:	Jerry				Vote:	consensus

...........................................................................................................................................

- 9.7.25	Channel BER

Motion:	Delete text as in 068r6

Moved: 	Ed		2nd:	Jerry				Vote:	consensus

..........................................................................................................................................

- 9.7.26 to 9.7.28_OK as 1M Phy

....................................................................................................................................

- 9.7.29 Jitter tol

Motion:	Delete section

Moved: 	Jerry			2nd:	Peter		Vote:	consensus

.....................................................................................................................................

- 9.7.30 to 9.7.31_OK as 1M Phy

......................................................................................................................................

- 9.6.32 Rx sensitivity

Motion:	Accept text as in 068r6

Moved: 	Jerry			2nd:	Peter		Vote:	consensus

.............................................................................................................................................

- 9.6.33 Intermodulation

Motion:	Accept text as in 068r6

Moved: 	Ed			2nd:	Jerry			Vote:	consensus

.............................................................................................................................................

- 9.6.34 Desense

Motion:	Accept text as in 068r6

Moved: 	Ed			2nd:	Jerry			Vote:	consensus

................................................................................................................

-9.6.20	Tx spectrum mask

Motion:	Accept inputs as defined by Jerry and edited into 068r6

Moved: 	Jerry			2nd:	Nathan		Vote:	consensus

Chair’s Question: Is the editors’ translation of Jerry’s input accurate as shown in r6?

- 9.6.13	RxTx switch time

Motion:	Accept inputs as defined by Jerry and edited into 068r6

Moved: 	Ed			2nd:	Jerry			Vote:	consensus

- 9.6.18	Tx power level control

Motion:	Power level adjustment, if performed will be done by a management entity and deletes Twpwrlvl parameter in 9.2.2 of 068r5

Moved: 	Jerry			2nd:	Ed		Vote:	consensus

...................................................................................................................................................

- 9.6.28	CCA Threshold

Motion:	Accept edits in 068/r6 (motion of 11/8 by Charlie amended by Dean)

Moved: 	Jerry			2nd:	Charlie		Vote:	10/1/2

..............................................................................................................................................

- 9.6.23	CCA response time

Motion:	Add sentence to 9.3.3.2.1 to cover async case (Deans wording)

Moved: 	Dean			2nd:	Charlie		Vote:	consensus

Motion:	Delete section 9.6.23 (covered in 9.3.3.2.1)

Moved: 	Dean			2nd:	Charlie		Vote:	consensus

........................................................................................................................................................

- 9.6.30 & 31	Ramp up & Ramp down periods

Motion:	reopen .31

Moved: 	Peter			2nd:	Charlie		Vote:	consensus

Motion:	Modify text as defined in 068/r6 - remove slope limit, change bit period to uS & power levels relative to full Tx power in 9.6.30 & 31

Moved: 	Peter			2nd:	Charlie		Vote:	consensus

..........................................................................................................................................................

- 9.2 	FHSS service parameter lists

Motion:	Modify text as defined in 068/r6 - put in text for antsel and RSSI

Moved: 	Ed			2nd:	Charlie		Vote:	consensus

...........................................................................................................................................................

- 9.7.10 	4 level GFSK Modulation

Motion:	Modify text as defined in 068/r6

Moved: 	Jerry		2nd:	Brian Messenger		Vote:	consensus

...................................................................................................................................................................

• 	Motion:	Accept text as defined in 068/r6 with understanding that text is being updated to reflect changes approved during FH Phy session to date.

Moved: 	Dean			2nd:	Keith			Vote:	16/0/1

...........................................................................................................................................................

•	Motion:	Present to Phy & Plenary, a motion " Accept the 068/r5 plus the errata 94/242 which summarizes the 94/068r6 document."

Moved: 	Dean			2nd:	Keith			Vote:	13/0/0

.....................................................................................................................................................

- Par req of 99.5% probability of channel availability

Motion:	Delete 9.6.14

Moved: 	John			2nd:	Nathan		Vote:	consensus

Action: Jim M to prepare a motion to present to plenary in Jan.

................................................................................................................................................

- 4 level FSK _ Text has been generated jointly by Naftali and Jerry and given to the editors

Motion:	Accept the text as defined for 94/068r7 for Jan.

Moved: 	Jerry			2nd:	Wayne		Vote:	13/0/1